"""

无人机通信性能劣化场景优化 - 高级遗传算法实现

将无人机高度、干扰源参数作为染色体进行优化

干扰源与建筑物紧密结合，考虑实际城市环境

支持在建筑物和无建筑物位置生成干扰源

"""

import numpy as np

import geatpy as ea

import math

import time

import matplotlib.pyplot as plt

from mpl_toolkits.mplot3d import Axes3D

import matplotlib

from matplotlib.patches import Rectangle

from matplotlib.lines import Line2D

import os

import json

from typing import List, Dict, Tuple, Optional

# 创建输出目录

os.makedirs("output", exist_ok=True)

# 设置中文字体支持

def set_chinese_font():

"""设置中文字体支持"""

try:

# 尝试不同平台的中文字体

if matplotlib.get_backend().lower() in ['agg', 'tkagg', 'gtkagg', 'wxagg']:

# 对于非交互式后端，需要特殊处理

plt.rcParams['font.sans-serif'] = ['SimHei', 'DejaVu Sans']

plt.rcParams['axes.unicode_minus'] = False

else:

# 对于交互式显示

plt.rcParams['font.sans-serif'] = ['SimHei', 'Microsoft YaHei', 'DejaVu Sans']

plt.rcParams['axes.unicode_minus'] = False

# 设置字体大小

plt.rcParams['font.size'] = 10

plt.rcParams['axes.titlesize'] = 12

plt.rcParams['axes.labelsize'] = 11

except Exception as e:

print(f"字体设置警告: {e}")

print("继续使用默认字体...")

# 初始化中文字体

set_chinese_font()

class InterferenceSourceConfig:

"""干扰源配置类 - 定义不同干扰源的特性"""

def __init__(self):

# 定义干扰源类型及其特性

self.interference_configs = {

'wifi_2_4g': {

'name': 'WiFi 2.4GHz AP',

'frequency': 2.4415e9,

'frequency_range': (2.400e9, 2.483e9),

'evidence_screenshots': {'frequency': 'fcc_15_247_24ghz_band.png', 'background_PPP': 'stochastic_ppp_3d.png'},

'power_range': (5, 25),  # dBm（下界锚点：2.4GHz 最低 5 dBm）

'typical_height_range': (5, 50),  # 米，通常部署在室内或低矮建筑

'height_distribution': 'building_low',  # 建筑物低层

'path_loss_factor': 3.0,  # 路径损耗系数

'coverage_radius': 100,  # 覆盖半径（米）

'description': 'WiFi接入点，常见于室内和公共场所',

'color': '#FF6B6B',  # 红色

'calibration': {

'eirp_min_dbm': 5.0,  # 锚点：Wi‑Fi 2.4GHz 最低发射功率门槛

'wifi_beacon_period_s': 0.1024,  # 锚点：beacon 周期 102.4 ms

'wifi_beacon_duration_ms_default': 0.5,  # 经验默认（可由 user_params 覆盖）

'wifi_traffic_duty_cycle_default': 0.15,  # 经验默认（可由 user_params 覆盖）

'sources': {

'eirp_min_dbm': '图片大纲：Wi‑Fi 2.4GHz 最低发射功率门槛 5 dBm',

'wifi_beacon_period_s': '图片大纲：Wi‑Fi beacon 周期 102.4 ms'

}

}

},

'wifi_5_8g': {

'name': 'WiFi 5.8GHz AP',

'frequency': 5.7875e9,

'frequency_range': (5.725e9, 5.850e9),

'evidence_screenshots': {'frequency': 'fcc_15_247_58ghz_band.png', 'background_PPP': 'stochastic_ppp_3d.png'},

'power_range': (8, 25),  # dBm（下界锚点：5GHz 最低 8 dBm）

'typical_height_range': (5, 50),  # 米

'height_distribution': 'building_low',

'path_loss_factor': 3.5,

'coverage_radius': 80,

'description': 'WiFi 5G接入点，穿墙能力稍弱',

'color': '#4ECDC4',  # 青色

'calibration': {

'eirp_min_dbm': 8.0,  # 锚点：Wi‑Fi 5GHz 最低发射功率门槛

'wifi_beacon_period_s': 0.1024,

'wifi_beacon_duration_ms_default': 0.5,

'wifi_traffic_duty_cycle_default': 0.15,

'sources': {

'eirp_min_dbm': '图片大纲：Wi‑Fi 5GHz 最低发射功率门槛 8 dBm',

'wifi_beacon_period_s': '图片大纲：Wi‑Fi beacon 周期 102.4 ms'

}

}

},

'cellular_4g': {

'name': '4G基站',

'frequency': 2.6e9,

'power_range': (40, 55),  # dBm

'typical_height_range': (20, 100),  # 米，通常在建筑物顶部

'height_distribution': 'building_top',

'path_loss_factor': 2.7,

'coverage_radius': 500,

'description': '4G移动通信基站',

'color': '#45B7D1'  # 蓝色

},

'cellular_5g': {

'name': '5G基站',

'frequency': 3.6e9,

'frequency_range': (3.4e9, 3.8e9),

'evidence_screenshots': {'frequency': 'dominance_n78_midband.png', 'power_range': 'tr138913_p27_power_46dBm_23d*.png'},

# 图片大纲建议将“5G小基站”按 femto/pico/micro 分档；这里将 5G 默认按“小基站档位”标定

'power_range': (20, 46),  # dBm（覆盖 femto/pico/micro + 适度余量）

'typical_height_range': (20, 80),  # 米，部署更密集

'height_distribution': 'building_top',

'path_loss_factor': 2.8,

'coverage_radius': 300,

'description': '5G移动通信基站',

'color': '#96CEB4',  # 绿色

'calibration': {

'eirp_anchors_dbm': {

'femto': 20.0,

'pico': 24.0,

'micro': 36.0

},

'activity_factor': 0.70,  # 锚点：用 70% 发射活动因子把峰值 EIRP 折算成平均干扰强度

'sources': {

'eirp_anchors_dbm': '图片大纲：5G小基站 femto/pico/micro = 20/24/36 dBm',

'activity_factor': '图片大纲：发射活动因子 70%'

}

}

},

'cellular_ue_ul': {

'name': '蜂窝UE上行',

'frequency': 2.6e9,

'power_range': (0, 23),  # dBm（典型 UE Pmax=23 dBm）

'typical_height_range': (0, 300),  # 米（地面 UE ~0；空中 UE 可到数百米）

'height_distribution': 'building_low',

'path_loss_factor': 2.5,

'coverage_radius': 300,

'description': '蜂窝 UE 上行（分数功控 FPC 标定）',

'color': '#A29BFE',

'calibration': {

'ul_fpc': {

'pmax_dbm': 23.0,

'p0_candidates_dbm': [-83.0, -88.0, -95.0, -101.0, -110.0],

'alpha_ground_or_low_uav': 0.8,

'alpha_high_uav': 0.7,

'alpha_switch_height_m': 100.0,

'm_rb_candidates': [1, 5, 10, 25, 50],

'delta_tf_db': 0.0,

'f_delta_tpc_db': 0.0,

'pl_extra_db_default': 20.0,

'serving_distance_m_range': (30.0, 500.0),

},

'sources': {

'ul_fpc': '图片大纲：分数功控公式 + α(高度分段) + TR 36.777 典型 (α,P0) 组合'

}

}

},

'gnss_jammer': {

'name': 'GNSS干扰机',

'frequency': 1.575e9,

'evidence_screenshots': {'frequency': 'isgps_l1_1575_42.png', 'power_range': 'jammertest23_power_settings.png'},

'power_range': (20, 43),  # dBm

'typical_height_range': (5, 30),  # 米，地面或低矮建筑

'height_distribution': 'ground',

'path_loss_factor': 2.2,

'coverage_radius': 200,

'description': 'GPS/北斗导航干扰设备',

'color': '#FFEAA7',  # 黄色

# 口径钉死：本工程 power_range(dBm) 在链路预算里作为 P_tx[dBm] 使用，等效为 EIRP-equivalent（天线增益/馈损等折算进该项）

'power_semantics': 'eirp_equivalent_dbm',

'calibration': {

'eirp_range_dbm': (20.0, 40.0),

'source_type': 'portable/vehicular GNSS jammer (EIRP-equivalent)',

'notes': [

'本工程使用 EIRP-equivalent dBm 口径（P_rx=P_tx-PL+...），因此该区间应理解为等效辐射功率范围',

'上限 40 dBm 可作为“可实现输出上限以下的保守值”，下限 20 dBm 作为场景轻量级覆盖'

],

'sources': {

'eirp_range_dbm': '待补齐：ESA Jammertest 2023（文档提到最高约 43 dBm / 20 W 输出量级；本代码取 20–40 dBm 作为保守扫描范围）'

}

}

},

'industrial_device': {

'name': '工业设备干扰',

'frequency': 915e6,

'frequency_range': (902e6, 928e6),

'evidence_screenshots': {'frequency': 'fcc_15_247_902_928_band.png'},

'power_range': (25, 45),  # dBm

'typical_height_range': (5, 100),

'height_distribution': 'building_mid',

'path_loss_factor': 2.5,

'coverage_radius': 150,

'description': '工业设备产生的电磁干扰',

'color': '#DDA0DD',  # 紫色

'power_semantics': 'eirp_equivalent_dbm',

'calibration': {

'eirp_range_dbm': (25.0, 45.0),

'band_hz': 900e6,

'assumption': 'active transmitter in 902–928 MHz ISM band (EIRP-equivalent), not broadband EMI/PSD',

'eirp_mapping': 'P_EIRP[dBm] = P_cond[dBm] + G_t[dBi] - L_t[dB]',

'anchors': {

'p_cond_max_dbm': 30.0,

'eirp_upper_example_dbm': 45.0,

'eirp_upper_example_condition': '示例：P_cond=30 dBm + G_t≈15 dBi（定向天线）- L_t≈0 dB => P_EIRP≈45 dBm'

},

'notes': [

'若你要表达“工业EMI噪声源”，应改用 PSD/场强口径（dBm/Hz 或 dBµV/m），而不是 power_range(dBm)',

'当前实现是“单频/窄带等效发射机”模型：frequency+P_tx 直接进入 P_rx=P_tx-PL+...'

],

'sources': {

'eirp_range_dbm': '待补齐：FCC 47 CFR §15.247（902–928 MHz ISM 扩频/数字调制设备的 conducted power 上限与天线增益相关约束；本代码将其折算为 EIRP-equivalent 上界示例）'

}

}

},

'satellite_ground': {

'name': '卫星地面站',

'frequency': 14.25e9,

'frequency_range': (14.0e9, 14.5e9),

'evidence_screenshots': {'frequency': 'ku_buc_14ghz_power.png'},

'power_range': (50, 70),  # dBm

'typical_height_range': (10, 50),

'height_distribution': 'ground',

'path_loss_factor': 1.8,

'coverage_radius': 1000,

'description': '卫星通信地面接收站',

'color': '#FFA07A',  # 橙色

'power_semantics': 'eirp_equivalent_dbm',

'calibration': {

'eirp_range_dbm': (50.0, 70.0),

'eirp_range_dbw': (20.0, 40.0),  # dBW = dBm - 30

'eirp_mapping_dbw': 'P_EIRP[dBW] = 10log10(P_BUC[W]) + G_t[dBi] - L_t[dB]',

'unit_note': '本工程内部使用 dBm；若参考资料以 dBW 给出 EIRP，请用 dBm = dBW + 30 换算',

'notes': [

'该类型在仿真中作为“等效辐射源/干扰源”使用；不区分 conducted power 与天线增益，统一折算进 EIRP-equivalent',

'50–70 dBm (=20–40 dBW) 是“Ku 地面端等效EIRP”保守区间，用于覆盖不同终端/BUC 等级与系统余量'

],

'sources': {

'eirp_range_dbw': '待补齐：便携 Ku 终端规格（EIRP, dBW）+ BUC 规格（W）双锚点；将终端 EIRP(dBW) 与 BUC(W) 通过上式统一到 EIRP-equivalent 口径'

}

}

}

}

# 干扰源类型映射

self.type_mapping = {

0: 'wifi_2_4g',

1: 'wifi_5_8g',

2: 'cellular_4g',

3: 'cellular_5g',

4: 'gnss_jammer',

5: 'industrial_device',

6: 'satellite_ground',

7: 'cellular_ue_ul',

}

# 高度分布类型说明

self.height_distribution_types = {

'ground': '地面安装',

'building_low': '建筑物低层(1-5层)',

'building_mid': '建筑物中层(6-15层)',

'building_top': '建筑物顶层'

}

def get_config(self, type_id: int) -> Dict:

"""获取指定类型的配置"""

type_name = self.type_mapping.get(type_id, 'wifi_2_4g')

return self.interference_configs[type_name]

def get_all_types(self) -> List[str]:

"""获取所有类型名称"""

return [config['name'] for config in self.interference_configs.values()]

def get_type_id_by_name(self, name: str) -> int:

"""根据名称获取类型ID"""

for type_id, type_name in self.type_mapping.items():

if self.interference_configs[type_name]['name'] == name:

return type_id

return 0

def get_allowed_semantics(self, type_id: int) -> List[str]:

"""

每类干扰源允许出现的“场景语义集合” Γ(type)。

这是场景一致性约束层，不改变点过程/功率生成逻辑，只做条件重采样/投影。

"""

type_name = self.type_mapping.get(int(type_id), 'wifi_2_4g')

# 约定语义标签：street/sidewalk/ground/roof/indoor/air/pole/tower

if type_name in ('wifi_2_4g', 'wifi_5_8g'):

return ['indoor']

if type_name == 'cellular_4g':

return ['roof', 'tower']

if type_name == 'cellular_5g':

return ['roof', 'pole', 'tower']

if type_name == 'cellular_ue_ul':

return ['street', 'sidewalk', 'indoor', 'air']

if type_name == 'gnss_jammer':

return ['street', 'sidewalk']

if type_name == 'industrial_device':

return ['indoor', 'ground', 'sidewalk']

if type_name == 'satellite_ground':

return ['ground', 'roof']

return ['ground']

def get_duty_cycle(self, type_id: int) -> float:

"""burstiness：仅用于 WiFi 等间歇式业务，返回 [0,1] 占空比。"""

type_name = self.type_mapping.get(int(type_id), 'wifi_2_4g')

if type_name in ('wifi_2_4g', 'wifi_5_8g'):

return 0.25

return 1.0

# 复用配置实例，避免在热点路径中重复构造大字典

INTERFERENCE_SOURCE_CONFIG = InterferenceSourceConfig()

class Building:

"""建筑物类"""

def __init__(self, x: float, y: float, height: float, width: float, length: float):

self.x = x

self.y = y

self.height = height

self.width = width

self.length = length

self.color = '#45B7D1'

def footprint_area_m2(self) -> float:

return float(self.width) * float(self.length)

def volume_m3(self) -> float:

return self.footprint_area_m2() * float(self.height)

def floors(self, floor_height_m: float) -> int:

floor_h = max(float(floor_height_m), 0.5)

return max(1, int(math.floor(float(self.height) / floor_h)))

def floor_area_m2(self, floor_height_m: float) -> float:

return self.footprint_area_m2() * float(self.floors(floor_height_m))

def contains_point(self, point_x: float, point_y: float) -> bool:

"""判断点是否在建筑物平面范围内"""

return (self.x - self.width / 2 <= point_x <= self.x + self.width / 2 and

self.y - self.length / 2 <= point_y <= self.y + self.length / 2)

def intersects_segment_3d(self, p0: np.ndarray, p1: np.ndarray, eps: float = 1e-6) -> bool:

"""

判断 3D 线段 p0->p1 是否与建筑物轴对齐长方体相交（建筑物底面在 z=0，高度为 self.height）。

说明：

- 建筑物在平面上是轴对齐矩形，空间体为 AABB。

- 为避免“端点恰好落在边界上”造成误判，使用 eps 排除 t=0/1 的纯端点触碰。

"""

p0 = np.asarray(p0, dtype=float)

p1 = np.asarray(p1, dtype=float)

d = p1 - p0

xmin = float(self.x - self.width / 2)

xmax = float(self.x + self.width / 2)

ymin = float(self.y - self.length / 2)

ymax = float(self.y + self.length / 2)

zmin = 0.0

zmax = float(self.height)

tmin = 0.0

tmax = 1.0

for axis, (bmin, bmax) in enumerate(((xmin, xmax), (ymin, ymax), (zmin, zmax))):

if abs(float(d[axis])) < 1e-12:

# 与该轴平行，若起点不在 slab 内则无相交

if float(p0[axis]) < bmin or float(p0[axis]) > bmax:

return False

continue

inv = 1.0 / float(d[axis])

t1 = (bmin - float(p0[axis])) * inv

t2 = (bmax - float(p0[axis])) * inv

t_enter = min(t1, t2)

t_exit = max(t1, t2)

tmin = max(tmin, t_enter)

tmax = min(tmax, t_exit)

if tmax < tmin:

return False

# 排除纯端点触碰：要求相交区间落在 (eps, 1-eps) 内

return (tmax > eps) and (tmin < 1.0 - eps)

def get_suitable_interference_heights(self, interference_type: str) -> List[float]:

"""根据干扰源类型获取合适的安装高度"""

height_distribution = {

'ground': [2],  # 地面

'building_low': [self.height * 0.2, self.height * 0.4],  # 低层

'building_mid': [self.height * 0.5, self.height * 0.7],  # 中层

'building_top': [self.height * 0.9, self.height + 5]  # 顶层+天线

}

type_configs = INTERFERENCE_SOURCE_CONFIG.interference_configs

for _, config_info in type_configs.items():

if config_info['name'] == interference_type:

dist_type = config_info['height_distribution']

return height_distribution.get(dist_type, [self.height * 0.5])

return [self.height * 0.5]  # 默认中层

def get_random_position_on_building(self, height_offset: float = 0) -> Tuple[float, float, float]:

"""获取建筑物上的随机位置"""

# 在建筑物范围内随机位置

pos_x = np.random.uniform(self.x - self.width / 2, self.x + self.width / 2)

pos_y = np.random.uniform(self.y - self.length / 2, self.y + self.length / 2)

# 加上高度偏移（天线高度等）

total_height = self.height + height_offset

return pos_x, pos_y, total_height

class InterferenceSource:

"""干扰源类"""

def __init__(self, x: float, y: float, height: float,

type_id: int, power: float, building=None, is_ground: bool = False,

building_idx: Optional[int] = None,

is_indoor: bool = False,

indoor_floor: Optional[int] = None,

indoor_floors_total: Optional[int] = None,

indoor_distance_m: Optional[float] = None,

skip_height_clip: bool = False):

self.x = x

self.y = y

self.height = height

self.type_id = type_id

self.power = power

self.building = building  # 关联的建筑物

self.building_idx = building_idx  # 关联建筑物在列表中的索引（用于加速遮挡判断）

self.is_ground = is_ground  # 是否为地面干扰源

self.is_indoor = bool(is_indoor)

self.indoor_floor = indoor_floor

self.indoor_floors_total = indoor_floors_total

self.indoor_distance_m = indoor_distance_m

# 获取配置信息

self.config = INTERFERENCE_SOURCE_CONFIG.get_config(type_id)

self.name = self.config['name']

self.frequency = self.config['frequency']

self.color = self.config['color']

# 保持原始含义：配置中的“路径损耗系数”（线性域因子）

self.path_loss_factor = float(self.config.get('path_loss_factor', 1.0))

self.coverage_radius = self.config['coverage_radius']

# 调整功率在合理范围内

min_power, max_power = self.config['power_range']

self.power = np.clip(power, min_power * 0.8, max_power * 1.2)

# 调整高度在典型范围内

min_height, max_height = self.config['typical_height_range']

# 如果是地面干扰源，高度固定为0

if is_ground:

self.height = 0.0

elif skip_height_clip:

self.height = float(max(0.0, height))

else:

self.height = float(np.clip(height, min_height, max_height))

def calculate_path_loss(self, distance: float, drone_height: float, is_blocked: bool = False) -> float:

"""计算路径损耗（dB）"""

if distance < 1:

distance = 1.0

# 自由空间路径损耗（保持原公式）

fspl_db = float(20.0 * np.log10(distance) + 20.0 * np.log10(self.frequency) - 147.55)

# 考虑高度的额外损耗

height_factor = self._calculate_height_factor(drone_height)

# 将原先的“乘法因子”改为 dB 域加法：

# total_loss_db = FSPL(dB) + 10*log10(path_loss_factor * height_factor) + L_obstruction(dB)

factor = float(self.path_loss_factor) * float(height_factor)

extra_loss_db = float(10.0 * np.log10(max(factor, 1e-12)))

# 遮挡衰减（保持原来的 0.7 常数，但作为“功率衰减因子”使用）：

# P_rx *= 0.7  <=>  PL += -10*log10(0.7)

obstruction_attenuation = 0.7 if is_blocked else 1.0

obstruction_loss_db = float(-10.0 * np.log10(max(obstruction_attenuation, 1e-12)))

total_loss_db = float(fspl_db + extra_loss_db + obstruction_loss_db)

return total_loss_db

def _calculate_height_factor(self, drone_height: float) -> float:

"""计算高度因子"""

# 如果是地面干扰源（高度为0），使用不同的高度因子

if self.is_ground:

# 地面干扰源对低空无人机影响更大

if drone_height < 50:

return 0.9  # 地面干扰对低空影响较大

elif drone_height < 100:

return 1.1

else:

return 1.3  # 地面干扰对高空影响较小

# 非地面干扰源的原有逻辑

min_h, max_h = self.config['typical_height_range']

if drone_height < min_h:

# 无人机低于干扰源典型高度范围

return 1.2  # 增加损耗

elif drone_height > max_h:

# 无人机高于干扰源典型高度范围

return 0.8  # 减少损耗（高空视距更好）

else:

# 在典型范围内

normalized = (drone_height - min_h) / (max_h - min_h)

return 1.0 - normalized * 0.4

def get_effective_power(self, drone_pos: np.ndarray, buildings: Optional[List[Building]] = None) -> float:

"""计算对无人机的有效干扰功率"""

# 计算距离

distance = np.sqrt((self.x - drone_pos[0]) ** 2 +

(self.y - drone_pos[1]) ** 2 +

(self.height - drone_pos[2]) ** 2)

# 计算遮挡：线段是否穿过任意建筑体（排除干扰源自身所在建筑）

is_blocked = False

if buildings:

p0 = np.array([self.x, self.y, self.height], dtype=float)

p1 = np.asarray(drone_pos, dtype=float)

for b in buildings:

if (self.building is not None) and (b is self.building):

continue

if b.intersects_segment_3d(p0, p1):

is_blocked = True

break

# 计算路径损耗（dB）

path_loss_db = self.calculate_path_loss(float(distance), float(drone_pos[2]), is_blocked=is_blocked)

# 有效功率 = 发射功率 - 路径损耗

effective_power_dbm = float(self.power - path_loss_db)

# 考虑距离衰减（超出覆盖范围衰减）

if distance > self.coverage_radius and self.coverage_radius > 0:

excess_ratio = float(distance / self.coverage_radius)

# 以 1/d^2 形式额外衰减：P(dBm) -= 20log10(excess_ratio)

effective_power_dbm -= float(20.0 * np.log10(max(excess_ratio, 1e-6)))

return float(effective_power_dbm)

def get_info(self) -> Dict:

"""获取干扰源信息"""

pm = getattr(self, 'profile_meta', None)

power_semantics = pm.get('power_semantics', 'eirp_equivalent_dbm') if isinstance(pm, dict) else 'eirp_equivalent_dbm'

return {

'name': self.name,

'type_id': self.type_id,

'position': [float(self.x), float(self.y), float(self.height)],

'power_dbm': float(self.power),

'raw_power_dbm': float(getattr(self, 'raw_power_dbm', self.power)),

'profile_key': getattr(self, 'profile_key', None),

'profile_meta': getattr(self, 'profile_meta', None),

'power_semantics': str(power_semantics),

'frequency_ghz': float(self.frequency / 1e9),

'building_associated': self.building is not None,

'is_ground': self.is_ground,

'is_indoor': self.is_indoor,

'indoor_floor': self.indoor_floor,

'indoor_floors_total': self.indoor_floors_total,

'semantic': getattr(self, 'semantic', None),

'activity': float(getattr(self, 'activity', 1.0)),

'color': self.color

}

class DroneCommProblem(ea.Problem):

"""无人机通信性能劣化问题定义"""

ATG_ENV_PRESETS = {

# 常用环境参数（Optimal LAP / Al-Hourani 模型常见取值）

# C,B 对应 P(LoS)=1/(1+C·exp(-B(θ-C)))（θ:度）

'suburban': {'eta_los_db': 0.1, 'eta_nlos_db': 21.0, 'los_C': 4.88, 'los_B': 0.43},

'urban': {'eta_los_db': 1.0, 'eta_nlos_db': 20.0, 'los_C': 9.61, 'los_B': 0.16},

'dense_urban': {'eta_los_db': 1.6, 'eta_nlos_db': 23.0, 'los_C': 12.08, 'los_B': 0.11},

'highrise_urban': {'eta_los_db': 2.3, 'eta_nlos_db': 34.0, 'los_C': 27.23, 'los_B': 0.08},

}

# Al-Hourani 2014: Surface polynomial coefficients mapping (αβ, γ) -> a,b in Eq.(5)/(6)

# z = Σ_{j=0..3} Σ_{i=0..3-j} C_ij * (αβ)^i * γ^j

# Table I: coefficients for a

LOS_A_COEFFS: Dict[int, Dict[int, float]] = {

0: {0: 9.34e-01, 1: 2.30e-01, 2: -2.25e-02, 3: 1.86e-05},

1: {0: 1.97e-02, 1: 2.44e-03, 2: 6.58e-06},

2: {0: -1.24e-04, 1: -3.34e-06},

3: {0: 2.73e-07},

}

# Table II: coefficients for b

LOS_B_COEFFS: Dict[int, Dict[int, float]] = {

0: {0: 1.17e+00, 1: -7.56e-02, 2: 1.98e-03, 3: -1.78e-05},

1: {0: -5.79e-03, 1: 1.81e-04, 2: -1.65e-06},

2: {0: 1.73e-05, 1: -2.02e-07},

3: {0: -2.00e-08},

}

def __init__(self, user_params: Dict):

name = '无人机通信性能劣化优化'

m = 1  # 目标维度（单目标优化）

maxormins = [-1]  # -1表示最大化适应度（最大化劣化程度）

self.user_params = dict(user_params)

# 用户参数

self.num_drones = int(user_params['num_drones'])  # 无人机数量

self.num_stations = int(user_params['num_stations'])  # 地面站数量

self.area_size = float(user_params['area_size'])  # 区域大小

self.drone_height_max = float(user_params['drone_height_max'])  # 无人机最大高度

self.building_height_max = float(user_params['building_height_max'])  # 建筑物最大高度

self.building_density = float(user_params['building_density'])  # 建筑物密度

self.drone_speed_max = float(user_params['drone_speed_max'])  # 无人机最大速度

# ITU 城市统计参数（用于建筑分布与 LoS 概率）

# α：建筑覆盖率（0~1），β：建筑密度（buildings/km^2），γ：建筑高度 Rayleigh 分布尺度参数（m）

# 必须在生成建筑物之前初始化

self.itu_city = {

'alpha': float(user_params.get('itu_alpha', self.building_density)),

'beta_km2': float(user_params.get('itu_beta', 300.0)),

'gamma_m': float(user_params.get('itu_gamma', max(5.0, self.building_height_max / 2.0)))

}

# 固定参数

self.num_interference = int(user_params.get('num_controlled_interference', 8))  # 受控（染色体）干扰源数量

self.communication_freq = 2.4e9  # 无人机通信频率

self.light_speed = 3e8  # 光速

# 背景干扰源：用 3D PPP 生成大量“城市干扰场”（不进入染色体维度）

self.background_interference_enabled = bool(user_params.get('background_interference_enabled', True))

self.background_interference_density_scale = float(user_params.get('background_interference_density_scale', 1.0))

# 2D/3D PPP 强度口径：

# - '2d'：base_lambda_per_km2 直接表示 λ（sources/km^2）

# - '3d'：可额外提供 bg_rho_m3_{type}（sources/m^3），并用 ρ=λ/Z 换算，Z=background_interference_box_height_m

self.background_interference_density_mode = str(user_params.get('background_interference_density_mode', '2d')).strip().lower()

self.background_interference_box_height_m = float(user_params.get('background_interference_box_height_m', self.building_height_max))

self.background_interference_max_sources = int(user_params.get('background_interference_max_sources', 500))

self.max_sources_to_plot = int(user_params.get('max_sources_to_plot', 60))

# 城市场景“标签层”（street/sidewalk/roof/indoor/air/facade/pole...）

# 只做约束/投影，不推翻原始随机生成公式

self.scene_consistency_mode = str(user_params.get('scene_consistency_mode', 'resample')).strip().lower()

self.scene_consistency_max_attempts = int(user_params.get('scene_consistency_max_attempts', 12))

self.semantic_road_spacing_m = float(user_params.get('semantic_road_spacing_m', 0.0))  # 0 表示自动从 ITU 统计量推断

self.semantic_road_width_m = float(user_params.get('semantic_road_width_m', 20.0))

self.semantic_sidewalk_width_m = float(user_params.get('semantic_sidewalk_width_m', 6.0))

self.semantic_facade_band_m = float(user_params.get('semantic_facade_band_m', 2.0))

self.semantic_pole_snap_m = float(user_params.get('semantic_pole_snap_m', 3.0))

# 室内/楼层参数（用于“地面+楼层+屋顶”的 3D 分布）

self.floor_height_m = float(user_params.get('floor_height_m', 3.0))

self.indoor_device_height_m = float(user_params.get('indoor_device_height_m', 1.5))

# ITU P.1238（室内）简化项：L = 20log(f_MHz) + Nlog(d_m) + Lf(n) - 28

self.p1238_enabled = bool(user_params.get('p1238_enabled', True))

self.p1238_N = float(user_params.get('p1238_N', 30.0))

self.p1238_d_in_m = float(user_params.get('p1238_d_in_m', 10.0))

# ITU P.2109（BEL）参数化项：仅启用时生效（默认不启用，避免硬编码系数）

self.p2109_bel_enabled = bool(user_params.get('p2109_bel_enabled', False))

self.p2109_bel_r = user_params.get('p2109_bel_r', None)

self.p2109_bel_s = user_params.get('p2109_bel_s', None)

self.p2109_bel_t = user_params.get('p2109_bel_t', None)

self.p2109_bel_u = user_params.get('p2109_bel_u', None)

self.p2109_bel_li_db = user_params.get('p2109_bel_li_db', None)

self.p2109_bel_lg_clip_db = float(user_params.get('p2109_bel_lg_clip_db', 0.0))

# 统一“强度/损耗”劣化指标选择：'power_ratio'(默认，原逻辑) 或 'cdi'（强度/损耗合成）

# 干扰源配置（必须在生成固定场景元素/背景PPP前就绪）

self.interference_config = INTERFERENCE_SOURCE_CONFIG

# 生成固定场景元素

np.random.seed(42)  # 固定随机种子确保可重复性

self._generate_fixed_scene_elements()

# 决策变量定义

# 染色体包括：

# 1. 无人机高度 (num_drones 个)

# 2. 无人机速度 (num_drones 个，0到最大速度)

# 3. 干扰源类型 (num_interference 个)

# 4. 干扰源功率 (num_interference 个)

# 5. 干扰源位置类型 (num_interference 个，0=地面，1=建筑物)

# 6. 干扰源在建筑物上的位置偏移或地面位置 (num_interference * 2 个)

# 计算维度

dim = (self.num_drones * 2 +  # 无人机高度和速度

self.num_interference * 5)  # 干扰源类型、功率、位置类型、位置(x,y)

# 变量类型

var_types = ([0] * self.num_drones +  # 无人机高度: 连续

[0] * self.num_drones +  # 无人机速度: 连续

[1] * self.num_interference +  # 干扰源类型: 离散

[0] * self.num_interference +  # 干扰源功率: 连续

[1] * self.num_interference +  # 干扰源位置类型: 离散 (0=地面, 1=建筑物)

[0] * self.num_interference * 2)  # 位置: 连续

# 变量下界

lb = ([10] * self.num_drones +  # 无人机高度下限

[0.0] * self.num_drones +  # 无人机速度下限（允许悬停 V=0）

[0] * self.num_interference +  # 干扰源类型下限

[0] * self.num_interference +  # 干扰源功率下限

[0] * self.num_interference +  # 干扰源位置类型下限

[0] * self.num_interference * 2)  # 位置范围 [0, area_size]

# 变量上界

ub = ([self.drone_height_max] * self.num_drones +  # 无人机高度上限

[self.drone_speed_max] * self.num_drones +  # 无人机速度上限

[7] * self.num_interference +  # 干扰源类型上限（0-7）

[70] * self.num_interference +  # 干扰源功率上限

[1] * self.num_interference +  # 干扰源位置类型上限 (0-1)

[self.area_size] * self.num_interference * 2)  # 位置范围

# 验证维度一致性

assert len(var_types) == dim, f"var_types长度{len(var_types)}与dim{dim}不匹配"

assert len(lb) == dim, f"lb长度{len(lb)}与dim{dim}不匹配"

assert len(ub) == dim, f"ub长度{len(ub)}与dim{dim}不匹配"

# 调用父类构造方法

super().__init__(name, m, maxormins, dim, var_types, lb, ub)

# 权重参数

self.metric_weights = {

'power_intensity_change': 0.7,

'speed_energy_efficiency': 0.3

}

# 通信劣化评估模型：

# - legacy：沿用“基准功率 + 干扰/信号比”的启发式

# - cdi：按你给的公式，只用“损耗Λ + 干扰强度I(shot-noise)”合成 CDI（默认）

self.degradation_model = str(user_params.get('degradation_model', 'margin')).strip().lower()

# 功率域参数：噪声功率 Ny = N0·B·F（线性域 mW 累加）

self.bandwidth_hz = float(user_params.get('bandwidth_hz', 20e6))

self.noise_figure_db = float(user_params.get('noise_figure_db', 7.0))

self.thermal_noise_dbm_hz = float(user_params.get('thermal_noise_dbm_hz', -174.0))

# 功率裕量门限/斜率：M[dB]=P_rx[dBm]-P_IN[dBm]，Outage=1{M<θ_M}

self.margin_threshold_db = float(user_params.get('margin_threshold_db', 0.0))

self.margin_slope_db = float(user_params.get('margin_slope_db', 3.0))

# CDI 参数（单位约定：路径损耗 dB；干扰强度 I 用 mW 累加，再映射到 dB-like）

self.cdi_pl_max_db = float(user_params.get('cdi_pl_max_db', 125.0))

self.cdi_w_link = float(user_params.get('cdi_w_link', 1.0))

self.cdi_w_int = float(user_params.get('cdi_w_int', 1.0))

# g(I) = 10log10(1 + I/I_ref)，I_ref 用 mW（默认 -30 dBm = 1e-3 mW）

self.cdi_int_ref_mw = float(user_params.get('cdi_int_ref_mw', 1e-3))

# 将 CDI 映射到 [0,1] 的尺度（越大越“饱和”得慢）

self.cdi_scale_db = float(user_params.get('cdi_scale_db', 20.0))

# 可选：在 shot-noise 中加入小尺度衰落 h_i（Rayleigh 功率增益 ~ Exp(1)），并用稳定伪随机保证可复现

self.interference_fading_enabled = bool(user_params.get('interference_fading_enabled', False))

self.signal_fading_enabled = bool(user_params.get('signal_fading_enabled', False))

# 大纲式“功率主导”最终评估参数（概率型输出）

self.objective_model = str(user_params.get('objective_model', 'power_score')).strip().lower()

self.succ_threshold_dbm = float(user_params.get('succ_threshold_dbm', -90.0))  # θ_p：有用接收功率阈值

self.int_threshold_dbm = float(user_params.get('int_threshold_dbm', -80.0))    # 干扰功率阈值

self.pl_max_db = float(user_params.get('pl_max_db', 125.0))                    # PL_max：路径损耗阈值

self.power_score_weights = {

'w1_succ': float(user_params.get('w1_succ', 1.0)),

'w2_int': float(user_params.get('w2_int', 1.0)),

'w3_pl': float(user_params.get('w3_pl', 1.0)),

}

# 可选：U2U 链路纳入同构评估（A2G/U2U 同一套几何+LoS+功率预算，只差高度项）

self.u2u_enabled = bool(user_params.get('u2u_enabled', False))

self.u2u_tx_power_dbm = float(user_params.get('u2u_tx_power_dbm', 23.0))

self.u2u_pairing = str(user_params.get('u2u_pairing', 'all')).strip().lower()  # all | nearest

# 可选：离散时间 TDMA + EE 评估（不改变默认静态场景；仅在 tdma_enabled=True 时启用）

self.tdma_enabled = bool(user_params.get('tdma_enabled', False))

# 总评分（大纲 E3）：D_total = w1 D_comm + w2 D_thr + w3 D_EE + w4 D_penalty

self.outline_weights = {

'w_comm': float(user_params.get('outline_w_comm', 1.0)),

'w_thr': float(user_params.get('outline_w_thr', 0.0)),

'w_ee': float(user_params.get('outline_w_ee', 0.0)),

'w_penalty': float(user_params.get('outline_w_penalty', 0.0)),

}

self.power_model = {

'comm_tx_power_dbm': 20.0,

'baseline_distance_m': 100.0,

'nlos_excess_loss_db': 20.0,

'power_change_scale_db': 15.0,

'speed_power_beta': 1.0,

# 旋翼无人机推进功率模型（直线水平飞行）

# 参考：Y. Zeng et al., IEEE TWC 2019（rotary-wing UAV power model）

'rotor_P0_w': 79.86,      # blade profile power at hover (W)

'rotor_Pi_w': 88.63,      # induced power at hover (W)

'rotor_U_tip_mps': 120.0, # rotor blade tip speed (m/s)

'rotor_v0_mps': 4.03,     # mean rotor induced velocity in hover (m/s)

'rotor_d0': 0.6,          # fuselage drag ratio

'air_rho': 1.225,         # air density (kg/m^3)

'rotor_s': 0.05,          # rotor solidity

'rotor_A_m2': 0.503       # rotor disc area (m^2)

}

# 空对地（ATG）LoS/NLoS 路径损耗模型（Optimal LAP Altitude for Maximum Coverage）

# PL_ξ = FSPL + η_ξ

# Λ = P(LoS)·PL_LoS + (1-P(LoS))·PL_NLoS

# P(LoS,θ) = 1 / (1 + C·exp(-B(θ-C)))，θ为仰角（度）

self.atg_model = {

'eta_los_db': 1.0,

'eta_nlos_db': 20.0,

# 论文/图片中常用记号为 C、B；此处保留历史字段名 los_a/los_b

'los_a': float(user_params.get('los_C', user_params.get('los_a', 9.61))),

'los_b': float(user_params.get('los_B', user_params.get('los_b', 0.16))),

# LoS 概率模型选择：

# - 'sigmoid'：Eq.(5)/(2)，使用 los_a/los_b（默认）

# - 'sigmoid_fit'：Eq.(6) + Table I/II，由 (αβ,γ) 自动拟合计算 a,b

# - 'itu_product'：Eq.(4) 原始乘积形式

'los_model': str(user_params.get('los_model', 'sigmoid_fit')).strip().lower()

}

# LoS 指示量模式（“有地图→几何/射线追踪；无地图→概率模型”）

# - deterministic: 只要线段不穿建筑 => LoS=1（否则0）

# - probabilistic: 始终用概率模型 P_LoS(θ)

# - deterministic_if_map: buildings 不为空时用 deterministic；否则用 probabilistic（默认）

self.los_indicator_mode = str(user_params.get('los_indicator_mode', 'deterministic_if_map')).strip().lower()

# 频谱耦合/邻频泄漏（用于把不同频段干扰源纳入同一 SINR/功率叠加）

# - acir: 近似的 ACIR(Δf) -> 耦合系数，满足“同频=1，邻频<1”

# - lorentzian: 兼容旧实现 1/(1+(Δf/f0)^2)

self.spectral_mode = str(user_params.get('spectral_mode', 'acir')).strip().lower()

self.spectral_model = {

'acir_db_at_1bw': float(user_params.get('acir_db_at_1bw', 30.0)),            # Δf=BW 时的抑制(dB)

'acir_slope_db_per_decade': float(user_params.get('acir_slope_db_per_decade', 20.0)),  # 频偏每十倍增加的抑制(dB)

'acir_db_max': float(user_params.get('acir_db_max', 80.0)),

'lorentz_f0_hz': float(user_params.get('lorentz_f0_hz', 1e8)),

'acir_db_at_1bw_by_type': dict(user_params.get('acir_db_at_1bw_by_type', {})),

'acir_slope_by_type': dict(user_params.get('acir_slope_by_type', {})),

}

# 可选：按环境类型一键覆盖 ATG 参数（不改变默认行为；仅当传入 env_type 时生效）

env_type = str(user_params.get('env_type', '')).strip().lower()

if env_type in self.ATG_ENV_PRESETS:

preset = self.ATG_ENV_PRESETS[env_type]

self.atg_model['eta_los_db'] = float(preset['eta_los_db'])

self.atg_model['eta_nlos_db'] = float(preset['eta_nlos_db'])

self.atg_model['los_a'] = float(preset['los_C'])

self.atg_model['los_b'] = float(preset['los_B'])

# 可选：按 Table I/II 由 ITU 参数拟合得到 a,b（与论文 Eq.(6) 一致）

has_manual_sigmoid_params = ('los_C' in user_params) or ('los_B' in user_params) or ('los_a' in user_params) or ('los_b' in user_params)

if self.atg_model['los_model'] == 'sigmoid_fit' and env_type not in self.ATG_ENV_PRESETS and not has_manual_sigmoid_params:

fitted_a, fitted_b = self._fit_sigmoid_params_from_itu()

self.atg_model['los_a'] = fitted_a

self.atg_model['los_b'] = fitted_b

# 预计算常量：避免在每条链路上重复计算

self._comm_tx_power_dbm = float(self.power_model['comm_tx_power_dbm'])

self._baseline_distance_m = float(self.power_model['baseline_distance_m'])

self._power_change_scale_db = float(self.power_model['power_change_scale_db'])

baseline_pl_db = self.calculate_fspl_db(self._baseline_distance_m) + float(self.atg_model['eta_los_db'])

baseline_rx_dbm = self._comm_tx_power_dbm - baseline_pl_db

self._baseline_rx_mw = self._dbm_to_mw(baseline_rx_dbm)

def is_los(self,

tx_pos: np.ndarray,

rx_pos: np.ndarray,

buildings: Optional[List['Building']] = None,

ignore_building_idx: Optional[int] = None) -> bool:

"""

统一 LoS 判定接口（确定性）：线段 tx->rx 是否与任意建筑棱柱体相交。

- 有 buildings：用几何遮挡作为 LoS 指示量

- 无 buildings：视为 LoS（交给概率模型的 fallback 做统计口径）

"""

buildings_list = buildings if buildings is not None else []

if not buildings_list:

return True

tx_pos = np.asarray(tx_pos, dtype=float)

rx_pos = np.asarray(rx_pos, dtype=float)

if buildings_list is self.buildings:

blocked = self._is_segment_blocked_by_buildings(tx_pos, rx_pos, ignore_building_idx=ignore_building_idx)

else:

blocked = self._is_segment_blocked_by_buildings_naive(tx_pos, rx_pos, buildings_list, ignore_building=None)

return not bool(blocked)

def _spectral_coupling_factor(self,

tx_freq_hz: float,

rx_freq_hz: float,

rx_bw_hz: float,

interferer_type_id: Optional[int] = None) -> float:

"""

频谱耦合系数 η(Δf)∈(0,1]：

- 同频（|Δf|<=BW/2）: η=1

- 邻频/异频：用简化 ACIR(Δf) -> 10^(-ACIR/10)

"""

df = abs(float(tx_freq_hz) - float(rx_freq_hz))

bw = max(float(rx_bw_hz), 1.0)

if df <= 0.5 * bw:

return 1.0

mode = str(getattr(self, 'spectral_mode', 'acir')).strip().lower()

# 论文同款（Zeng 2019）常用：只建模同频 cochannel 干扰；异频按 0 处理

if mode in ('cochannel', 'cochannel_only', 'same_channel'):

return 0.0

if mode in ('lorentz', 'lorentzian', 'legacy'):

f0 = max(float(self.spectral_model.get('lorentz_f0_hz', 1e8)), 1.0)

return float(1.0 / (1.0 + (df / f0) ** 2))

# acir

base = float(self.spectral_model.get('acir_db_at_1bw', 30.0))

slope = float(self.spectral_model.get('acir_slope_db_per_decade', 20.0))

if interferer_type_id is not None:

by_type = self.spectral_model.get('acir_db_at_1bw_by_type', {}) or {}

if str(int(interferer_type_id)) in by_type:

base = float(by_type[str(int(interferer_type_id))])

elif int(interferer_type_id) in by_type:

base = float(by_type[int(interferer_type_id)])

slope_by = self.spectral_model.get('acir_slope_by_type', {}) or {}

if str(int(interferer_type_id)) in slope_by:

slope = float(slope_by[str(int(interferer_type_id))])

elif int(interferer_type_id) in slope_by:

slope = float(slope_by[int(interferer_type_id)])

ratio = max(df / bw, 1.0)

acir_db = float(base + slope * np.log10(ratio))

acir_db = float(np.clip(acir_db, 0.0, float(self.spectral_model.get('acir_db_max', 80.0))))

return float(10.0 ** (-acir_db / 10.0))

@staticmethod

def _dbm_to_mw(power_dbm: float) -> float:

return float(10 ** (power_dbm / 10.0))

@staticmethod

def _mw_to_dbm(power_mw: float) -> float:

power_mw = max(float(power_mw), 1e-15)

return float(10.0 * np.log10(power_mw))

def _fit_sigmoid_params_from_itu(self) -> Tuple[float, float]:

"""

使用 Al-Hourani 2014 的 surface fitting（Eq.(6) + Table I/II）

由 ITU 城市参数 (α,β,γ) 计算 Sigmoid 的 a,b（θ:度）。

说明：论文中使用 x=(αβ)（β: buildings/km^2）, y=γ（m）。

"""

alpha = float(np.clip(self.itu_city['alpha'], 0.01, 0.99))

beta_km2 = max(float(self.itu_city['beta_km2']), 1e-9)

gamma = max(float(self.itu_city['gamma_m']), 1e-9)

x = alpha * beta_km2

y = gamma

def eval_poly(coeffs: Dict[int, Dict[int, float]]) -> float:

z = 0.0

for j, row in coeffs.items():

for i, c_ij in row.items():

z += float(c_ij) * (x ** int(i)) * (y ** int(j))

return float(z)

a = eval_poly(self.LOS_A_COEFFS)

b = eval_poly(self.LOS_B_COEFFS)

# 保护性裁剪：避免极端 ITU 参数导致 Sigmoid 失效

a = float(np.clip(a, 0.1, 50.0))

b = float(np.clip(b, 1e-4, 5.0))

return a, b

def _generate_fixed_scene_elements(self):

"""生成固定的场景元素（建筑物、地面站位置）"""

# 生成建筑物

self._generate_buildings()

self._building_observables_cache = self._compute_building_observables()

wifi_density = self._estimate_wifi_ap_density(self._building_observables_cache)

self._wifi_density_cache = wifi_density if wifi_density is not None else {}

# 生成地面站位置

self._generate_station_positions()

# 生成无人机初始水平位置

self._generate_drone_xy_positions()

# 构建建筑物空间索引（用于干扰源“落楼”和遮挡加速）

self._build_building_spatial_index()

# 生成城市场景“语义标签层”（用于干扰源场景一致性约束/投影）

self._generate_semantic_layers()

# 生成背景干扰源（大量、多类型、多高度；不进入染色体）

self.background_interference_sources = []

self.background_interference_sources_viz = []

if self.background_interference_enabled:

self._generate_background_interference_sources()

def _generate_buildings(self):

"""生成建筑物"""

# 按 ITU 城市统计参数生成：

# - 数量由 β（buildings/km^2）决定

# - 高度服从 Rayleigh(γ)

# - 平面占地总面积近似满足覆盖率 α

max_buildings = 500

alpha = float(np.clip(self.itu_city['alpha'], 0.01, 0.99))

beta_km2 = max(float(self.itu_city['beta_km2']), 1e-6)

beta_m2 = beta_km2 / 1e6

area_m2 = float(self.area_size ** 2)

num_buildings = int(beta_m2 * area_m2)

num_buildings = max(0, min(num_buildings, max_buildings))

self.buildings = []

if num_buildings <= 0:

return

target_footprint_area = alpha * area_m2

avg_footprint_area = max(target_footprint_area / num_buildings, 25.0)

base_side = float(np.sqrt(avg_footprint_area))

gamma = max(float(self.itu_city['gamma_m']), 1e-6)

for _ in range(num_buildings):

# 建筑高度：Rayleigh 分布

height = float(np.random.rayleigh(scale=gamma))

height = float(np.clip(height, 5.0, self.building_height_max))

# 建筑平面尺寸：围绕平均占地面积随机扰动（再裁剪到合理范围）

width = float(base_side * np.random.uniform(0.6, 1.4))

length = float((avg_footprint_area / max(width, 1.0)) * np.random.uniform(0.6, 1.4))

width = float(np.clip(width, 10.0, 80.0))

length = float(np.clip(length, 10.0, 80.0))

# 随机位置（确保建筑物完全落在区域内）

margin = 5.0

x_min = width / 2 + margin

x_max = self.area_size - width / 2 - margin

y_min = length / 2 + margin

y_max = self.area_size - length / 2 - margin

x = float(np.random.uniform(x_min, x_max)) if x_min < x_max else float(self.area_size / 2)

y = float(np.random.uniform(y_min, y_max)) if y_min < y_max else float(self.area_size / 2)

building = Building(x, y, height, width, length)

# 便于调试/索引：记录建筑物序号

building.idx = len(self.buildings)

self.buildings.append(building)

# 记录 W,S（便于报告与调参）：W=1000*sqrt(alpha/beta), S=(1000/sqrt(beta))*(1-sqrt(alpha))

alpha_cov = float(np.clip(self.itu_city['alpha'], 0.01, 0.99))

beta_km2_val = max(float(self.itu_city['beta_km2']), 1e-6)

self.building_geom_W_m = float(1000.0 * np.sqrt(alpha_cov / beta_km2_val))

self.building_geom_S_m = float((1000.0 / np.sqrt(beta_km2_val)) * (1.0 - np.sqrt(alpha_cov)))

def _compute_building_observables(self) -> Dict:

buildings = list(getattr(self, 'buildings', []) or [])

floor_h = max(float(getattr(self, 'floor_height_m', 3.0)), 0.5)

obs = {

'floor_height_m': float(floor_h),

'building_count': int(len(buildings)),

'total_footprint_m2': 0.0,

'total_volume_m3': 0.0,

'total_floor_area_m2': 0.0,

'avg_footprint_m2': 0.0,

'avg_floors': 0.0,

'avg_floor_area_m2': 0.0,

'per_building': []

}

if not buildings:

return obs

total_footprint = 0.0

total_volume = 0.0

total_floor_area = 0.0

total_floors = 0.0

per_building = []

for idx, b in enumerate(buildings):

footprint = float(b.footprint_area_m2())

floors = int(b.floors(floor_h))

floor_area = float(footprint * float(floors))

volume = float(footprint * float(b.height))

per_building.append({

'id': int(getattr(b, 'idx', idx)),

'footprint_m2': footprint,

'volume_m3': volume,

'floors': int(floors),

'floor_area_m2': float(floor_area)

})

total_footprint += footprint

total_volume += volume

total_floor_area += floor_area

total_floors += float(floors)

count = float(len(buildings))

obs.update({

'total_footprint_m2': float(total_footprint),

'total_volume_m3': float(total_volume),

'total_floor_area_m2': float(total_floor_area),

'avg_footprint_m2': float(total_footprint / count),

'avg_floors': float(total_floors / count),

'avg_floor_area_m2': float(total_floor_area / count),

'per_building': per_building

})

return obs

def _estimate_wifi_ap_density(self, building_obs: Optional[Dict] = None) -> Optional[Dict]:

model = str(self.user_params.get('wifi_density_model', 'floor_area')).strip().lower()

if model in ('legacy', 'fixed', 'off', 'none', 'disable'):

return None

area_km2 = float((self.area_size / 1000.0) ** 2)

if area_km2 <= 0.0:

return None

if building_obs is None:

building_obs = self._compute_building_observables()

total_floor_m2 = float(building_obs.get('total_floor_area_m2', 0.0))

if total_floor_m2 <= 0.0:

return None

def clamp01(value: float, default: float) -> float:

try:

v = float(value)

except Exception:

v = float(default)

return float(np.clip(v, 0.0, 1.0))

def safe_pos(value: float, default: float) -> float:

try:

v = float(value)

except Exception:

v = float(default)

if v <= 0.0:

v = float(default)

return float(v)

eta_res = clamp01(self.user_params.get('wifi_eta_res', 0.6), 0.6)

eta_bus_raw = self.user_params.get('wifi_eta_bus', None)

if eta_bus_raw is None:

eta_bus = max(0.0, 1.0 - eta_res)

else:

eta_bus = clamp01(eta_bus_raw, max(0.0, 1.0 - eta_res))

total_share = float(eta_res + eta_bus)

if total_share > 1.0:

eta_res = float(eta_res / total_share)

eta_bus = float(eta_bus / total_share)

a_dwell = safe_pos(self.user_params.get('wifi_avg_dwell_area_m2', 80.0), 80.0)

kappa_res = max(0.0, float(self.user_params.get('wifi_res_router_per_dwell', 1.0)))

p_res = clamp01(self.user_params.get('wifi_res_adoption', 0.8), 0.8)

n_dwell = float(eta_res * total_floor_m2 / a_dwell)

n_res = float(kappa_res * p_res * n_dwell)

business_model = str(self.user_params.get('wifi_business_model', 'coverage_area')).strip().lower()

bus_detail: Dict[str, float] = {}

if business_model in ('office_unit', 'office', 'unit', 'b1'):

a_office = safe_pos(self.user_params.get('wifi_avg_office_area_m2', 120.0), 120.0)

kappa_bus = max(0.0, float(self.user_params.get('wifi_bus_ap_per_office', 1.0)))

p_bus = clamp01(self.user_params.get('wifi_bus_adoption', 0.8), 0.8)

n_office = float(eta_bus * total_floor_m2 / a_office)

n_bus = float(kappa_bus * p_bus * n_office)

bus_detail = {

'business_model': 'office_unit',

'A_office_m2': float(a_office),

'kappa_bus': float(kappa_bus),

'p_bus': float(p_bus),

'N_office': float(n_office)

}

else:

alpha_bus = clamp01(self.user_params.get('wifi_bus_coverage_ratio', 0.7), 0.7)

c_ap = safe_pos(self.user_params.get('wifi_ap_coverage_area_m2', 200.0), 200.0)

a_bus_floor = float(eta_bus * total_floor_m2)

n_bus = float(alpha_bus * a_bus_floor / c_ap)

bus_detail = {

'business_model': 'coverage_area',

'alpha_bus': float(alpha_bus),

'C_ap_m2': float(c_ap),

'A_bus_floor_m2': float(a_bus_floor)

}

n_wifi = float(n_res + n_bus)

lambda_wifi = float(n_wifi / area_km2)

f_24 = clamp01(self.user_params.get('wifi_band_share_24', 0.6), 0.6)

lambda_24_base = float(lambda_wifi * f_24)

lambda_58_base = float(lambda_wifi * (1.0 - f_24))

p_tx = clamp01(self.user_params.get('wifi_tx_activity_prob', 1.0), 1.0)

lambda_24 = float(lambda_24_base * p_tx)

lambda_58 = float(lambda_58_base * p_tx)

return {

'model': str(model),

'area_km2': float(area_km2),

'eta_res': float(eta_res),

'eta_bus': float(eta_bus),

'A_dwell_m2': float(a_dwell),

'kappa_res': float(kappa_res),

'p_res': float(p_res),

'N_dwell': float(n_dwell),

'N_res': float(n_res),

'N_bus': float(n_bus),

'N_wifi': float(n_wifi),

'lambda_wifi_km2': float(lambda_wifi),

'lambda_24_base_km2': float(lambda_24_base),

'lambda_58_base_km2': float(lambda_58_base),

'lambda_24_km2': float(lambda_24),

'lambda_58_km2': float(lambda_58),

'f_24': float(f_24),

'p_tx': float(p_tx),

**bus_detail

}

def _generate_background_interference_sources(self):

"""生成背景干扰源（2D PPP 投影 + 高度采样，混合地面/屋顶/楼层）。"""

area_km2 = float((self.area_size / 1000.0) ** 2)

# 每类干扰源的“面密度”（buildings/km^2 同量纲：sources/km^2），可由 scale 缩放

base_lambda_per_km2 = {

0: 1854.0,  # WiFi 2.4

1: 1854.0,  # WiFi 5.8

2: 6.0,   # 4G

3: 8.0,   # 5G

4: 1.0,   # GNSS jammer

5: 6.0,   # industrial

6: 0.5,   # satellite ground

7: 0.0,   # UE UL（默认不启用；可用 bg_lambda_km2_7 显式打开）

}

# 每类的高度/位置混合（地面/屋顶/室内楼层）

building_obs = self._compute_building_observables()

self._building_observables_cache = building_obs

wifi_density = self._estimate_wifi_ap_density(building_obs)

self._wifi_density_cache = wifi_density if wifi_density is not None else {}

if wifi_density:

if 'lambda_24_km2' in wifi_density:

base_lambda_per_km2[0] = float(max(0.0, wifi_density['lambda_24_km2']))

if 'lambda_58_km2' in wifi_density:

base_lambda_per_km2[1] = float(max(0.0, wifi_density['lambda_58_km2']))

loc_mix = {

0: {'ground': 0.10, 'rooftop': 0.10, 'indoor': 0.80},

1: {'ground': 0.10, 'rooftop': 0.10, 'indoor': 0.80},

2: {'ground': 0.05, 'rooftop': 0.90, 'indoor': 0.05},

3: {'ground': 0.10, 'rooftop': 0.85, 'indoor': 0.05},

4: {'ground': 0.90, 'rooftop': 0.05, 'indoor': 0.05},

5: {'ground': 0.30, 'rooftop': 0.20, 'indoor': 0.50},

6: {'ground': 0.95, 'rooftop': 0.05, 'indoor': 0.00},

7: {'ground': 0.30, 'rooftop': 0.05, 'indoor': 0.65},

}

# 允许用户用 user_params 覆盖某些密度（不会在交互里强制询问）

for type_id in list(base_lambda_per_km2.keys()):

key = f'bg_lambda_km2_{type_id}'

if key in self.user_params:

base_lambda_per_km2[type_id] = float(self.user_params[key])

sources = []

for type_id, lam in base_lambda_per_km2.items():

density_mode = str(getattr(self, 'background_interference_density_mode', '2d')).strip().lower()

scale = float(getattr(self, 'background_interference_density_scale', 1.0))

if density_mode == '3d':

# ρ（sources/m^3） -> λ（sources/km^2）：λ = ρ * Z * 1e6

z_box = max(float(getattr(self, 'background_interference_box_height_m', self.building_height_max)), 1.0)

rho_key = f'bg_rho_m3_{int(type_id)}'

if rho_key in self.user_params:

rho_m3 = max(0.0, float(self.user_params[rho_key]))

lam_eff = float(rho_m3 * z_box * 1e6 * scale)

else:

lam_eff = float(max(0.0, lam) * scale)

else:

lam_eff = float(max(0.0, lam) * scale)

if lam_eff <= 0.0:

continue

n = int(np.random.poisson(lam_eff * area_km2))

if n <= 0:

continue

cfg = self.interference_config.get_config(int(type_id))

min_p, max_p = cfg['power_range']

for _ in range(n):

mix = loc_mix.get(int(type_id), {'ground': 1.0, 'rooftop': 0.0, 'indoor': 0.0})

allowed = set(self.interference_config.get_allowed_semantics(int(type_id)))

u = float(np.random.random())

if not self.buildings:

placement = 'ground'

else:

candidates = []

weights = []

if 'indoor' in allowed:

candidates.append('indoor'); weights.append(float(mix.get('indoor', 0.0)))

if any(s in allowed for s in ('roof', 'tower', 'pole')):

candidates.append('rooftop'); weights.append(float(mix.get('rooftop', 0.0)))

if any(s in allowed for s in ('street', 'sidewalk', 'ground')):

candidates.append('ground'); weights.append(float(mix.get('ground', 0.0)))

if not candidates:

candidates = ['ground', 'rooftop', 'indoor']

weights = [float(mix.get('ground', 0.0)), float(mix.get('rooftop', 0.0)), float(mix.get('indoor', 0.0))]

ws = float(sum(weights))

if ws <= 1e-12:

placement = candidates[int(u * len(candidates)) % len(candidates)]

else:

r = u * ws

acc = 0.0

placement = candidates[-1]

for c, w in zip(candidates, weights):

acc += float(w)

if r <= acc:

placement = c

break

power = float(np.random.uniform(min_p, max_p))

if placement == 'ground':

# 先按原公式采样，再做“场景一致性”重采样/投影

x = float(np.random.uniform(0.0, self.area_size))

y = float(np.random.uniform(0.0, self.area_size))

if self._building_idx_containing_xy(x, y) is not None:

# 地面源不允许落进建筑平面，重采样

for _try in range(int(self.scene_consistency_max_attempts)):

x = float(np.random.uniform(0.0, self.area_size))

y = float(np.random.uniform(0.0, self.area_size))

if self._building_idx_containing_xy(x, y) is None:

break

semantic = self._classify_semantic(x, y, 0.0, building=None, is_indoor=False)

if allowed and semantic not in allowed and 'ground' not in allowed:

# 目标优先级：street > sidewalk > ground

target = 'street' if 'street' in allowed else ('sidewalk' if 'sidewalk' in allowed else 'ground')

if str(self.scene_consistency_mode) == 'project':

x, y = self._project_xy_to_semantic(x, y, target)

semantic = self._classify_semantic(x, y, 0.0, building=None, is_indoor=False)

else:

ok = False

for _try in range(int(self.scene_consistency_max_attempts)):

x = float(np.random.uniform(0.0, self.area_size))

y = float(np.random.uniform(0.0, self.area_size))

if self._building_idx_containing_xy(x, y) is not None:

continue

semantic = self._classify_semantic(x, y, 0.0, building=None, is_indoor=False)

if semantic in allowed:

ok = True

break

if not ok:

x, y = self._project_xy_to_semantic(x, y, target)

semantic = self._classify_semantic(x, y, 0.0, building=None, is_indoor=False)

src = InterferenceSource(x, y, 0.0, int(type_id), power,

building=None, is_ground=True,

building_idx=None, is_indoor=False, skip_height_clip=True)

src.semantic = semantic

idx_local = int(len(sources))

self._calibrate_source_profile_and_power(idx_local, src, float(power))

src.activity = float(self._activity_factor_for_source(idx_local, src))

sources.append(src)

continue

# rooftop / indoor：随机选择建筑

b_idx = int(np.random.randint(0, len(self.buildings)))

b = self.buildings[b_idx]

x = float(np.random.uniform(b.x - b.width / 2, b.x + b.width / 2))

y = float(np.random.uniform(b.y - b.length / 2, b.y + b.length / 2))

if placement == 'rooftop':

rooftop_offset_m = float(cfg.get('rooftop_offset_m', 2.0))

h = float(b.height + rooftop_offset_m)

src = InterferenceSource(x, y, h, int(type_id), power,

building=b, is_ground=False,

building_idx=b_idx, is_indoor=False, skip_height_clip=True)

src.semantic = 'roof'

idx_local = int(len(sources))

self._calibrate_source_profile_and_power(idx_local, src, float(power))

src.activity = float(self._activity_factor_for_source(idx_local, src))

sources.append(src)

continue

# indoor：按楼层离散化（楼层高 floor_height_m），高度 = (floor-1)*H + indoor_device_height_m

floor_h = max(float(self.floor_height_m), 0.5)

floors_total = max(1, int(math.floor(float(b.height) / floor_h)))

floor = int(np.random.randint(1, floors_total + 1))

h = float((floor - 1) * floor_h + float(self.indoor_device_height_m))

indoor_d = float(self.p1238_d_in_m)

src = InterferenceSource(x, y, h, int(type_id), power,

building=b, is_ground=False,

building_idx=b_idx, is_indoor=True,

indoor_floor=floor, indoor_floors_total=floors_total,

indoor_distance_m=indoor_d,

skip_height_clip=True)

src.semantic = 'indoor'

idx_local = int(len(sources))

self._calibrate_source_profile_and_power(idx_local, src, float(power))

src.activity = float(self._activity_factor_for_source(idx_local, src))

sources.append(src)

# 限制背景源总数，避免极端参数导致性能问题（保持确定性：按功率排序取前 max）

if len(sources) > int(self.background_interference_max_sources):

sources.sort(key=lambda s: float(s.power), reverse=True)

sources = sources[:int(self.background_interference_max_sources)]

self.background_interference_sources = sources

# 可视化只取少量（同样按功率排序取前 max_sources_to_plot）

viz_n = max(0, int(self.max_sources_to_plot))

if viz_n > 0:

viz_sources = list(sources)

viz_sources.sort(key=lambda s: float(s.power), reverse=True)

self.background_interference_sources_viz = viz_sources[:viz_n]

else:

self.background_interference_sources_viz = []

def _build_building_spatial_index(self):

"""构建建筑物空间索引（网格），加速点落楼与线段遮挡判断。"""

self._building_grid = {}

self._building_grid_cell_size = float(max(30.0, min(150.0, self.area_size / 20.0)))

if not getattr(self, 'buildings', None):

self._building_centers_xy = np.zeros((0, 2), dtype=float)

return

centers = []

cs = self._building_grid_cell_size

for idx, b in enumerate(self.buildings):

centers.append([float(b.x), float(b.y)])

xmin = float(b.x - b.width / 2)

xmax = float(b.x + b.width / 2)

ymin = float(b.y - b.length / 2)

ymax = float(b.y + b.length / 2)

cx0 = int(math.floor(xmin / cs))

cx1 = int(math.floor(xmax / cs))

cy0 = int(math.floor(ymin / cs))

cy1 = int(math.floor(ymax / cs))

for cx in range(cx0, cx1 + 1):

for cy in range(cy0, cy1 + 1):

self._building_grid.setdefault((cx, cy), []).append(idx)

self._building_centers_xy = np.asarray(centers, dtype=float)

def _generate_semantic_layers(self):

"""

生成“场景标签层”（不改变原生成公式，只用于场景一致性约束/投影）：

- street：道路车行带

- sidewalk：人行道

- ground：其余地面（街区/空地）

- roof：屋顶

- indoor：室内楼层

- facade：建筑外立面附近（近似）

- pole：路侧杆塔点（近似）

"""

spacing = float(getattr(self, 'semantic_road_spacing_m', 0.0))

if spacing <= 1e-6:

spacing = float(getattr(self, 'building_geom_S_m', 0.0))

if spacing <= 1e-6:

spacing = float(max(80.0, self.area_size / 5.0))

spacing = float(np.clip(spacing * 2.0, 80.0, 300.0))

road_w = float(max(getattr(self, 'semantic_road_width_m', 20.0), 5.0))

sidewalk_w = float(max(getattr(self, 'semantic_sidewalk_width_m', 6.0), 0.0))

facade_band = float(max(getattr(self, 'semantic_facade_band_m', 2.0), 0.0))

pole_snap = float(max(getattr(self, 'semantic_pole_snap_m', 3.0), 0.0))

self.semantic_layers = {

'road_spacing_m': spacing,

'road_width_m': road_w,

'sidewalk_width_m': sidewalk_w,

'facade_band_m': facade_band,

'pole_snap_m': pole_snap,

}

def _road_distance_components(self, x: float, y: float) -> Tuple[float, float]:

"""到最近竖向/横向道路中心线的距离（m）。"""

spacing = float(getattr(self, 'semantic_layers', {}).get('road_spacing_m', 100.0))

if spacing <= 1e-9:

return float('inf'), float('inf')

xm = float(x) % spacing

ym = float(y) % spacing

dv = min(xm, spacing - xm)

dh = min(ym, spacing - ym)

return float(dv), float(dh)

def _classify_semantic(self,

x: float,

y: float,

z: float,

building: Optional[Building] = None,

is_indoor: bool = False) -> str:

if is_indoor:

return 'indoor'

if building is not None and z >= float(building.height) - 1e-6:

return 'roof'

# 关键修复：若已经“绑定到建筑”，但高度在屋顶以下且未标记室内，

# 则不应被归类为 street/sidewalk（会导致“干扰源漂浮在空中”的错觉/不一致）。

# 统一把这类点视作建筑相关的“facade”（外立面设备）语义，后续再由 Γ(type) 约束转为 indoor/roof/pole 等合法集合。

if building is not None and z > 0.5:

return 'facade'

# 空域：离地且不在建筑内

if z > 1.0 and building is None:

return 'air'

dv, dh = self._road_distance_components(x, y)

road_half = float(getattr(self, 'semantic_layers', {}).get('road_width_m', 20.0)) / 2.0

sidewalk_w = float(getattr(self, 'semantic_layers', {}).get('sidewalk_width_m', 6.0))

d_road = min(dv, dh)

if d_road <= road_half:

return 'street'

if d_road <= road_half + sidewalk_w:

pole_snap = float(getattr(self, 'semantic_layers', {}).get('pole_snap_m', 3.0))

if dv <= road_half + sidewalk_w and dh <= road_half + sidewalk_w and min(dv, dh) <= pole_snap:

return 'pole'

return 'sidewalk'

return 'ground'

def _project_xy_to_semantic(self, x: float, y: float, target: str) -> Tuple[float, float]:

"""将 (x,y) 投影到最近的目标语义集合（简化几何投影）。"""

x = float(np.clip(x, 0.0, self.area_size))

y = float(np.clip(y, 0.0, self.area_size))

spacing = float(getattr(self, 'semantic_layers', {}).get('road_spacing_m', 100.0))

if spacing <= 1e-9:

return x, y

road_half = float(getattr(self, 'semantic_layers', {}).get('road_width_m', 20.0)) / 2.0

sidewalk_w = float(getattr(self, 'semantic_layers', {}).get('sidewalk_width_m', 6.0))

kx = round(x / spacing)

ky = round(y / spacing)

cx = float(kx * spacing)

cy = float(ky * spacing)

dv, dh = self._road_distance_components(x, y)

if target == 'street':

if dv <= dh:

return float(np.clip(cx, 0.0, self.area_size)), y

return x, float(np.clip(cy, 0.0, self.area_size))

if target in ('sidewalk', 'pole'):

offset = road_half + sidewalk_w / 2.0

if target == 'pole':

x0 = float(np.clip(cx, 0.0, self.area_size))

y0 = float(np.clip(cy, 0.0, self.area_size))

return float(np.clip(x0 + offset, 0.0, self.area_size)), float(np.clip(y0 + offset, 0.0, self.area_size))

if dv <= dh:

sign = 1.0 if (x - cx) >= 0.0 else -1.0

return float(np.clip(cx + sign * offset, 0.0, self.area_size)), y

sign = 1.0 if (y - cy) >= 0.0 else -1.0

return x, float(np.clip(cy + sign * offset, 0.0, self.area_size))

return x, y

def _building_idx_containing_xy(self, x: float, y: float) -> Optional[int]:

"""若 (x,y) 落在某建筑平面投影内，返回其索引，否则返回 None。"""

if not getattr(self, 'buildings', None):

return None

cs = float(getattr(self, '_building_grid_cell_size', 100.0))

cx = int(math.floor(float(x) / cs))

cy = int(math.floor(float(y) / cs))

for idx in self._building_grid.get((cx, cy), []):

b = self.buildings[int(idx)]

if b.contains_point(float(x), float(y)):

return int(idx)

return None

def _find_or_snap_building(self, x: float, y: float) -> Tuple[Optional[int], Optional[Building]]:

"""根据 (x,y) 找到包含该点的建筑；若没有，返回最近建筑（用于 location_type=建筑物）。"""

if not self.buildings:

return None, None

cs = float(getattr(self, '_building_grid_cell_size', 100.0))

cx = int(math.floor(float(x) / cs))

cy = int(math.floor(float(y) / cs))

for idx in self._building_grid.get((cx, cy), []):

b = self.buildings[idx]

if b.contains_point(float(x), float(y)):

return int(idx), b

# 未落在任何建筑 footprint 内：确定性地“吸附”到最近建筑

centers = getattr(self, '_building_centers_xy', None)

if centers is None or len(centers) == 0:

return None, None

dx = centers[:, 0] - float(x)

dy = centers[:, 1] - float(y)

nearest_idx = int(np.argmin(dx * dx + dy * dy))

return nearest_idx, self.buildings[nearest_idx]

def _candidate_building_indices_for_segment(self, p0: np.ndarray, p1: np.ndarray) -> List[int]:

"""根据线段 (p0,p1) 的 XY 包围盒，返回可能相交的建筑索引集合（去重）。"""

if not self.buildings:

return []

cs = float(getattr(self, '_building_grid_cell_size', 100.0))

x0 = float(p0[0]); y0 = float(p0[1])

x1 = float(p1[0]); y1 = float(p1[1])

xmin = min(x0, x1); xmax = max(x0, x1)

ymin = min(y0, y1); ymax = max(y0, y1)

cx0 = int(math.floor(xmin / cs))

cx1 = int(math.floor(xmax / cs))

cy0 = int(math.floor(ymin / cs))

cy1 = int(math.floor(ymax / cs))

idx_set = set()

for cx in range(cx0, cx1 + 1):

for cy in range(cy0, cy1 + 1):

for idx in self._building_grid.get((cx, cy), []):

idx_set.add(int(idx))

return list(idx_set)

def _is_segment_blocked_by_buildings(self,

p0: np.ndarray,

p1: np.ndarray,

ignore_building_idx: Optional[int] = None) -> bool:

"""判断线段 p0->p1 是否被任何建筑遮挡（可忽略干扰源所在建筑）。"""

candidates = self._candidate_building_indices_for_segment(p0, p1)

if not candidates:

return False

for idx in candidates:

if ignore_building_idx is not None and int(idx) == int(ignore_building_idx):

continue

if self.buildings[idx].intersects_segment_3d(p0, p1):

return True

return False

def _generate_station_positions(self):

"""生成地面站位置"""

self.station_positions = []

# 在地面均匀分布地面站

for i in range(self.num_stations):

if self.num_stations == 1:

x = self.area_size / 2

y = self.area_size / 2

elif self.num_stations == 2:

x = self.area_size * (i + 1) / 3

y = self.area_size / 2

elif self.num_stations == 3:

angle = 2 * np.pi * i / 3

x = self.area_size / 2 + self.area_size / 3 * np.cos(angle)

y = self.area_size / 2 + self.area_size / 3 * np.sin(angle)

else:

# 网格分布

rows = int(np.sqrt(self.num_stations))

cols = int(np.ceil(self.num_stations / rows))

row = i // cols

col = i % cols

x = (col + 0.5) * self.area_size / cols

y = (row + 0.5) * self.area_size / rows

# 避免站点落在建筑内部（启用“几何 LoS”时会导致链路被必然遮挡）

x = float(x)

y = float(y)

for _ in range(30):

if not any(b.contains_point(x, y) for b in getattr(self, 'buildings', []) or []):

break

x = float(np.random.uniform(0.0, self.area_size))

y = float(np.random.uniform(0.0, self.area_size))

self.station_positions.append([x, y, 2.0])  # 地面站高度2m

self.station_positions = np.array(self.station_positions)

def _generate_drone_xy_positions(self):

"""生成无人机水平位置"""

xy = np.random.rand(self.num_drones, 2) * self.area_size

# 同样避免无人机初始水平位置落在建筑内部（几何 LoS 下会产生不合理的“穿楼起飞”链路）

for i in range(int(self.num_drones)):

x = float(xy[i, 0])

y = float(xy[i, 1])

for _ in range(30):

if not any(b.contains_point(x, y) for b in getattr(self, 'buildings', []) or []):

break

x = float(np.random.uniform(0.0, self.area_size))

y = float(np.random.uniform(0.0, self.area_size))

xy[i, 0] = x

xy[i, 1] = y

self.drone_xy_positions = xy

@staticmethod

def _is_segment_blocked_by_buildings_naive(p0: np.ndarray,

p1: np.ndarray,

buildings: List[Building],

ignore_building: Optional[Building] = None) -> bool:

"""无遮挡加速索引时的兜底：逐栋检查线段是否与建筑体相交。"""

if not buildings:

return False

for b in buildings:

if ignore_building is not None and b is ignore_building:

continue

if b.intersects_segment_3d(p0, p1):

return True

return False

def check_dimensions(self):

"""检查维度是否正确"""

print("\n维度检查:")

print(f"无人机数量: {self.num_drones}")

print(f"干扰源数量: {self.num_interference}")

print(f"Geatpy维度: {self.Dim}")

# 验证各个部分的维度

total_vars = (self.num_drones * 2 +  # 无人机高度和速度

self.num_interference * 5)  # 干扰源类型、功率、位置类型、位置(x,y)

print(f"计算总维度: {total_vars}")

if self.Dim != total_vars:

print(f"错误: 维度不匹配! Geatpy维度: {self.Dim}, 计算维度: {total_vars}")

return False

else:

print("维度正确!")

return True

def generate_scenario(self, x: np.ndarray) -> Dict:

"""根据染色体生成场景"""

idx = 0

# 1. 无人机高度

drone_heights = x[idx:idx + self.num_drones]

idx += self.num_drones

# 2. 无人机速度

drone_speeds = x[idx:idx + self.num_drones]

idx += self.num_drones

# 无人机完整位置

drone_positions = np.zeros((self.num_drones, 3))

drone_positions[:, :2] = self.drone_xy_positions

drone_positions[:, 2] = drone_heights

# 3. 干扰源类型

interference_types = [int(t) for t in x[idx:idx + self.num_interference]]

idx += self.num_interference

# 4. 干扰源功率

interference_powers = x[idx:idx + self.num_interference]

idx += self.num_interference

# 5. 干扰源位置类型 (0=地面, 1=建筑物)

interference_location_types = [int(t) for t in x[idx:idx + self.num_interference]]

idx += self.num_interference

# 6. 干扰源位置

interference_positions = x[idx:idx + self.num_interference * 2]

# 生成“受控干扰源”（由染色体控制）

controlled_sources = []

for i in range(self.num_interference):

type_id = interference_types[i]

power = interference_powers[i]

location_type = interference_location_types[i]

# 获取位置坐标

pos_x = interference_positions[i * 2]

pos_y = interference_positions[i * 2 + 1]

# 根据位置类型确定高度和建筑物

if location_type == 1:  # 建筑物上

# 先裁剪到区域内，再根据 (x,y) 找到/吸附到最近建筑

pos_x = float(np.clip(pos_x, 0.0, self.area_size))

pos_y = float(np.clip(pos_y, 0.0, self.area_size))

building_idx, building = self._find_or_snap_building(pos_x, pos_y)

if building is None:

# 没有建筑物可用，退化为地面干扰源

source = InterferenceSource(pos_x, pos_y, 0.0, type_id, power, None, is_ground=True, building_idx=None)

else:

# 确保位置在建筑物范围内

pos_x = float(np.clip(pos_x, building.x - building.width / 2, building.x + building.width / 2))

pos_y = float(np.clip(pos_y, building.y - building.length / 2, building.y + building.length / 2))

# 根据干扰源类型确定高度（确定性选择，降低 GA 评估噪声）

config = self.interference_config.get_config(type_id)

suitable_heights = building.get_suitable_interference_heights(config['name'])

height = self._select_deterministic_height(i, type_id, power, pos_x, pos_y, suitable_heights)

source = InterferenceSource(pos_x, pos_y, height, type_id, power, building,

is_ground=False, building_idx=building_idx)

else:  # 地面干扰源

pos_x = float(np.clip(pos_x, 0.0, self.area_size))

pos_y = float(np.clip(pos_y, 0.0, self.area_size))

height = 0.0

source = InterferenceSource(pos_x, pos_y, height, type_id, power, None, is_ground=True, building_idx=None)

source = self._apply_scene_constraints_to_controlled_source(int(i), source)

# 基于“可追溯 profile”对功率做标定（不改变染色体维度，只改变解释/映射）

self._calibrate_source_profile_and_power(int(i), source, float(power))

if not hasattr(source, 'semantic'):

source.semantic = self._classify_semantic(float(source.x), float(source.y), float(source.height),

building=source.building, is_indoor=bool(source.is_indoor))

controlled_sources.append(source)

# 背景干扰源（PPP 生成，多类型多高度）

all_sources = list(controlled_sources)

if self.background_interference_enabled and getattr(self, 'background_interference_sources', None):

all_sources.extend(self.background_interference_sources)

# 可视化子集：受控源 + 背景子集（避免画面拥挤）

viz_sources = list(controlled_sources)

if self.background_interference_enabled and getattr(self, 'background_interference_sources_viz', None):

viz_sources.extend(self.background_interference_sources_viz)

scenario = {

'drone_positions': drone_positions,

'drone_speeds': drone_speeds,

'station_positions': self.station_positions,

'interference_sources': all_sources,

'interference_sources_viz': viz_sources,

'controlled_interference_sources': controlled_sources,

'buildings': self.buildings,

'semantic_layers': getattr(self, 'semantic_layers', {}),

'area_size': self.area_size,

'building_density': self.building_density,

'drone_speed_max': self.drone_speed_max,

'drone_height_max': self.drone_height_max,

'building_height_max': self.building_height_max

}

# 预计算：每架无人机处的总干扰功率（mW），避免在每条 A2G 链路上重复计算

scenario['floor_height_m'] = float(getattr(self, 'floor_height_m', 3.0))

if getattr(self, '_building_observables_cache', None) is not None:

scenario['building_observables'] = self._building_observables_cache

if getattr(self, '_wifi_density_cache', None) is not None:

scenario['wifi_density'] = self._wifi_density_cache

scenario['interference_power_mw_per_drone'] = self._precompute_interference_power_mw_per_drone(

drone_positions=scenario['drone_positions'],

interference_sources=scenario.get('interference_sources', []),

buildings=scenario.get('buildings', [])

)

return scenario

@staticmethod

def _stable_unit_float(*values: float) -> float:

"""

基于输入数值生成稳定的 [0,1) “伪随机”数（不依赖全局 RNG 状态）。

用于让同一染色体在重复评估时得到确定性场景，从而降低 GA 目标噪声。

"""

acc = 0.0

for i, v in enumerate(values):

acc += float(v) * (12.9898 + 17.0 * i)

s = math.sin(acc) * 43758.5453

return float(s - math.floor(s))

def _activity_factor_for_source(self, source_index: int, source: 'InterferenceSource') -> float:

"""

返回“平均占空/活动因子”∈[0,1]，用于把峰值/瞬时 EIRP 折算为平均干扰强度。

- Wi‑Fi：traffic duty + beacon 周期性占空（可通过 user_params 覆盖）

- 5G小基站：活动因子 70%（图片大纲锚点）

- 其他：默认 1

"""

type_id = int(getattr(source, 'type_id', -1))

cfg = self.interference_config.get_config(type_id)

type_key = self.interference_config.type_mapping.get(type_id, None)

cal = (cfg.get('calibration', None) or {})

if type_key in ('wifi_2_4g', 'wifi_5_8g'):

beacon_period_s = float(cal.get('wifi_beacon_period_s', 0.1024))

beacon_dur_ms = float(self.user_params.get(

'wifi_beacon_duration_ms',

cal.get('wifi_beacon_duration_ms_default', 0.5)

))

traffic_dc = float(self.user_params.get(

'wifi_traffic_duty_cycle',

cal.get('wifi_traffic_duty_cycle_default', 0.15)

))

beacon_dc = float(max(0.0, beacon_dur_ms) / 1000.0 / max(beacon_period_s, 1e-9))

dc = float(np.clip(max(0.0, traffic_dc) + beacon_dc, 0.0, 1.0))

return dc

if type_key == 'cellular_5g':

return float(np.clip(float(cal.get('activity_factor', 1.0)), 0.0, 1.0))

return 1.0

def _ue_ul_fpc_power_dbm(self,

source_index: int,

source: 'InterferenceSource',

raw_power_dbm: float,

fpc_cfg: Dict) -> Tuple[float, Dict]:

"""

蜂窝 UE 上行分数功控（FPC）标定：

Pu = min(Pmax, P0 + α·PL + 10log10(M) + ΔTF + f(ΔTPC))

说明：PL 为 UE→服务小区路径损耗（这里用“典型服务距离 + FSPL + 额外损耗”做可标定代理）。

"""

pmax_dbm = float(fpc_cfg.get('pmax_dbm', 23.0))

p0_candidates = list(fpc_cfg.get('p0_candidates_dbm', [-88.0]))

if not p0_candidates:

p0_candidates = [-88.0]

m_candidates = list(fpc_cfg.get('m_rb_candidates', [1]))

if not m_candidates:

m_candidates = [1]

# 用 raw_power_dbm “编码”选择 P0 档位（保持 GA 仍能探索该维度）

u_p0 = float(np.clip(float(raw_power_dbm) / 70.0, 0.0, 1.0 - 1e-12))

p0_dbm = float(p0_candidates[int(u_p0 * len(p0_candidates))])

u_m = self._stable_unit_float(source_index, float(source.x), float(source.y), float(source.height), 9.17)

m_rb = int(m_candidates[int(np.clip(u_m, 0.0, 1.0 - 1e-12) * len(m_candidates))])

m_rb = max(1, int(m_rb))

alpha_switch_h = float(fpc_cfg.get('alpha_switch_height_m', 100.0))

alpha = float(fpc_cfg.get('alpha_ground_or_low_uav', 0.8)) if float(source.height) <= alpha_switch_h else float(

fpc_cfg.get('alpha_high_uav', 0.7)

)

dmin, dmax = fpc_cfg.get('serving_distance_m_range', (30.0, 500.0))

dmin = float(max(dmin, 1.0))

dmax = float(max(dmax, dmin))

u_d = self._stable_unit_float(source_index, float(source.x), float(source.y), 4.2)

# log-uniform in [dmin, dmax]

d_serv = float(dmin * ((dmax / dmin) ** float(np.clip(u_d, 0.0, 1.0))))

pl_db = float(self.calculate_fspl_db_at_freq(d_serv, float(source.frequency)))

pl_extra_db = float(self.user_params.get('ue_ul_pl_extra_db', fpc_cfg.get('pl_extra_db_default', 20.0)))

pl_db = float(pl_db + max(0.0, pl_extra_db))

delta_tf_db = float(fpc_cfg.get('delta_tf_db', 0.0))

f_delta_tpc_db = float(fpc_cfg.get('f_delta_tpc_db', 0.0))

tx_dbm = float(p0_dbm + alpha * pl_db + 10.0 * np.log10(float(m_rb)) + delta_tf_db + f_delta_tpc_db)

tx_dbm = float(min(pmax_dbm, tx_dbm))

tx_dbm = float(max(-50.0, tx_dbm))

meta = {

'fpc': {

'pmax_dbm': pmax_dbm,

'p0_dbm': p0_dbm,

'alpha': alpha,

'm_rb': m_rb,

'pl_serv_db': pl_db,

'pl_serv_distance_m': d_serv,

'pl_extra_db': pl_extra_db,

'delta_tf_db': delta_tf_db,

'f_delta_tpc_db': f_delta_tpc_db

}

}

return tx_dbm, meta

def _calibrate_source_profile_and_power(self,

source_index: int,

source: 'InterferenceSource',

raw_power_dbm: float) -> None:

"""

把“宽泛功率范围”映射为可追溯的标定 profile：

- 5G：femto/pico/micro 锚点 + 小偏移

- Wi‑Fi：按 band 的最小功率门槛裁剪

- UE UL：按 FPC 公式计算 TxPower

其余类型保持原功率（仅做裁剪/记录）。

"""

type_id = int(getattr(source, 'type_id', -1))

type_key = self.interference_config.type_mapping.get(type_id, 'unknown')

cfg = self.interference_config.get_config(type_id)

cal = (cfg.get('calibration', None) or {})

source.raw_power_dbm = float(raw_power_dbm)

source.profile_key = str(type_key)

profile_meta: Dict = {

'type_key': str(type_key),

'sources': dict(cal.get('sources', {}) or {}),

'power_semantics': str(cfg.get('power_semantics', 'eirp_equivalent_dbm')),

}

if type_key == 'cellular_5g':

anchors = dict(cal.get('eirp_anchors_dbm', {}) or {})

if anchors:

p_in = float(source.power)

anchor_name, anchor_dbm = min(anchors.items(), key=lambda kv: abs(p_in - float(kv[1])))

anchor_dbm = float(anchor_dbm)

offset_db = float(np.clip(p_in - anchor_dbm, -3.0, 3.0))

source.power = float(anchor_dbm + offset_db)

source.profile_key = f'5g_smallcell_{anchor_name}'

profile_meta.update({

'eirp_anchor_dbm': anchor_dbm,

'eirp_offset_db': offset_db,

})

elif type_key in ('wifi_2_4g', 'wifi_5_8g'):

floor_dbm = float(cal.get('eirp_min_dbm', float(cfg.get('power_range', (0.0, 0.0))[0])))

source.power = float(max(float(source.power), floor_dbm))

source.profile_key = f'{type_key}_ap'

profile_meta.update({'eirp_floor_dbm': floor_dbm})

elif type_key == 'cellular_ue_ul':

fpc_cfg = dict((cal.get('ul_fpc', None) or {}))

tx_dbm, ue_meta = self._ue_ul_fpc_power_dbm(source_index, source, raw_power_dbm, fpc_cfg)

source.power = float(tx_dbm)

source.profile_key = 'ue_ul_fpc'

profile_meta.update(ue_meta)

source.profile_meta = profile_meta

def _stable_exp_fading_gain(self, drone_idx: int, source_idx: int, source: 'InterferenceSource') -> float:

"""

可复现的 Rayleigh 小尺度衰落功率增益 h（Exp(1)）。

- 关闭时返回 1（即用均值，不引入随机性）

- 开启时仍保持同一场景/染色体重复评估一致，避免 GA 目标噪声

"""

if not getattr(self, 'interference_fading_enabled', False):

return 1.0

u = self._stable_unit_float(

float(drone_idx) + 1.0,

float(source_idx) + 1.0,

float(source.power),

float(source.x),

float(source.y),

float(source.height),

)

u = float(np.clip(u, 1e-12, 1.0 - 1e-12))

return float(-math.log(1.0 - u))

def _stable_exp_fading_gain_for_link(self, key: Tuple[float, ...]) -> float:

"""可复现的 Rayleigh 功率增益 h~Exp(1)，用于“有用信号”链路。"""

if not getattr(self, 'signal_fading_enabled', False):

return 1.0

u = self._stable_unit_float(*key)

u = float(np.clip(u, 1e-12, 1.0 - 1e-12))

return float(-math.log(1.0 - u))

def _select_deterministic_height(self,

source_index: int,

type_id: int,

power_dbm: float,

pos_x: float,

pos_y: float,

suitable_heights: List[float]) -> float:

if not suitable_heights:

return 0.0

if len(suitable_heights) == 1:

return float(suitable_heights[0])

u = self._stable_unit_float(source_index, type_id, power_dbm, pos_x, pos_y)

idx = int(math.floor(u * len(suitable_heights)))

idx = max(0, min(idx, len(suitable_heights) - 1))

return float(suitable_heights[idx])

def _apply_scene_constraints_to_controlled_source(self, source_index: int, source: InterferenceSource) -> InterferenceSource:

"""

对 GA 控制的干扰源做“场景一致性”约束（不改你原先的生成公式，只做条件化重采样/投影）：

- WiFi：强制 indoor

- 蜂窝：优先 roof/tower（或 pole 近似）

- GNSS jammer：优先 street/sidewalk

"""

type_id = int(source.type_id)

allowed = set(self.interference_config.get_allowed_semantics(type_id))

# activity：使用“平均占空/活动因子”，避免 Bernoulli 抖动引入目标噪声

source.activity = float(self._activity_factor_for_source(int(source_index), source))

semantic = self._classify_semantic(float(source.x), float(source.y), float(source.height),

building=source.building, is_indoor=bool(source.is_indoor))

# tower 视作 roof 的同义

allowed_effective = set(allowed)

if 'tower' in allowed_effective:

allowed_effective.add('roof')

if not allowed_effective:

source.semantic = semantic

return source

if semantic in allowed_effective or ('ground' in allowed_effective and semantic == 'ground'):

source.semantic = semantic

return source

mode = str(getattr(self, 'scene_consistency_mode', 'resample')).strip().lower()

# 1) 优先 indoor

if 'indoor' in allowed_effective and self.buildings:

b_idx, b = self._find_or_snap_building(float(source.x), float(source.y))

if b is not None:

x = float(np.clip(source.x, b.x - b.width / 2, b.x + b.width / 2))

y = float(np.clip(source.y, b.y - b.length / 2, b.y + b.length / 2))

floor_h = max(float(self.floor_height_m), 0.5)

floors_total = max(1, int(math.floor(float(b.height) / floor_h)))

u_floor = self._stable_unit_float(source_index, type_id, float(source.power), x, y, 1.414)

floor = 1 + int(math.floor(u_floor * floors_total))

floor = max(1, min(floor, floors_total))

h = float((floor - 1) * floor_h + float(self.indoor_device_height_m))

new_src = InterferenceSource(x, y, h, type_id, float(source.power),

building=b, is_ground=False, building_idx=b_idx,

is_indoor=True, indoor_floor=floor, indoor_floors_total=floors_total,

indoor_distance_m=float(self.p1238_d_in_m),

skip_height_clip=True)

new_src.activity = float(self._activity_factor_for_source(int(source_index), new_src))

new_src.semantic = 'indoor'

return new_src

# 2) roof

if 'roof' in allowed_effective and self.buildings:

b_idx, b = self._find_or_snap_building(float(source.x), float(source.y))

if b is not None:

x = float(np.clip(source.x, b.x - b.width / 2, b.x + b.width / 2))

y = float(np.clip(source.y, b.y - b.length / 2, b.y + b.length / 2))

cfg = self.interference_config.get_config(type_id)

rooftop_offset_m = float(cfg.get('rooftop_offset_m', 2.0))

h = float(b.height + rooftop_offset_m)

new_src = InterferenceSource(x, y, h, type_id, float(source.power),

building=b, is_ground=False, building_idx=b_idx,

is_indoor=False, skip_height_clip=True)

new_src.activity = float(self._activity_factor_for_source(int(source_index), new_src))

new_src.semantic = 'roof'

return new_src

# 3) street/sidewalk：投影/重采样到道路语义

if any(s in allowed_effective for s in ('street', 'sidewalk', 'pole', 'ground')):

target = 'street' if 'street' in allowed_effective else ('sidewalk' if 'sidewalk' in allowed_effective else ('pole' if 'pole' in allowed_effective else 'ground'))

x = float(source.x)

y = float(source.y)

if mode == 'project':

x, y = self._project_xy_to_semantic(x, y, target if target != 'ground' else 'sidewalk')

else:

# 确定性重采样：用 stable_unit_float 产生伪随机 (x,y)

for t in range(int(getattr(self, 'scene_consistency_max_attempts', 12))):

u1 = self._stable_unit_float(source_index, type_id, float(source.power), float(x), float(y), 2.0 + t)

u2 = self._stable_unit_float(source_index, type_id, float(source.power), float(x), float(y), 3.0 + t)

x = float(np.clip(u1 * self.area_size, 0.0, self.area_size))

y = float(np.clip(u2 * self.area_size, 0.0, self.area_size))

if self._building_idx_containing_xy(x, y) is not None:

continue

s = self._classify_semantic(x, y, 0.0, building=None, is_indoor=False)

if (s in allowed_effective) or ('ground' in allowed_effective and s == 'ground'):

break

if target in ('street', 'sidewalk', 'pole'):

x, y = self._project_xy_to_semantic(x, y, target)

new_sem = self._classify_semantic(x, y, 0.0, building=None, is_indoor=False)

new_src = InterferenceSource(x, y, 0.0, type_id, float(source.power),

building=None, is_ground=True, building_idx=None,

is_indoor=False, skip_height_clip=True)

new_src.activity = float(self._activity_factor_for_source(int(source_index), new_src))

new_src.semantic = new_sem

return new_src

source.semantic = semantic

source.activity = float(self._activity_factor_for_source(int(source_index), source))

return source

def _precompute_interference_power_mw_per_drone(self,

drone_positions: np.ndarray,

interference_sources: List[InterferenceSource],

buildings: List[Building]) -> np.ndarray:

if drone_positions is None or len(drone_positions) == 0:

return np.zeros((0,), dtype=float)

if not interference_sources:

return np.zeros((len(drone_positions),), dtype=float)

totals = np.zeros((len(drone_positions),), dtype=float)

source_cache = []

for src_idx, source in enumerate(interference_sources):

p0 = np.array([source.x, source.y, source.height], dtype=float)

freq_coupling = self._spectral_coupling_factor(

tx_freq_hz=float(source.frequency),

rx_freq_hz=float(self.communication_freq),

rx_bw_hz=float(getattr(self, 'bandwidth_hz', 20e6)),

interferer_type_id=int(getattr(source, 'type_id', -1)),

)

source_cache.append((

int(src_idx),

source,

p0,

float(freq_coupling),

float(source.coverage_radius),

float(source.power),

))

for drone_idx, drone_pos in enumerate(drone_positions):

total_mw = 0.0

p1 = np.asarray(drone_pos, dtype=float)

for src_idx, source, p0, freq_coupling, coverage_radius, source_power_dbm in source_cache:

distance = float(np.linalg.norm(p0 - p1))

path_loss_db, _ = self._interference_path_loss_db(

source=source,

uav_pos=p1,

buildings=buildings,

p0=p0,

distance_m=distance,

blocked=None,

)

effective_power_dbm = float(source_power_dbm - float(path_loss_db))

if distance > coverage_radius and coverage_radius > 0.0:

excess_ratio = float(distance / coverage_radius)

effective_power_dbm -= float(20.0 * np.log10(max(excess_ratio, 1e-6)))

fading_gain = self._stable_exp_fading_gain(int(drone_idx), int(src_idx), source)

activity = float(getattr(source, 'activity', 1.0))

total_mw += self._dbm_to_mw(effective_power_dbm) * float(freq_coupling) * float(fading_gain) * activity

totals[drone_idx] = float(total_mw)

return totals

def calculate_los_probability(self, pos1: np.ndarray, pos2: np.ndarray,

buildings: List[Building]) -> float:

"""

计算视距（LoS）概率。

与三张图片对应关系：

- Sigmoid 近似（推荐，图片 Eq.(5) / 中文(2)）：P_LoS = 1 / (1 + C·exp(-B(θ-C)))

- ITU 原始乘积（图片 Eq.(4)）：P_LoS = ∏[1-exp(-h_n^2/(2γ^2))], m=floor(r*sqrt(αβ)-1)

注：两者在论文中是“原始形式 vs 工程化近似”的关系。

"""

r = float(np.linalg.norm(pos1[:2] - pos2[:2]))  # 水平距离（m）

h = float(abs(pos1[2] - pos2[2]))               # 高度差（m）

# 仰角 θ（度）

if r < 1e-6:

theta_deg = 90.0

else:

theta_deg = float(np.degrees(np.arctan(h / r)))

model = str(self.atg_model.get('los_model', 'sigmoid')).strip().lower()

if model in ('sigmoid', 's', 'approx'):

C = float(self.atg_model.get('los_a', 9.61))

B = float(self.atg_model.get('los_b', 0.16))

p_los = 1.0 / (1.0 + C * np.exp(-B * (theta_deg - C)))

return float(np.clip(p_los, 0.01, 0.99))

if model in ('sigmoid_fit', 'fit'):

C = float(self.atg_model.get('los_a', 9.61))

B = float(self.atg_model.get('los_b', 0.16))

p_los = 1.0 / (1.0 + C * np.exp(-B * (theta_deg - C)))

return float(np.clip(p_los, 0.01, 0.99))

# Height-dependent 原始乘积形式（基于 α、β、γ）

# 说明：该模型本质是统计模型，与显式建筑列表无直接关系；保留 buildings 参数以兼容调用。

h_tx = float(max(pos1[2], pos2[2]))

h_rx = float(min(pos1[2], pos2[2]))

alpha = float(np.clip(self.itu_city['alpha'], 0.01, 0.99))

beta_km2 = max(float(self.itu_city['beta_km2']), 1e-6)

beta_m2 = beta_km2 / 1e6

gamma = max(float(self.itu_city['gamma_m']), 1e-6)

# 论文中 r 通常以 km 表示，β 以 buildings/km^2 表示；这里按单位一致性换算：

# N_b = floor(r_km * sqrt(alpha * beta_km2))

r_km = r / 1000.0

nb = int(np.floor(r_km * np.sqrt(alpha * beta_km2)))

nb = max(0, nb)

# P(LoS) = Π_{i=0}^{N_b} P_i,  P_i = 1 - exp(-(h_i)^2/(2γ^2))

prod = 1.0

denom = float(nb + 1)

for n in range(nb + 1):

frac = (n + 0.5) / denom

h_los = h_tx - frac * (h_tx - h_rx)

prod *= (1.0 - np.exp(-(h_los ** 2) / (2.0 * (gamma ** 2))))

return float(np.clip(prod, 0.01, 0.99))

def calculate_fspl_db_at_freq(self, distance_m: float, freq_hz: float) -> float:

"""自由空间路径损耗（FSPL, dB），支持任意频率。"""

d = max(float(distance_m), 1e-6)

f = max(float(freq_hz), 1e-6)

c = float(self.light_speed)

return float(20.0 * np.log10(d) + 20.0 * np.log10(f) + 20.0 * np.log10(4.0 * np.pi / c))

@staticmethod

def _elevation_angle_deg(tx_pos: np.ndarray, rx_pos: np.ndarray) -> float:

tx_pos = np.asarray(tx_pos, dtype=float)

rx_pos = np.asarray(rx_pos, dtype=float)

r = float(np.linalg.norm(tx_pos[:2] - rx_pos[:2]))

h = float(abs(tx_pos[2] - rx_pos[2]))

if r < 1e-9:

return 90.0

return float(np.degrees(np.arctan(h / r)))

def _itu_p1238_floor_loss_db(self, floors: int) -> float:

floors = int(max(1, floors))

# 图中给出的可加项：Lf(n)=15+4(n-1)

return float(15.0 + 4.0 * (floors - 1))

def _itu_p1238_indoor_loss_db(self, freq_hz: float, distance_m: float, floors: int) -> float:

if not self.p1238_enabled:

return 0.0

f_mhz = max(float(freq_hz) / 1e6, 1e-6)

d_m = max(float(distance_m), 1.0)

N = float(self.p1238_N)

lf = self._itu_p1238_floor_loss_db(floors)

return float(20.0 * np.log10(f_mhz) + N * np.log10(d_m) + lf - 28.0)

def _itu_p2109_bel_loss_db(self, freq_hz: float, elevation_deg: float) -> float:

if not self.p2109_bel_enabled:

return 0.0

if None in (self.p2109_bel_r, self.p2109_bel_s, self.p2109_bel_t, self.p2109_bel_u, self.p2109_bel_li_db):

raise ValueError("启用了 p2109_bel_enabled，但未提供 p2109_bel_r/s/t/u/li_db 参数")

f_ghz = max(float(freq_hz) / 1e9, 1e-9)

r = float(self.p2109_bel_r)

s = float(self.p2109_bel_s)

t = float(self.p2109_bel_t)

u = float(self.p2109_bel_u)

li = float(self.p2109_bel_li_db)

lh = r + s * np.log10(f_ghz)

lg = max(float(self.p2109_bel_lg_clip_db), t + u * np.log10(f_ghz))

le = 0.212 * abs(float(elevation_deg))

return float(lh + le + lg + li)

def _interference_path_loss_db(self,

source: InterferenceSource,

uav_pos: np.ndarray,

buildings: List[Building],

p0: Optional[np.ndarray] = None,

distance_m: Optional[float] = None,

blocked: Optional[bool] = None) -> Tuple[float, float]:

"""

干扰源→无人机路径损耗（dB），按“LoS/NLoS 两态 + 概率加权”的 A2G 形式计算，并返回 (PL_dB, P_LoS)。

- 室内源：强制 NLoS，并叠加 ITU P.1238 的室内项（可选）与 P.2109 的 BEL（可选）。

- 室外源：若几何遮挡，则强制 NLoS。

"""

if p0 is None:

p0 = np.array([source.x, source.y, source.height], dtype=float)

p1 = np.asarray(uav_pos, dtype=float)

if distance_m is None:

distance = float(np.linalg.norm(p0 - p1))

else:

distance = float(distance_m)

if blocked is None:

blocked = False

if buildings:

# 仅当 buildings 是 self.buildings 时才可安全使用“网格加速”版本（依赖 self._building_grid 与索引一致性）

if buildings is self.buildings:

blocked = self._is_segment_blocked_by_buildings(p0, p1, ignore_building_idx=source.building_idx)

else:

blocked = self._is_segment_blocked_by_buildings_naive(p0, p1, buildings, ignore_building=source.building)

if source.is_indoor or blocked:

p_los = 0.0

else:

mode = str(getattr(self, 'los_indicator_mode', 'deterministic_if_map')).strip().lower()

if mode == 'deterministic' or (mode in ('deterministic_if_map', 'hybrid') and len(buildings) > 0):

p_los = 1.0

else:

p_los = float(self.calculate_los_probability(p0, p1, buildings))

fspl = self.calculate_fspl_db_at_freq(distance, float(source.frequency))

eta_los = float(self.atg_model['eta_los_db'])

eta_nlos = float(self.atg_model['eta_nlos_db'])

pl_los = fspl + eta_los

pl_nlos = fspl + eta_nlos

pl = float(p_los * pl_los + (1.0 - p_los) * pl_nlos)

# 设备类型相关的额外损耗（将“线性域因子”转到 dB 域加法）

height_factor = float(source._calculate_height_factor(float(uav_pos[2])))

factor = float(source.path_loss_factor) * float(height_factor)

pl += float(10.0 * np.log10(max(factor, 1e-12)))

# 室内额外损耗（可加链）：P.1238 + BEL（可选）

if source.is_indoor:

floors = int(source.indoor_floor) if source.indoor_floor is not None else 1

d_in = float(source.indoor_distance_m) if source.indoor_distance_m is not None else float(self.p1238_d_in_m)

pl += self._itu_p1238_indoor_loss_db(float(source.frequency), d_in, floors)

elev = self._elevation_angle_deg(p0, p1)

pl += self._itu_p2109_bel_loss_db(float(source.frequency), elev)

return float(pl), float(p_los)

def calculate_fspl_db(self, distance_m: float) -> float:

"""自由空间路径损耗（FSPL, dB）"""

d = max(float(distance_m), 1e-6)

f = float(self.communication_freq)

c = float(self.light_speed)

return float(20.0 * np.log10(d) + 20.0 * np.log10(f) + 20.0 * np.log10(4.0 * np.pi / c))

def calculate_atg_mean_path_loss_db(self, uav_pos: np.ndarray, gt_pos: np.ndarray, buildings: List[Building]) -> float:

"""平均路径损耗 Λ（dB），LoS/NLoS 空间期望"""

r = float(np.linalg.norm(uav_pos[:2] - gt_pos[:2]))

h = float(abs(uav_pos[2] - gt_pos[2]))

d = float(np.sqrt(h ** 2 + r ** 2))

fspl = self.calculate_fspl_db(d)

eta_los = float(self.atg_model['eta_los_db'])

eta_nlos = float(self.atg_model['eta_nlos_db'])

pl_los = fspl + eta_los

pl_nlos = fspl + eta_nlos

mode = str(getattr(self, 'los_indicator_mode', 'deterministic_if_map')).strip().lower()

if mode == 'deterministic' or (mode in ('deterministic_if_map', 'hybrid') and len(buildings) > 0):

p_los = 1.0 if self.is_los(uav_pos, gt_pos, buildings=buildings) else 0.0

else:

p_los = self.calculate_los_probability(uav_pos, gt_pos, buildings)

return float(p_los * pl_los + (1.0 - p_los) * pl_nlos)

@staticmethod

def _distance_3d_and_elevation_deg(tx_pos: np.ndarray, rx_pos: np.ndarray) -> Tuple[float, float]:

"""统一几何：三维距离 d 与仰角 θ（度）。"""

tx_pos = np.asarray(tx_pos, dtype=float)

rx_pos = np.asarray(rx_pos, dtype=float)

d = float(np.linalg.norm(tx_pos - rx_pos))

r = float(np.linalg.norm(tx_pos[:2] - rx_pos[:2]))

h = float(abs(tx_pos[2] - rx_pos[2]))

if r < 1e-9:

theta = 90.0

else:

theta = float(np.degrees(np.arctan(h / r)))

return max(d, 1e-6), theta

def calculate_mean_path_loss_db(self,

tx_pos: np.ndarray,

rx_pos: np.ndarray,

freq_hz: float,

buildings: Optional[List[Building]] = None,

force_nlos: bool = False,

ignore_building_idx: Optional[int] = None,

tx_rx_eta_los_db: Optional[float] = None,

tx_rx_eta_nlos_db: Optional[float] = None) -> Tuple[float, float]:

"""

统一链路预算：PL(dB)=P(LoS)*PL_LoS+(1-P(LoS))*PL_NLoS，并返回 (PL_dB, P_LoS)。

- A2G/U2U 同一套公式，差别仅来自几何高度（tx_pos/rx_pos）。

- 若 force_nlos=True 或被真实建筑遮挡，则 P_LoS=0。

"""

tx_pos = np.asarray(tx_pos, dtype=float)

rx_pos = np.asarray(rx_pos, dtype=float)

d, _ = self._distance_3d_and_elevation_deg(tx_pos, rx_pos)

buildings_list = buildings if buildings is not None else []

blocked = False

if buildings_list:

if buildings_list is self.buildings:

blocked = self._is_segment_blocked_by_buildings(tx_pos, rx_pos, ignore_building_idx=ignore_building_idx)

else:

blocked = self._is_segment_blocked_by_buildings_naive(tx_pos, rx_pos, buildings_list, ignore_building=None)

if force_nlos or blocked:

p_los = 0.0

else:

mode = str(getattr(self, 'los_indicator_mode', 'deterministic_if_map')).strip().lower()

if mode == 'deterministic' or (mode in ('deterministic_if_map', 'hybrid') and len(buildings_list) > 0):

p_los = 1.0

else:

p_los = float(self.calculate_los_probability(tx_pos, rx_pos, buildings_list))

fspl = self.calculate_fspl_db_at_freq(d, float(freq_hz))

eta_los = float(self.atg_model['eta_los_db'] if tx_rx_eta_los_db is None else tx_rx_eta_los_db)

eta_nlos = float(self.atg_model['eta_nlos_db'] if tx_rx_eta_nlos_db is None else tx_rx_eta_nlos_db)

pl_los = fspl + eta_los

pl_nlos = fspl + eta_nlos

pl = float(p_los * pl_los + (1.0 - p_los) * pl_nlos)

return pl, float(p_los)

def calculate_rx_power_dbm(self,

tx_power_dbm: float,

tx_pos: np.ndarray,

rx_pos: np.ndarray,

freq_hz: float,

buildings: Optional[List[Building]] = None,

force_nlos: bool = False,

ignore_building_idx: Optional[int] = None,

extra_loss_db: float = 0.0,

fading_gain_linear: float = 1.0,

duty_cycle_activity: float = 1.0) -> Tuple[float, float]:

"""

统一接收功率模板（功率域第一公民）：

P_rx[dBm] = P_tx[dBm] - PL_mean[dB] - extra_loss[dB] + 10log10(fading) + 10log10(activity)

"""

pl_db, p_los = self.calculate_mean_path_loss_db(

tx_pos=tx_pos,

rx_pos=rx_pos,

freq_hz=float(freq_hz),

buildings=buildings,

force_nlos=force_nlos,

ignore_building_idx=ignore_building_idx,

)

fade = max(float(fading_gain_linear), 1e-12)

act = max(float(duty_cycle_activity), 0.0)

act_db = 10.0 * np.log10(act) if act > 0.0 else -300.0

rx_dbm = float(tx_power_dbm - pl_db - float(extra_loss_db) + 10.0 * np.log10(fade) + act_db)

return rx_dbm, float(p_los)

def calculate_interference_power_mw(self,

drone_pos: np.ndarray,

interference_sources: List[InterferenceSource],

buildings: Optional[List[Building]] = None,

drone_idx: Optional[int] = None) -> float:

if not interference_sources:

return 0.0

total_mw = 0.0

p1 = np.asarray(drone_pos, dtype=float)

buildings_list = buildings if buildings is not None else []

for src_idx, source in enumerate(interference_sources):

p0 = np.array([source.x, source.y, source.height], dtype=float)

distance = float(np.linalg.norm(p0 - p1))

path_loss_db, _ = self._interference_path_loss_db(

source=source,

uav_pos=p1,

buildings=buildings_list,

p0=p0,

distance_m=distance,

blocked=None,

)

effective_power_dbm = float(source.power - float(path_loss_db))

if distance > float(source.coverage_radius) and float(source.coverage_radius) > 0.0:

excess_ratio = float(distance / float(source.coverage_radius))

effective_power_dbm -= float(20.0 * np.log10(max(excess_ratio, 1e-6)))

freq_coupling = self._spectral_coupling_factor(

tx_freq_hz=float(source.frequency),

rx_freq_hz=float(self.communication_freq),

rx_bw_hz=float(getattr(self, 'bandwidth_hz', 20e6)),

interferer_type_id=int(getattr(source, 'type_id', -1)),

)

fading_gain = self._stable_exp_fading_gain(int(drone_idx) if drone_idx is not None else 0, int(src_idx), source)

activity = float(getattr(source, 'activity', 1.0))

total_mw += self._dbm_to_mw(effective_power_dbm) * float(freq_coupling) * float(fading_gain) * activity

return float(total_mw)

def calculate_power_intensity_change(self,

drone_pos: np.ndarray,

station_pos: np.ndarray,

scenario: Dict,

drone_idx: Optional[int] = None) -> float:

distance = np.linalg.norm(drone_pos - station_pos)

if distance < 1.0:

distance = 1.0

path_loss_db = self.calculate_atg_mean_path_loss_db(drone_pos, station_pos, scenario['buildings'])

rx_signal_dbm = self._comm_tx_power_dbm - path_loss_db

rx_signal_mw = self._dbm_to_mw(rx_signal_dbm)

baseline_rx_mw = float(self._baseline_rx_mw)

interference_mw: Optional[float] = None

if drone_idx is not None and 'interference_power_mw_per_drone' in scenario:

try:

interference_mw = float(scenario['interference_power_mw_per_drone'][int(drone_idx)])

except Exception:

interference_mw = None

if interference_mw is None:

interference_mw = self.calculate_interference_power_mw(

drone_pos,

scenario.get('interference_sources', []),

buildings=scenario.get('buildings', []),

drone_idx=drone_idx

)

eps = 1e-12

power_increase_ratio = (baseline_rx_mw / (rx_signal_mw + eps)) * (1.0 + interference_mw / (rx_signal_mw + eps))

power_increase_ratio = max(power_increase_ratio, 1.0)

ratio_db = 10.0 * np.log10(power_increase_ratio)

scale_db = float(self._power_change_scale_db)

if ratio_db <= 0.0:

return 0.0

return float(np.clip(1.0 - np.exp(-ratio_db / max(scale_db, 1e-6)), 0.0, 1.0))

def _noise_power_mw(self) -> float:

"""噪声功率 Ny（mW）：N0[dBm/Hz] + 10log10(B) + NF。"""

b = max(float(getattr(self, 'bandwidth_hz', 1.0)), 1.0)

n0_dbm_hz = float(getattr(self, 'thermal_noise_dbm_hz', -174.0))

nf_db = float(getattr(self, 'noise_figure_db', 0.0))

noise_dbm = float(n0_dbm_hz + 10.0 * np.log10(b) + nf_db)

return float(self._dbm_to_mw(noise_dbm))

def calculate_power_margin_components(self,

drone_pos: np.ndarray,

station_pos: np.ndarray,

scenario: Dict,

drone_idx: Optional[int] = None) -> Dict[str, float]:

"""

统一功率域分解（不依赖“比值口径”的 SINR）：

- P_rx[dBm]：接收信号功率

- I_y[mW]：总干扰功率（线性累加）

- N_y[mW]：噪声功率（线性）

- P_IN[dBm]：干扰+噪声的等效功率

- M[dB] = P_rx[dBm] - P_IN[dBm]：功率裕量（等价于 SINR 的 dB 形式）

- SINR = 10^(M/10)（可选输出）

- R = B log2(1 + 10^(M/10))（可选输出）

"""

drone_pos = np.asarray(drone_pos, dtype=float)

station_pos = np.asarray(station_pos, dtype=float)

# 约定：A2G 链路为 “station -> drone”（地面站发射、无人机接收），干扰聚合也在无人机端计算

buildings_list = scenario.get('buildings', [])

fade = self._stable_exp_fading_gain_for_link((

9.0,

float(drone_idx if drone_idx is not None else 0),

float(drone_pos[0]), float(drone_pos[1]), float(drone_pos[2]),

float(station_pos[0]), float(station_pos[1]), float(station_pos[2]),

))

prx_dbm, p_los = self.calculate_rx_power_dbm(

tx_power_dbm=float(self._comm_tx_power_dbm),

tx_pos=station_pos,

rx_pos=drone_pos,

freq_hz=float(self.communication_freq),

buildings=buildings_list,

force_nlos=False,

ignore_building_idx=None,

extra_loss_db=0.0,

fading_gain_linear=float(fade),

duty_cycle_activity=1.0,

)

prx_mw = float(self._dbm_to_mw(prx_dbm))

# PL 便于阈值事件：P(PL > PL_max)

pl_db, _ = self.calculate_mean_path_loss_db(

tx_pos=station_pos, rx_pos=drone_pos, freq_hz=float(self.communication_freq), buildings=buildings_list

)

# 干扰：I_y（mW）

iy_mw: Optional[float] = None

if drone_idx is not None and 'interference_power_mw_per_drone' in scenario:

try:

iy_mw = float(scenario['interference_power_mw_per_drone'][int(drone_idx)])

except Exception:

iy_mw = None

if iy_mw is None:

iy_mw = float(self.calculate_interference_power_mw(

drone_pos=drone_pos,

interference_sources=scenario.get('interference_sources', []),

buildings=buildings_list,

drone_idx=drone_idx,

))

# 噪声：N_y（mW）

ny_mw = float(self._noise_power_mw())

pin_mw = float(max(iy_mw, 0.0) + max(ny_mw, 0.0))

pin_dbm = float(self._mw_to_dbm(pin_mw))

n_dbm = float(self._mw_to_dbm(ny_mw))

delta_in_db = float(pin_dbm - n_dbm)

iy_dbm = float(self._mw_to_dbm(max(float(iy_mw), 0.0)))

m_db = float(prx_dbm - pin_dbm)

sinr_lin = float(10.0 ** (m_db / 10.0))

b = max(float(getattr(self, 'bandwidth_hz', 1.0)), 1.0)

rate_bps = float(b * np.log2(1.0 + sinr_lin))

outage = 1.0 if m_db < float(getattr(self, 'margin_threshold_db', 0.0)) else 0.0

return {

'P_rx_dbm': prx_dbm,

'P_rx_mw': prx_mw,

'I_y_mw': float(iy_mw),

'I_y_dbm': iy_dbm,

'N_y_mw': ny_mw,

'N_y_dbm': n_dbm,

'P_IN_dbm': pin_dbm,

'Delta_IN_db': delta_in_db,

'M_db': m_db,

'SINR_linear': sinr_lin,

'R_bps': rate_bps,

'Outage': outage,

'PL_db': float(pl_db),

'P_LoS': float(p_los),

}

def _cdi_intensity_mapping_db(self, interference_mw: float) -> float:

"""g(I)：把“聚合干扰强度 I（mW）”映射到 dB-like 尺度，避免直接用比值/SINR。"""

i_mw = max(float(interference_mw), 0.0)

i_ref = max(float(getattr(self, 'cdi_int_ref_mw', 1e-12)), 1e-12)

return float(10.0 * np.log10(1.0 + i_mw / i_ref))

def calculate_cdi_components(self,

drone_pos: np.ndarray,

station_pos: np.ndarray,

scenario: Dict,

drone_idx: Optional[int] = None) -> Tuple[float, float, float]:

"""

统一通信劣化指数（CDI）分量：

- D_link = max(0, Λ - PL_max)  （只用路径损耗）

- D_int  = I = Σ h_i l(r_i)   （聚合干扰强度，线性累加）

- CDI    = w_L·D_link + w_I·g(D_int)

"""

lambda_db = float(self.calculate_atg_mean_path_loss_db(drone_pos, station_pos, scenario.get('buildings', [])))

d_link = float(max(0.0, lambda_db - float(self.cdi_pl_max_db)))

interference_mw: Optional[float] = None

if drone_idx is not None and 'interference_power_mw_per_drone' in scenario:

try:

interference_mw = float(scenario['interference_power_mw_per_drone'][int(drone_idx)])

except Exception:

interference_mw = None

if interference_mw is None:

interference_mw = float(self.calculate_interference_power_mw(

drone_pos=np.asarray(drone_pos, dtype=float),

interference_sources=scenario.get('interference_sources', []),

buildings=scenario.get('buildings', []),

drone_idx=drone_idx,

))

g_int_db = self._cdi_intensity_mapping_db(interference_mw)

cdi = float(self.cdi_w_link * d_link + self.cdi_w_int * g_int_db)

return d_link, float(interference_mw), cdi

def calculate_comm_degradation(self,

drone_pos: np.ndarray,

station_pos: np.ndarray,

scenario: Dict,

drone_idx: Optional[int] = None) -> float:

"""

通信劣化程度 ∈[0,1]：

- legacy：原始启发式（基准功率 + 干扰/信号）

- cdi：按公式 CDI 计算，并用 1-exp(-CDI/scale) 映射到 [0,1]

"""

model = str(getattr(self, 'degradation_model', 'margin')).strip().lower()

if model in ('legacy', 'heuristic', 'old', 'power_ratio'):

base = float(self.calculate_power_intensity_change(drone_pos, station_pos, scenario, drone_idx=drone_idx))

return float(self._blend_u2u_comm_degradation(base, scenario, drone_idx=drone_idx))

if model in ('margin', 'power_margin', 'm'):

comp = self.calculate_power_margin_components(drone_pos, station_pos, scenario, drone_idx=drone_idx)

m_db = float(comp['M_db'])

theta = float(getattr(self, 'margin_threshold_db', 0.0))

slope = max(float(getattr(self, 'margin_slope_db', 1.0)), 1e-6)

# comm_deg 越大表示越差：当 M 低于门限时快速趋近 1

comm_deg = 1.0 / (1.0 + np.exp((m_db - theta) / slope))

return float(self._blend_u2u_comm_degradation(float(np.clip(comm_deg, 0.0, 1.0)), scenario, drone_idx=drone_idx))

_, _, cdi = self.calculate_cdi_components(drone_pos, station_pos, scenario, drone_idx=drone_idx)

scale = max(float(getattr(self, 'cdi_scale_db', 1.0)), 1e-6)

base = float(np.clip(1.0 - np.exp(-float(cdi) / scale), 0.0, 1.0))

return float(self._blend_u2u_comm_degradation(base, scenario, drone_idx=drone_idx))

def _comm_deg_from_margin_db(self, m_db: float) -> float:

"""将功率裕量 M[dB] 映射为通信劣化程度∈[0,1]（越大越差）。"""

theta = float(getattr(self, 'margin_threshold_db', 0.0))

slope = max(float(getattr(self, 'margin_slope_db', 1.0)), 1e-6)

comm_deg = 1.0 / (1.0 + np.exp((float(m_db) - theta) / slope))

return float(np.clip(comm_deg, 0.0, 1.0))

def _u2u_comm_degradation_for_drone(self, scenario: Dict, drone_idx: int) -> Optional[float]:

"""

UAV↔UAV 通信链路的“有用信道”劣化（不作为干扰 I_aer）。

计算方式：复用 U2U 的功率预算（calculate_u2u_power_margin_components），

取与该 UAV 相关的所有 U2U 方向链路的劣化并聚合（mean/max 可选）。

"""

if not getattr(self, 'u2u_enabled', False):

return None

drones = np.asarray(scenario.get('drone_positions', np.zeros((0, 3))), dtype=float)

if int(drone_idx) < 0 or int(drone_idx) >= len(drones) or len(drones) < 2:

return None

agg = str(self.user_params.get('u2u_comm_aggregate', 'mean')).strip().lower()

vals: List[float] = []

it = self._iter_u2u_links(scenario)

if it is None:

return None

for i, j, pi, pj in it:

# 对于每个无向 pair，计算两个方向（i->j 与 j->i），并把涉及 drone_idx 的方向计入

if int(drone_idx) == int(i):

pm_ij = self.calculate_u2u_power_margin_components(pi, pj, scenario, tx_idx=int(i), rx_idx=int(j))

vals.append(self._comm_deg_from_margin_db(float(pm_ij.get('M_db', 0.0))))

pm_ji = self.calculate_u2u_power_margin_components(pj, pi, scenario, tx_idx=int(j), rx_idx=int(i))

vals.append(self._comm_deg_from_margin_db(float(pm_ji.get('M_db', 0.0))))

elif int(drone_idx) == int(j):

pm_ij = self.calculate_u2u_power_margin_components(pi, pj, scenario, tx_idx=int(i), rx_idx=int(j))

vals.append(self._comm_deg_from_margin_db(float(pm_ij.get('M_db', 0.0))))

pm_ji = self.calculate_u2u_power_margin_components(pj, pi, scenario, tx_idx=int(j), rx_idx=int(i))

vals.append(self._comm_deg_from_margin_db(float(pm_ji.get('M_db', 0.0))))

if not vals:

return None

if agg in ('max', 'worst', 'worst_case'):

return float(max(vals))

return float(np.mean(vals))

def _blend_u2u_comm_degradation(self, a2g_comm_deg: float, scenario: Dict, drone_idx: Optional[int]) -> float:

"""

将 U2U 链路损耗“纳入通信劣化评估”。

说明：UAV↔UAV 是“有用信道”，因此不进入干扰项；其链路质量通过此处融合到 comm_deg。

"""

base = float(np.clip(a2g_comm_deg, 0.0, 1.0))

if drone_idx is None or not getattr(self, 'u2u_enabled', False):

return base

u2u_deg = self._u2u_comm_degradation_for_drone(scenario, int(drone_idx))

if u2u_deg is None:

return base

w = self.user_params.get('u2u_comm_weight', None)

if w is None:

# 默认：启用 U2U 时，A2G 与 U2U 各占一半

w_u2u = 0.5

else:

w_u2u = float(w)

w_u2u = float(np.clip(w_u2u, 0.0, 1.0))

return float(np.clip((1.0 - w_u2u) * base + w_u2u * float(u2u_deg), 0.0, 1.0))

def calculate_speed_energy_efficiency_degradation(self, drone_speed_mps: float) -> float:

"""

速度导致的通信能效劣化（基于旋翼无人机推进功率模型 P(V)）。

使用图中公式（blade profile + induced + parasite），将“能效”近似为单位时间可用能量的倒数：

- 以悬停功率 P(0)=P0+Pi 为参考，速度能效劣化 = max(0, 1 - P(0)/P(V))，范围[0,1]。

"""

v = max(float(drone_speed_mps), 0.0)

P0 = float(self.power_model['rotor_P0_w'])

Pi = float(self.power_model['rotor_Pi_w'])

U_tip = max(float(self.power_model['rotor_U_tip_mps']), 1e-6)

v0 = max(float(self.power_model['rotor_v0_mps']), 1e-6)

d0 = float(self.power_model['rotor_d0'])

rho = float(self.power_model['air_rho'])

s = float(self.power_model['rotor_s'])

A = float(self.power_model['rotor_A_m2'])

# P(V)=P0(1+3V^2/U_tip^2) + Pi*sqrt( sqrt(1+V^4/(4v0^4)) - V^2/(2v0^2) ) + 1/2*d0*rho*s*A*V^3

blade_profile = P0 * (1.0 + 3.0 * (v ** 2) / (U_tip ** 2))

induced_inner = np.sqrt(1.0 + (v ** 4) / (4.0 * (v0 ** 4))) - (v ** 2) / (2.0 * (v0 ** 2))

induced = Pi * np.sqrt(max(float(induced_inner), 0.0))

parasite = 0.5 * d0 * rho * s * A * (v ** 3)

propulsion_power_w = float(blade_profile + induced + parasite)

hover_power_w = float(P0 + Pi)

propulsion_power_w = max(propulsion_power_w, 1e-6)

hover_power_w = max(hover_power_w, 1e-6)

degradation = 1.0 - (hover_power_w / propulsion_power_w)

return float(np.clip(max(0.0, degradation), 0.0, 1.0))

def _rotor_propulsion_power_w(self, drone_speed_mps: float) -> float:

"""旋翼无人机推进功率 P(V)（W），用于 bit/J 能效计算。"""

v = max(float(drone_speed_mps), 0.0)

P0 = float(self.power_model['rotor_P0_w'])

Pi = float(self.power_model['rotor_Pi_w'])

U_tip = max(float(self.power_model['rotor_U_tip_mps']), 1e-6)

v0 = max(float(self.power_model['rotor_v0_mps']), 1e-6)

d0 = float(self.power_model['rotor_d0'])

rho = float(self.power_model['air_rho'])

s = float(self.power_model['rotor_s'])

A = float(self.power_model['rotor_A_m2'])

blade_profile = P0 * (1.0 + 3.0 * (v ** 2) / (U_tip ** 2))

induced_inner = np.sqrt(1.0 + (v ** 4) / (4.0 * (v0 ** 4))) - (v ** 2) / (2.0 * (v0 ** 2))

induced = Pi * np.sqrt(max(float(induced_inner), 0.0))

parasite = 0.5 * d0 * rho * s * A * (v ** 3)

return float(max(blade_profile + induced + parasite, 1e-6))

def calculate_energy_efficiency_metrics(self, scenario: Dict) -> Dict[str, float]:

"""

能效 EE（bit/J）：EE = (Σ_k R_k) / (Σ_k P_prop(V_k))

这里用“无人机 k 对所有地面站的平均速率 R_k”来近似其吞吐。

"""

if bool(getattr(self, 'tdma_enabled', False)):

return self.calculate_tdma_energy_efficiency_metrics(scenario)

drones = np.asarray(scenario.get('drone_positions', np.zeros((0, 3))), dtype=float)

stations = np.asarray(scenario.get('station_positions', np.zeros((0, 3))), dtype=float)

speeds = np.asarray(scenario.get('drone_speeds', np.zeros((len(drones),))), dtype=float)

if drones.size == 0 or stations.size == 0:

return {'sum_rate_bps': 0.0, 'sum_propulsion_w': 0.0, 'EE_bpj': 0.0}

sum_rate_bps = 0.0

sum_prop_w = 0.0

for drone_idx in range(int(len(drones))):

rate_list = []

for station_idx in range(int(len(stations))):

comp = self.calculate_power_margin_components(

drones[drone_idx],

stations[station_idx],

scenario,

drone_idx=int(drone_idx),

)

rate_list.append(float(comp.get('R_bps', 0.0)))

mean_rate_bps = float(np.mean(rate_list)) if rate_list else 0.0

v = float(speeds[drone_idx]) if drone_idx < len(speeds) else 0.0

sum_rate_bps += mean_rate_bps

sum_prop_w += self._rotor_propulsion_power_w(v)

ee_bpj = float(sum_rate_bps / max(sum_prop_w, 1e-12))

return {'sum_rate_bps': float(sum_rate_bps), 'sum_propulsion_w': float(sum_prop_w), 'EE_bpj': ee_bpj}

def _build_time_grid(self) -> Tuple[np.ndarray, float]:

"""

离散时间网格：t[n]=n·Δt，n=0..N-1。

用于把连续时间积分近似为求和（TDMA 吞吐/能量）。

"""

t_total = float(self.user_params.get('traj_total_time_s', 20.0))

dt = float(self.user_params.get('traj_dt_s', 1.0))

t_total = max(t_total, 1e-6)

dt = max(dt, 1e-6)

n = int(max(1, round(t_total / dt)))

t = np.arange(n, dtype=float) * dt

return t, float(dt)

def _trajectory_positions_discrete(self,

drone_idx: int,

scenario: Dict,

t: np.ndarray) -> Tuple[np.ndarray, np.ndarray]:

"""

给出离散时间轨迹 q[n] 与速度标量 V[n]。

- hover: q 固定，V=0

- orbit: q 围绕初始点做圆周，速度由染色体 speed 给出（V=rw）

- static: 不改变位置（保持现有“静态场景”口径），V 用染色体 speed（仅影响能耗）

"""

drones0 = np.asarray(scenario.get('drone_positions', np.zeros((0, 3))), dtype=float)

speeds0 = np.asarray(scenario.get('drone_speeds', np.zeros((len(drones0),))), dtype=float)

if drones0.size == 0 or int(drone_idx) < 0 or int(drone_idx) >= len(drones0):

return np.zeros((len(t), 3), dtype=float), np.zeros((len(t),), dtype=float)

p0 = np.asarray(drones0[int(drone_idx)], dtype=float)

v0 = float(speeds0[int(drone_idx)]) if int(drone_idx) < len(speeds0) else 0.0

mode = str(self.user_params.get('traj_mode', 'static')).strip().lower()

# 便捷映射：速度为 0 -> hover

if v0 <= 1e-6 and mode in ('orbit', 'orbit_from_speed', 'auto', 'mode_by_speed'):

mode = 'hover'

pos = np.zeros((len(t), 3), dtype=float)

spd = np.zeros((len(t),), dtype=float)

if mode in ('hover',):

pos[:] = p0

spd[:] = 0.0

return pos, spd

if mode in ('orbit', 'orbit_from_speed'):

r = float(self.user_params.get('orbit_radius_m', 30.0))

r = max(r, 1e-3)

w = float(v0 / r)  # rad/s

theta0 = float(self._stable_unit_float(31.0, float(p0[0]), float(p0[1]), float(p0[2])) * 2.0 * np.pi)

pos[:, 0] = float(p0[0]) + r * np.cos(w * t + theta0)

pos[:, 1] = float(p0[1]) + r * np.sin(w * t + theta0)

pos[:, 2] = float(p0[2])

spd[:] = float(abs(v0))

return pos, spd

# static（默认）：q 不随时间变化，speed 仅用于 P(V)

pos[:] = p0

spd[:] = float(abs(v0))

return pos, spd

def calculate_tdma_energy_efficiency_metrics(self, scenario: Dict) -> Dict[str, float]:

"""

离散 TDMA：对每个 UAV，在每个时间步只服务一个 GN（这里用 ground stations 作为 GN）。

- 速率：R_k[n] = B log2(1 + SINR/Γ)

- TDMA：λ_k[n]∈{0,1}，每步选择一个 GN：argmax_k R_k[n]

- 吞吐：R~_k = Σ_n λ_k[n] R_k[n] Δt

- 能量：E = Σ_n P(V[n])Δt + Pc Σ_n (Σ_k λ_k[n])Δt

- 能效：EE = (Σ_k R~_k) / E

"""

drones = np.asarray(scenario.get('drone_positions', np.zeros((0, 3))), dtype=float)

stations = np.asarray(scenario.get('station_positions', np.zeros((0, 3))), dtype=float)

if drones.size == 0 or stations.size == 0:

return {'sum_rate_bps': 0.0, 'sum_propulsion_w': 0.0, 'EE_bpj': 0.0, 'tdma_enabled': 1.0}

t, dt = self._build_time_grid()

b = max(float(getattr(self, 'bandwidth_hz', 1.0)), 1.0)

gamma = float(self.user_params.get('coding_gap_linear', 1.0))

gamma = max(gamma, 1.0)

pc = float(self.user_params.get('comm_circuit_power_w', 0.0))

pc = max(pc, 0.0)

total_bits = 0.0

total_energy_j = 0.0

station_bits = np.zeros((len(stations),), dtype=float)

# 轨迹评估不复用静态“每无人机干扰缓存”，避免位置变化导致口径错误

scenario_no_cache = dict(scenario)

scenario_no_cache.pop('interference_power_mw_per_drone', None)

for di in range(int(len(drones))):

traj, spd = self._trajectory_positions_discrete(int(di), scenario, t)

# 推进能量

e_prop = 0.0

for vn in spd:

e_prop += float(self._rotor_propulsion_power_w(float(vn))) * float(dt)

total_energy_j += float(e_prop)

# TDMA 通信：每步选一个 GN

comm_time = 0.0

for n in range(len(t)):

rates = []

for si in range(int(len(stations))):

drone_pos_n = traj[n]

station_pos = stations[si]

# 不使用预计算缓存（轨迹会变），每步按当前位置重算 I/N

pm = self.calculate_power_margin_components(

drone_pos=np.asarray(drone_pos_n, dtype=float),

station_pos=np.asarray(station_pos, dtype=float),

scenario=scenario_no_cache,

drone_idx=int(di),

)

sinr = float(pm.get('SINR_linear', 0.0))

r_bps = float(b * np.log2(1.0 + max(sinr, 0.0) / gamma))

rates.append(r_bps)

if not rates:

continue

k = int(np.argmax(np.asarray(rates, dtype=float)))

bits = float(max(rates[k], 0.0)) * float(dt)

station_bits[k] += bits

total_bits += bits

comm_time += float(dt)

total_energy_j += float(pc * comm_time)

ee_bpj = float(total_bits / max(total_energy_j, 1e-12))

sum_rate_bps = float(total_bits / max(float(len(t)) * dt, 1e-12))

sum_propulsion_w = float(total_energy_j / max(float(len(t)) * dt, 1e-12))

out = {

'sum_rate_bps': float(sum_rate_bps),

'sum_propulsion_w': float(sum_propulsion_w),

'EE_bpj': float(ee_bpj),

'tdma_enabled': 1.0,

'tdma_total_bits': float(total_bits),

'tdma_total_energy_j': float(total_energy_j),

'tdma_station_bits': station_bits.tolist(),

'traj_dt_s': float(dt),

'traj_steps': int(len(t)),

}

return out

def calculate_outline_total_score(self, scenario: Dict) -> Dict[str, float]:

"""

大纲式总评分（E3）：把“通信劣化 + 吞吐违约 + 能效劣化 + 约束罚”融合成 D_total（越大越差）。

约定：

- D_comm：沿用现有 power_score 的 D（或你选择的其他通信劣化口径）

- D_thr：TDMA 吞吐约束违约（可选）

- D_EE：能效劣化（可选）

"""

pm = self.calculate_power_score_metrics(scenario)

d_comm = float(pm.get('D', 0.0))

ee = self.calculate_energy_efficiency_metrics(scenario)

ee_bpj = float(ee.get('EE_bpj', 0.0))

d_thr = 0.0

if bool(self.tdma_enabled):

# 目标吞吐支持两种口径：bits 或 bps

q_bits = self.user_params.get('tdma_Qk_bits', None)

q_bps = self.user_params.get('tdma_Qk_bps', None)

station_bits = ee.get('tdma_station_bits', None)

dt = float(ee.get('traj_dt_s', self.user_params.get('traj_dt_s', 1.0)))

steps = int(ee.get('traj_steps', max(1, int(round(float(self.user_params.get('traj_total_time_s', 20.0)) / max(dt, 1e-6))))))

t_total = float(max(steps, 1) * dt)

if isinstance(station_bits, list) and station_bits:

station_bits_arr = np.asarray(station_bits, dtype=float)

if q_bits is None and q_bps is not None:

if isinstance(q_bps, (int, float)):

q_bits_arr = np.ones_like(station_bits_arr) * float(q_bps) * t_total

else:

q_bits_arr = np.asarray(q_bps, dtype=float) * t_total

elif q_bits is not None:

if isinstance(q_bits, (int, float)):

q_bits_arr = np.ones_like(station_bits_arr) * float(q_bits)

else:

q_bits_arr = np.asarray(q_bits, dtype=float)

else:

q_bits_arr = None

if q_bits_arr is not None and q_bits_arr.size == station_bits_arr.size:

denom = np.maximum(q_bits_arr, 1e-12)

d_thr = float(np.sum(np.maximum(0.0, q_bits_arr - station_bits_arr) / denom))

ee_target = self.user_params.get('ee_target_bpj', None)

if ee_target is not None:

ee_target = max(float(ee_target), 1e-12)

d_ee = float(np.clip(max(0.0, (ee_target - ee_bpj) / ee_target), 0.0, 1.0))

else:

ee_ref = max(float(self.user_params.get('ee_ref_bpj', 1e5)), 1e-12)

d_ee = float(1.0 / (1.0 + ee_bpj / ee_ref))

d_pen = 0.0

w = dict(getattr(self, 'outline_weights', {}))

d_total = float(

float(w.get('w_comm', 1.0)) * d_comm +

float(w.get('w_thr', 0.0)) * d_thr +

float(w.get('w_ee', 0.0)) * d_ee +

float(w.get('w_penalty', 0.0)) * d_pen

)

return {

'D_total': float(d_total),

'D_comm': float(d_comm),

'D_thr': float(d_thr),

'D_EE': float(d_ee),

'D_penalty': float(d_pen),

'EE_bpj': float(ee_bpj),

}

def _get_aerial_interferers(self, scenario: Dict) -> List['InterferenceSource']:

"""

“空中干扰项” I_aer 的来源入口（Zeng 2019 的 I_aer(Q) 结构）。

重要澄清：

- UAV↔UAV 通信在本项目中按“有用信道/有用链路”理解（见 u2u_enabled），不应作为 I_aer 干扰项。

- I_aer 仅在你显式提供“空中干扰机/同频空中干扰源”集合时启用。

约定：用户可在 scenario 里提供

- aerial_interferers: List[InterferenceSource]

兼容旧键：aerial_interference_sources（将其视为 aerial_interferers）。

"""

lst = scenario.get('aerial_interferers', None)

if isinstance(lst, list):

return lst

legacy = scenario.get('aerial_interference_sources', None)

if isinstance(legacy, list):

return legacy

return []

def calculate_aerial_interference_power_mw(self,

rx_pos: np.ndarray,

aerial_interferers: List['InterferenceSource'],

scenario: Dict,

rx_freq_hz: float,

rx_bw_hz: float,

rx_idx: Optional[int] = None) -> float:

"""

计算空中干扰功率 I_aer（mW）：按与 S(.) 同构的功率模板计算每个空中干扰源的到达功率并线性累加。

注意：这里默认按“空对空”/“空对点”的 mean path loss（LoS/NLoS）计算；

若你后续要更严格区分 U2U/ATG，可在此处替换 path loss 模块。

"""

if not aerial_interferers:

return 0.0

p_rx = np.asarray(rx_pos, dtype=float)

buildings_list = scenario.get('buildings', [])

total_mw = 0.0

for j, src in enumerate(aerial_interferers):

tx_pos = np.array([float(getattr(src, 'x')), float(getattr(src, 'y')), float(getattr(src, 'height'))], dtype=float)

tx_power_dbm = float(getattr(src, 'power'))

tx_freq = float(getattr(src, 'frequency', rx_freq_hz))

coupling = self._spectral_coupling_factor(

tx_freq_hz=tx_freq,

rx_freq_hz=float(rx_freq_hz),

rx_bw_hz=float(rx_bw_hz),

interferer_type_id=int(getattr(src, 'type_id', -1)),

)

if coupling <= 0.0:

continue

# 可复现的“干扰衰落”

fade = 1.0

if getattr(self, 'interference_fading_enabled', False):

u = self._stable_unit_float(

17.0,

float(rx_idx if rx_idx is not None else 0),

float(j) + 1.0,

float(tx_power_dbm),

float(tx_pos[0]), float(tx_pos[1]), float(tx_pos[2]),

float(p_rx[0]), float(p_rx[1]), float(p_rx[2]),

)

u = float(np.clip(u, 1e-12, 1.0 - 1e-12))

fade = float(-math.log(1.0 - u))  # Exp(1)

act = float(getattr(src, 'activity', 1.0))

rx_dbm, _ = self.calculate_rx_power_dbm(

tx_power_dbm=tx_power_dbm,

tx_pos=tx_pos,

rx_pos=p_rx,

freq_hz=float(tx_freq),

buildings=buildings_list,

force_nlos=False,

ignore_building_idx=None,

extra_loss_db=0.0,

fading_gain_linear=float(fade),

duty_cycle_activity=float(act),

)

total_mw += self._dbm_to_mw(rx_dbm) * float(coupling)

return float(total_mw)

def compute_sinr(self,

drone_pos: np.ndarray,

station_pos: np.ndarray,

scenario: Dict,

drone_idx: Optional[int] = None) -> Dict[str, float]:

"""SINR 统一接口：内部复用功率域模板，返回 SINR/Outage/速率等。"""

return self.calculate_power_margin_components(drone_pos, station_pos, scenario, drone_idx=drone_idx)

def compute_sinr_zeng(self,

drone_pos: np.ndarray,

station_pos: np.ndarray,

scenario: Dict,

drone_idx: Optional[int] = None,

include_aerial_interferers: bool = True) -> Dict[str, float]:

"""

Zeng 2019 “论文同款” SINR 接口（功率域实现）：

γ = S / (I_ter + I_aer + σ^2)

- S：沿用你现有的接收功率/LoS/NLoS/遮挡模型（不推翻）

- I_ter：默认使用 scenario['interference_sources']（已预计算可复用）

- I_aer：可选，来自 scenario['aerial_interferers']（空中干扰机/同频空中干扰源）；UAV↔UAV 通信不计入此项

"""

pm = self.calculate_power_margin_components(drone_pos, station_pos, scenario, drone_idx=drone_idx)

i_ter_mw = float(pm.get('I_y_mw', 0.0))

i_aer_mw = 0.0

if include_aerial_interferers:

aer_sources = self._get_aerial_interferers(scenario)

i_aer_mw = float(self.calculate_aerial_interference_power_mw(

rx_pos=np.asarray(drone_pos, dtype=float),

aerial_interferers=aer_sources,

scenario=scenario,

rx_freq_hz=float(self.communication_freq),

rx_bw_hz=float(getattr(self, 'bandwidth_hz', 20e6)),

rx_idx=drone_idx,

))

i_total_mw = float(max(i_ter_mw, 0.0) + max(i_aer_mw, 0.0))

ny_mw = float(pm.get('N_y_mw', self._noise_power_mw()))

pin_mw = float(i_total_mw + max(ny_mw, 0.0))

pin_dbm = float(self._mw_to_dbm(pin_mw))

n_dbm = float(self._mw_to_dbm(max(ny_mw, 0.0)))

delta_in_db = float(pin_dbm - n_dbm)

prx_dbm = float(pm.get('P_rx_dbm', -300.0))

m_db = float(prx_dbm - pin_dbm)

sinr_lin = float(10.0 ** (m_db / 10.0))

b = max(float(getattr(self, 'bandwidth_hz', 1.0)), 1.0)

rate_bps = float(b * np.log2(1.0 + sinr_lin))

outage = 1.0 if m_db < float(getattr(self, 'margin_threshold_db', 0.0)) else 0.0

out = dict(pm)

out.update({

'I_ter_mw': float(i_ter_mw),

'I_aer_mw': float(i_aer_mw),

'I_total_mw': float(i_total_mw),

'I_y_mw': float(i_total_mw),

'I_y_dbm': float(self._mw_to_dbm(max(i_total_mw, 0.0))),

'P_IN_dbm': float(pin_dbm),

'Delta_IN_db': float(delta_in_db),

'M_db': float(m_db),

'SINR_linear': float(sinr_lin),

'R_bps': float(rate_bps),

'Outage': float(outage),

})

return out

def calculate_link_degradation(self,

drone_pos: np.ndarray,

station_pos: np.ndarray,

scenario: Dict,

drone_speed_mps: Optional[float] = None,

drone_idx: Optional[int] = None) -> float:

comm_deg = self.calculate_comm_degradation(drone_pos, station_pos, scenario, drone_idx=drone_idx)

if drone_speed_mps is None:

speeds = scenario.get('drone_speeds', [])

drone_speed_mps = float(np.mean(speeds)) if len(speeds) else 0.0

speed_eff_deg = self.calculate_speed_energy_efficiency_degradation(drone_speed_mps)

weight_sum = sum(self.metric_weights.values())

total = (

self.metric_weights['power_intensity_change'] * comm_deg +

self.metric_weights['speed_energy_efficiency'] * speed_eff_deg

) / max(weight_sum, 1e-12)

return float(np.clip(total, 0.0, 1.0))

def analyze_scenario_metrics(self, scenario: Dict) -> Dict:

"""为可视化/报告生成一些汇总指标（不参与 GA）。"""

drones = np.asarray(scenario.get('drone_positions', np.zeros((0, 3))), dtype=float)

stations = np.asarray(scenario.get('station_positions', np.zeros((0, 3))), dtype=float)

if drones.size == 0 or stations.size == 0:

return {

'avg_comm_deg': 0.0,

'avg_speed_deg': 0.0,

'avg_total_deg': 0.0,

'avg_d_link_db': 0.0,

'avg_interference_dbm': -np.inf,

'avg_cdi': 0.0,

'avg_margin_db': 0.0,

'outage_prob': 0.0,

'avg_rate_mbps': 0.0,

'avg_sinr_db': -np.inf,

'model': str(getattr(self, 'degradation_model', 'cdi')),

}

comm_degs = []

total_degs = []

d_links = []

cdis = []

int_mws = []

margins = []

outages = []

rates_mbps = []

sinr_dbs = []

prx_dbms = []

pin_dbms = []

delta_in_dbs = []

iy_dbms = []

i_aer_dbms = []

u2u_comm_degs = []

for drone_idx in range(len(drones)):

for station_idx in range(len(stations)):

use_zeng = bool(self.user_params.get('sinr_zeng_enabled', False))

if use_zeng:

pm = self.compute_sinr_zeng(

drones[drone_idx],

stations[station_idx],

scenario,

drone_idx=int(drone_idx),

include_aerial_interferers=True,

)

else:

pm = self.calculate_power_margin_components(

drones[drone_idx],

stations[station_idx],

scenario,

drone_idx=int(drone_idx),

)

margins.append(float(pm.get('M_db', 0.0)))

outages.append(float(pm.get('Outage', 0.0)))

rates_mbps.append(float(pm.get('R_bps', 0.0)) / 1e6)

sinr_dbs.append(float(10.0 * np.log10(max(float(pm.get('SINR_linear', 0.0)), 1e-15))))

prx_dbms.append(float(pm.get('P_rx_dbm', 0.0)))

pin_dbms.append(float(pm.get('P_IN_dbm', 0.0)))

delta_in_dbs.append(float(pm.get('Delta_IN_db', 0.0)))

iy_dbms.append(float(pm.get('I_y_dbm', -np.inf)))

if 'I_aer_mw' in pm:

i_aer_dbms.append(float(self._mw_to_dbm(max(float(pm.get('I_aer_mw', 0.0)), 0.0))))

# 单独记录 U2U 的“有用信道”劣化，便于解释（不进入 I_aer）

u2u_d = self._u2u_comm_degradation_for_drone(scenario, int(drone_idx))

if u2u_d is not None:

u2u_comm_degs.append(float(u2u_d))

d_link, i_mw, cdi = self.calculate_cdi_components(

drones[drone_idx],

stations[station_idx],

scenario,

drone_idx=int(drone_idx),

)

d_links.append(float(d_link))

cdis.append(float(cdi))

int_mws.append(float(i_mw))

comm_degs.append(float(self.calculate_comm_degradation(

drones[drone_idx],

stations[station_idx],

scenario,

drone_idx=int(drone_idx),

)))

total_degs.append(float(self.calculate_link_degradation(

drones[drone_idx],

stations[station_idx],

scenario,

drone_speed_mps=float(scenario.get('drone_speeds', [0.0] * len(drones))[drone_idx]),

drone_idx=int(drone_idx),

)))

speed_degs = []

for v in scenario.get('drone_speeds', []):

speed_degs.append(float(self.calculate_speed_energy_efficiency_degradation(float(v))))

avg_i_mw = float(np.mean(int_mws)) if int_mws else 0.0

avg_i_dbm = float(self._mw_to_dbm(avg_i_mw)) if np.isfinite(avg_i_mw) else -np.inf

ee = self.calculate_energy_efficiency_metrics(scenario)

outline = {}

try:

outline = self.calculate_outline_total_score(scenario)

except Exception:

outline = {}

metrics = {

'avg_comm_deg': float(np.mean(comm_degs)) if comm_degs else 0.0,

'avg_speed_deg': float(np.mean(speed_degs)) if speed_degs else 0.0,

'avg_total_deg': float(np.mean(total_degs)) if total_degs else 0.0,

'avg_d_link_db': float(np.mean(d_links)) if d_links else 0.0,

'avg_interference_dbm': avg_i_dbm,

'avg_cdi': float(np.mean(cdis)) if cdis else 0.0,

'avg_margin_db': float(np.mean(margins)) if margins else 0.0,

'outage_prob': float(np.mean(outages)) if outages else 0.0,

'avg_rate_mbps': float(np.mean(rates_mbps)) if rates_mbps else 0.0,

'avg_sinr_db': float(np.mean(sinr_dbs)) if sinr_dbs else -np.inf,

'EE_bpj': float(ee.get('EE_bpj', 0.0)),

'sum_rate_bps': float(ee.get('sum_rate_bps', 0.0)),

'sum_propulsion_w': float(ee.get('sum_propulsion_w', 0.0)),

'tdma_enabled': float(ee.get('tdma_enabled', 0.0)) if isinstance(ee, dict) else 0.0,

'tdma_total_bits': float(ee.get('tdma_total_bits', 0.0)) if isinstance(ee, dict) else 0.0,

'tdma_total_energy_j': float(ee.get('tdma_total_energy_j', 0.0)) if isinstance(ee, dict) else 0.0,

'avg_prx_dbm': float(np.mean(prx_dbms)) if prx_dbms else 0.0,

'avg_pin_dbm': float(np.mean(pin_dbms)) if pin_dbms else 0.0,

'avg_delta_in_db': float(np.mean(delta_in_dbs)) if delta_in_dbs else 0.0,

'avg_iy_dbm': float(np.mean([v for v in iy_dbms if np.isfinite(v)])) if any(np.isfinite(v) for v in iy_dbms) else -np.inf,

'avg_i_aer_dbm': float(np.mean([v for v in i_aer_dbms if np.isfinite(v)])) if i_aer_dbms and any(np.isfinite(v) for v in i_aer_dbms) else -np.inf,

'avg_u2u_comm_deg': float(np.mean(u2u_comm_degs)) if u2u_comm_degs else 0.0,

'model': str(getattr(self, 'degradation_model', 'cdi')),

}

for k in ('D_total', 'D_comm', 'D_thr', 'D_EE', 'D_penalty'):

if k in outline:

metrics[k] = float(outline.get(k, 0.0))

try:

ps = self.calculate_power_score_metrics(scenario)

metrics.update({f'power_{k}': float(v) if isinstance(v, (int, float)) else v for k, v in ps.items()})

except Exception:

pass

return metrics

def _iter_a2g_links(self, scenario: Dict):

drones = np.asarray(scenario.get('drone_positions', np.zeros((0, 3))), dtype=float)

stations = np.asarray(scenario.get('station_positions', np.zeros((0, 3))), dtype=float)

for drone_idx in range(len(drones)):

for station_idx in range(len(stations)):

yield int(drone_idx), int(station_idx), drones[drone_idx], stations[station_idx]

def _iter_u2u_links(self, scenario: Dict):

if not getattr(self, 'u2u_enabled', False):

return

drones = np.asarray(scenario.get('drone_positions', np.zeros((0, 3))), dtype=float)

if len(drones) < 2:

return

pairing = str(getattr(self, 'u2u_pairing', 'all')).strip().lower()

if pairing == 'nearest':

for i in range(len(drones)):

d2 = np.sum((drones[:, :2] - drones[i, :2]) ** 2, axis=1)

d2[i] = np.inf

j = int(np.argmin(d2))

if np.isfinite(d2[j]):

yield int(i), int(j), drones[i], drones[j]

else:

for i in range(len(drones)):

for j in range(i + 1, len(drones)):

yield int(i), int(j), drones[i], drones[j]

def calculate_u2u_power_margin_components(self,

tx_drone_pos: np.ndarray,

rx_drone_pos: np.ndarray,

scenario: Dict,

tx_idx: int,

rx_idx: int) -> Dict[str, float]:

"""U2U：与 A2G 同构的功率预算，只是 tx/rx 都在空中。"""

buildings_list = scenario.get('buildings', [])

fade = self._stable_exp_fading_gain_for_link((

19.0,

float(tx_idx), float(rx_idx),

float(tx_drone_pos[0]), float(tx_drone_pos[1]), float(tx_drone_pos[2]),

float(rx_drone_pos[0]), float(rx_drone_pos[1]), float(rx_drone_pos[2]),

))

prx_dbm, p_los = self.calculate_rx_power_dbm(

tx_power_dbm=float(self.u2u_tx_power_dbm),

tx_pos=tx_drone_pos,

rx_pos=rx_drone_pos,

freq_hz=float(self.communication_freq),

buildings=buildings_list,

force_nlos=False,

ignore_building_idx=None,

fading_gain_linear=float(fade),

duty_cycle_activity=1.0,

)

prx_mw = float(self._dbm_to_mw(prx_dbm))

pl_db, _ = self.calculate_mean_path_loss_db(

tx_pos=tx_drone_pos, rx_pos=rx_drone_pos, freq_hz=float(self.communication_freq), buildings=buildings_list

)

iy_mw: Optional[float] = None

if 'interference_power_mw_per_drone' in scenario:

try:

iy_mw = float(scenario['interference_power_mw_per_drone'][int(rx_idx)])

except Exception:

iy_mw = None

if iy_mw is None:

iy_mw = float(self.calculate_interference_power_mw(

drone_pos=np.asarray(rx_drone_pos, dtype=float),

interference_sources=scenario.get('interference_sources', []),

buildings=buildings_list,

drone_idx=rx_idx,

))

ny_mw = float(self._noise_power_mw())

pin_mw = float(max(iy_mw, 0.0) + max(ny_mw, 0.0))

pin_dbm = float(self._mw_to_dbm(pin_mw))

n_dbm = float(self._mw_to_dbm(ny_mw))

delta_in_db = float(pin_dbm - n_dbm)

iy_dbm = float(self._mw_to_dbm(max(float(iy_mw), 0.0)))

m_db = float(prx_dbm - pin_dbm)

sinr_lin = float(10.0 ** (m_db / 10.0))

b = max(float(getattr(self, 'bandwidth_hz', 1.0)), 1.0)

rate_bps = float(b * np.log2(1.0 + sinr_lin))

outage = 1.0 if m_db < float(getattr(self, 'margin_threshold_db', 0.0)) else 0.0

return {

'P_rx_dbm': float(prx_dbm),

'P_rx_mw': float(prx_mw),

'I_y_mw': float(iy_mw),

'I_y_dbm': float(iy_dbm),

'N_y_mw': float(ny_mw),

'N_y_dbm': float(n_dbm),

'P_IN_dbm': float(pin_dbm),

'Delta_IN_db': float(delta_in_db),

'M_db': float(m_db),

'SINR_linear': float(sinr_lin),

'R_bps': float(rate_bps),

'Outage': float(outage),

'PL_db': float(pl_db),

'P_LoS': float(p_los),

}

def calculate_power_score_metrics(self, scenario: Dict) -> Dict[str, float]:

"""

按大纲的“功率主导”输出三类概率指标：

- P_succ(θ_p)=P(P_rx>θ_p)

- P_int(θ_I)=P(I_agg>θ_I)

- P_PL=P(PL>PL_max)

并给出最终拼接分数 D = w1(1-P_succ)+w2 P_int + w3 P_PL（越大表示越差）。

"""

theta_p_dbm = float(getattr(self, 'succ_threshold_dbm', -90.0))

theta_i_dbm = float(getattr(self, 'int_threshold_dbm', -80.0))

pl_max_db = float(getattr(self, 'pl_max_db', 125.0))

w = dict(getattr(self, 'power_score_weights', {}))

w1 = float(w.get('w1_succ', 1.0))

w2 = float(w.get('w2_int', 1.0))

w3 = float(w.get('w3_pl', 1.0))

prx_hits = 0

int_hits = 0

pl_hits = 0

n_links = 0

# A2G links (station -> drone)

for drone_idx, station_idx, drone_pos, station_pos in self._iter_a2g_links(scenario):

comp = self.calculate_power_margin_components(drone_pos, station_pos, scenario, drone_idx=drone_idx)

n_links += 1

if float(comp['P_rx_dbm']) > theta_p_dbm:

prx_hits += 1

if float(comp['I_y_dbm']) > theta_i_dbm:

int_hits += 1

if float(comp['PL_db']) > pl_max_db:

pl_hits += 1

# U2U links (optional)

if getattr(self, 'u2u_enabled', False):

for i, j, pi, pj in self._iter_u2u_links(scenario):

comp_ij = self.calculate_u2u_power_margin_components(pi, pj, scenario, tx_idx=i, rx_idx=j)

n_links += 1

if float(comp_ij['P_rx_dbm']) > theta_p_dbm:

prx_hits += 1

if float(comp_ij['I_y_dbm']) > theta_i_dbm:

int_hits += 1

if float(comp_ij['PL_db']) > pl_max_db:

pl_hits += 1

if n_links <= 0:

return {

'P_succ': 0.0,

'P_int': 0.0,

'P_PL': 0.0,

'D': 0.0,

'theta_p_dbm': theta_p_dbm,

'theta_i_dbm': theta_i_dbm,

'pl_max_db': pl_max_db,

}

p_succ = float(prx_hits / n_links)

p_int = float(int_hits / n_links)

p_pl = float(pl_hits / n_links)

d_score = float(w1 * (1.0 - p_succ) + w2 * p_int + w3 * p_pl)

return {

'P_succ': p_succ,

'P_int': p_int,

'P_PL': p_pl,

'D': d_score,

'theta_p_dbm': theta_p_dbm,

'theta_i_dbm': theta_i_dbm,

'pl_max_db': pl_max_db,

}

def evalVars(self, x: np.ndarray) -> Tuple[np.ndarray, np.ndarray]:

"""评估种群"""

N = x.shape[0]

f = np.zeros((N, 1))

for i in range(N):

try:

# 生成场景

scenario = self.generate_scenario(x[i, :])

obj_model = str(getattr(self, 'objective_model', 'power_score')).strip().lower()

if obj_model in ('power_score', 'outline', 'power'):

pm = self.calculate_power_score_metrics(scenario)

# D 越大表示越差（更“劣化”），与 maxormins=-1 的“最大化”保持一致

f[i, 0] = float(pm.get('D', 0.0))

elif obj_model in ('outline_total', 'd_total', 'total'):

out = self.calculate_outline_total_score(scenario)

f[i, 0] = float(out.get('D_total', 0.0))

elif obj_model in ('ee', 'ee_bpj', 'energy', 'energy_efficiency'):

ee = self.calculate_energy_efficiency_metrics(scenario)

f[i, 0] = float(ee.get('EE_bpj', 0.0))

elif obj_model in ('ee_log', 'ee_bpj_log', 'energy_log'):

ee = self.calculate_energy_efficiency_metrics(scenario)

f[i, 0] = float(np.log10(1.0 + float(ee.get('EE_bpj', 0.0))))

else:

# 兼容旧目标：平均劣化（可包含速度项）

total_degradation = 0.0

link_count = 0

for drone_idx in range(self.num_drones):

for station_idx in range(self.num_stations):

degradation = self.calculate_link_degradation(

scenario['drone_positions'][drone_idx],

scenario['station_positions'][station_idx],

scenario,

drone_speed_mps=float(scenario['drone_speeds'][drone_idx]),

drone_idx=int(drone_idx)

)

total_degradation += float(degradation)

link_count += 1

f[i, 0] = float(total_degradation / link_count) if link_count > 0 else 0.01

except Exception as e:

print(f"评估染色体时出错: {e}")

f[i, 0] = 0.01

return f, np.zeros((N, 1))

class EnhancedVisualizer:

"""增强版场景可视化类"""

def __init__(self, scenario: Dict):

self.scenario = scenario

# 可视化开关：默认“全量绘制”（用户要求），避免只画子集导致数量不一致

# 仍保留场景级开关，便于在极端密度下回退到子集显示

self.viz_plot_all_sources = bool(scenario.get('viz_plot_all_sources', True))

self.viz_plot_all_buildings = bool(scenario.get('viz_plot_all_buildings', True))

# 按原来风格：在顶视图标注建筑高度

self.viz_annotate_building_heights = bool(scenario.get('viz_annotate_building_heights', True))

self.viz_plot_interference_ranges = bool(

scenario.get('viz_plot_interference_ranges', not self.viz_plot_all_sources)

)

self.colors = {

'drones': '#FF6B6B',  # 红色 - 无人机（三角形）

'stations': '#4ECDC4',  # 青色 - 地面站（方形）

'buildings': '#45B7D1',  # 蓝色 - 建筑物

'ground': '#DFE6E9',  # 浅灰色 - 地面

'text': '#2D3436',  # 深灰色 - 文字

'interference': '#FFD700'  # 金色 - 干扰源（统一为星型）

}

# 干扰源颜色映射 - 使用干扰源自身的颜色

self.interference_colors = {}

for source in scenario.get('interference_sources', []):

self.interference_colors[source.name] = source.color

def _sources_for_plot(self) -> List:

"""获取要绘制的干扰源集合。默认全量绘制；可用 viz_plot_all_sources 回退到子集。"""

if self.viz_plot_all_sources:

sources = self.scenario.get('interference_sources', [])

else:

sources = self.scenario.get('interference_sources_viz', None)

if sources is None:

sources = self.scenario.get('interference_sources', [])

return list(sources) if sources else []

@staticmethod

def _count_sources_by_name(sources: List) -> Dict[str, int]:

counts: Dict[str, int] = {}

for s in sources or []:

name = getattr(s, 'name', 'unknown')

counts[name] = counts.get(name, 0) + 1

return counts

@staticmethod

def _count_ground_building(sources: List) -> Tuple[int, int]:

ground = 0

building = 0

for s in sources or []:

if getattr(s, 'is_ground', False):

ground += 1

else:

building += 1

return ground, building

def _format_source_summary(self, sources_plot: List, sources_all: List) -> str:

shown_total = len(sources_plot or [])

total_total = len(sources_all or [])

shown_ground, shown_building = self._count_ground_building(sources_plot)

total_ground, total_building = self._count_ground_building(sources_all)

return (

f"干扰源: 显示{shown_total}/总{total_total}\n"

f"  建筑: {shown_building}/{total_building}\n"

f"  地面: {shown_ground}/{total_ground}"

)

def _buildings_for_plot(self, limit: int = 60) -> List['Building']:

"""获取要绘制的建筑集合。默认全量绘制；否则确保与可视化干扰源关联的建筑一定会被画出来。"""

buildings = list(self.scenario.get('buildings', []))

if not buildings:

return []

if self.viz_plot_all_buildings:

return buildings

sources = self._sources_for_plot()

assoc_idx = []

for s in sources:

b_idx = getattr(s, 'building_idx', None)

b = getattr(s, 'building', None)

if b_idx is not None:

assoc_idx.append(int(b_idx))

elif b is not None:

try:

assoc_idx.append(int(buildings.index(b)))

except Exception:

pass

assoc_idx = sorted(set([i for i in assoc_idx if 0 <= i < len(buildings)]))

assoc_buildings = [buildings[i] for i in assoc_idx]

limit = int(max(limit, len(assoc_buildings)))

if len(buildings) <= limit:

return buildings

remaining = [b for i, b in enumerate(buildings) if i not in set(assoc_idx)]

remaining.sort(key=lambda bb: float(getattr(bb, 'height', 0.0)), reverse=True)

return assoc_buildings + remaining[: max(0, limit - len(assoc_buildings))]

def create_individual_plots(self, prefix: str = "") -> List[Tuple[str, str]]:

"""创建单独的图表"""

timestamp = time.strftime("%Y%m%d_%H%M%S")

prefix = f"{prefix}_{timestamp}" if prefix else timestamp

plots_info = []

try:

# 1. 3D场景图

fig1 = plt.figure(figsize=(14, 10))

ax1 = fig1.add_subplot(111, projection='3d')

self._plot_3d_scene(ax1)

plt.tight_layout()

filename1 = f"output/3d_scene_{prefix}.png"

plt.savefig(filename1, dpi=150, bbox_inches='tight')

plt.close(fig1)

plots_info.append(("3D场景图", filename1))

# 2. 顶视图

fig2, ax2 = plt.subplots(figsize=(12, 10))

self._plot_top_view(ax2)

plt.tight_layout()

filename2 = f"output/top_view_{prefix}.png"

plt.savefig(filename2, dpi=150, bbox_inches='tight')

plt.close(fig2)

plots_info.append(("顶视图", filename2))

# 3. 侧视图

fig3, ax3 = plt.subplots(figsize=(12, 10))

self._plot_side_view(ax3)

plt.tight_layout()

filename3 = f"output/side_view_{prefix}.png"

plt.savefig(filename3, dpi=150, bbox_inches='tight')

plt.close(fig3)

plots_info.append(("侧视图", filename3))

# 4. 干扰源分布

fig4, ax4 = plt.subplots(figsize=(12, 8))

self._plot_interference_distribution(ax4)

plt.tight_layout()

filename4 = f"output/interference_dist_{prefix}.png"

plt.savefig(filename4, dpi=150, bbox_inches='tight')

plt.close(fig4)

plots_info.append(("干扰源分布", filename4))

# 5. 高度分析

fig5, ax5 = plt.subplots(figsize=(12, 8))

self._plot_height_analysis(ax5)

plt.tight_layout()

filename5 = f"output/height_analysis_{prefix}.png"

plt.savefig(filename5, dpi=150, bbox_inches='tight')

plt.close(fig5)

plots_info.append(("高度分析", filename5))

# 6. 通信链路分析

fig6, ax6 = plt.subplots(figsize=(12, 8))

self._plot_communication_analysis(ax6)

plt.tight_layout()

filename6 = f"output/communication_analysis_{prefix}.png"

plt.savefig(filename6, dpi=150, bbox_inches='tight')

plt.close(fig6)

plots_info.append(("通信链路分析", filename6))

# 7. 场景参数图

fig7, ax7 = plt.subplots(figsize=(12, 8))

self._plot_scenario_parameters(ax7)

plt.tight_layout()

filename7 = f"output/scenario_params_{prefix}.png"

plt.savefig(filename7, dpi=150, bbox_inches='tight')

plt.close(fig7)

plots_info.append(("场景参数", filename7))

# 8. 干扰源位置类型分析

fig8, ax8 = plt.subplots(figsize=(12, 8))

self._plot_interference_location_type(ax8)

plt.tight_layout()

filename8 = f"output/interference_location_{prefix}.png"

plt.savefig(filename8, dpi=150, bbox_inches='tight')

plt.close(fig8)

plots_info.append(("干扰源位置类型", filename8))

except Exception as e:

print(f"生成图表时出错: {e}")

import traceback

traceback.print_exc()

return plots_info

def create_comprehensive_report(self, prefix: str = "") -> str:

"""创建综合报告图"""

timestamp = time.strftime("%Y%m%d_%H%M%S")

filename = f"output/comprehensive_report_{prefix}_{timestamp}.png"

fig = plt.figure(figsize=(20, 16))

# 1. 3D场景图

ax1 = fig.add_subplot(331, projection='3d')

self._plot_3d_scene(ax1)

# 2. 顶视图

ax2 = fig.add_subplot(332)

self._plot_top_view(ax2)

# 3. 侧视图

ax3 = fig.add_subplot(333)

self._plot_side_view(ax3)

# 4. 干扰源分析

ax4 = fig.add_subplot(334)

self._plot_interference_analysis(ax4)

# 5. 高度分析

ax5 = fig.add_subplot(335)

self._plot_detailed_height_analysis(ax5)

# 6. 通信链路分析（极坐标雷达图）

ax6 = fig.add_subplot(336, polar=True)

self._plot_detailed_communication_analysis(ax6)

# 7. 场景参数

ax7 = fig.add_subplot(337)

self._plot_detailed_scenario_parameters(ax7)

# 8. 干扰源详情

ax8 = fig.add_subplot(338)

self._plot_interference_details(ax8)

# 9. 干扰源位置类型

ax9 = fig.add_subplot(339)

self._plot_interference_location_type(ax9)

plt.suptitle('无人机通信场景综合报告', fontsize=18, fontweight='bold', y=0.98)

plt.tight_layout(rect=[0, 0, 1, 0.96])

plt.savefig(filename, dpi=150, bbox_inches='tight')

plt.show()

return filename

def _plot_3d_scene(self, ax):

"""绘制3D场景"""

area_size = self.scenario['area_size']

# 绘制地面

xx, yy = np.meshgrid([0, area_size], [0, area_size])

zz = np.zeros_like(xx)

ax.plot_surface(xx, yy, zz, alpha=0.1, color=self.colors['ground'])

# 绘制无人机 - 使用三角形标记

drones = self.scenario['drone_positions']

if len(drones) > 0:

ax.scatter(drones[:, 0], drones[:, 1], drones[:, 2],

c=self.colors['drones'], s=150, marker='^',

label=f'无人机 ({len(drones)}个)', depthshade=True)

# 标注无人机高度和速度

for i, drone in enumerate(drones):

speed = self.scenario['drone_speeds'][i] if i < len(self.scenario['drone_speeds']) else 0

ax.text(drone[0], drone[1], drone[2] + 10,

f'U{i}:{drone[2]:.0f}m\n{speed:.1f}m/s',

fontsize=8, ha='center')

# 绘制地面站 - 使用方形标记

stations = self.scenario['station_positions']

if len(stations) > 0:

ax.scatter(stations[:, 0], stations[:, 1], stations[:, 2],

c=self.colors['stations'], s=120, marker='s',

label=f'地面站 ({len(stations)}个)', depthshade=True)

for i, station in enumerate(stations):

ax.text(station[0], station[1], station[2] + 5,

f'S{i}', fontsize=8, ha='center')

# 绘制建筑物

buildings = self._buildings_for_plot(limit=60)

for building in buildings:

# 绘制建筑物立方体

vertices = [

[building.x - building.width / 2, building.y - building.length / 2, 0],

[building.x + building.width / 2, building.y - building.length / 2, 0],

[building.x + building.width / 2, building.y + building.length / 2, 0],

[building.x - building.width / 2, building.y + building.length / 2, 0],

[building.x - building.width / 2, building.y - building.length / 2, building.height],

[building.x + building.width / 2, building.y - building.length / 2, building.height],

[building.x + building.width / 2, building.y + building.length / 2, building.height],

[building.x - building.width / 2, building.y + building.length / 2, building.height]

]

from mpl_toolkits.mplot3d.art3d import Poly3DCollection

faces = [

[vertices[0], vertices[1], vertices[2], vertices[3]],  # 底面

[vertices[4], vertices[5], vertices[6], vertices[7]],  # 顶面

[vertices[0], vertices[1], vertices[5], vertices[4]],  # 侧面

[vertices[2], vertices[3], vertices[7], vertices[6]],  # 侧面

]

ax.add_collection3d(Poly3DCollection(faces, facecolors=building.color,

alpha=0.6, linewidths=0.5, edgecolors='k'))

# 绘制干扰源 - 优先用可视化子集，避免“建筑未画出导致看起来悬空”

sources_all = list(self.scenario.get('interference_sources', []) or [])

sources_plot = self._sources_for_plot()

total_by_name = self._count_sources_by_name(sources_all)

shown_by_name = self._count_sources_by_name(sources_plot)

grouped: Dict[str, List] = {}

for s in sources_plot:

grouped.setdefault(getattr(s, 'name', 'unknown'), []).append(s)

for name, group in grouped.items():

xs = [float(s.x) for s in group]

ys = [float(s.y) for s in group]

zs = [float(s.height) for s in group]

sizes = [80.0 + float(getattr(s, 'power', 0.0)) * 2.0 for s in group]

colors = [getattr(s, 'color', self.colors['interference']) for s in group]

label = f"{name} ({shown_by_name.get(name, 0)}/{total_by_name.get(name, 0)})"

ax.scatter(xs, ys, zs, c=colors, s=sizes, marker='*', label=label, depthshade=True)

# 绘制干扰范围（简化表示）：仍按“显示子集”逐个画，避免过度拥挤

if self.viz_plot_interference_ranges and len(sources_plot) <= 200:

for source in sources_plot:

theta = np.linspace(0, 2 * np.pi, 20)

r = float(getattr(source, 'coverage_radius', 0.0)) * 0.1

if r > 0:

color = getattr(source, 'color', self.colors['interference'])

x_circle = source.x + r * np.cos(theta)

y_circle = source.y + r * np.sin(theta)

z_circle = np.full_like(x_circle, source.height + 2)

ax.plot(x_circle, y_circle, z_circle, color=color, alpha=0.25, linewidth=0.8)

# 设置坐标轴

ax.set_xlabel('X (m)')

ax.set_ylabel('Y (m)')

ax.set_zlabel('高度 (m)')

ax.set_title('3D场景视图')

ax.set_xlim(0, area_size)

ax.set_ylim(0, area_size)

max_height = max(self.scenario['drone_height_max'], self.scenario['building_height_max'])

ax.set_zlim(0, max_height + 50)

# 显示“真实总数 vs 当前展示数量”（解决：生成数量未展现）

ax.text2D(0.02, 0.98, self._format_source_summary(sources_plot, sources_all),

transform=ax.transAxes, va='top', ha='left', fontsize=9,

bbox=dict(facecolor='white', alpha=0.75, edgecolor='none'))

# 简化图例

handles, labels = ax.get_legend_handles_labels()

unique = {}

for h, l in zip(handles, labels):

if l not in unique:

unique[l] = h

if unique:

ax.legend(unique.values(), unique.keys(), loc='upper right', fontsize=8)

ax.view_init(elev=35, azim=45)

ax.grid(True, alpha=0.3)

def _plot_top_view(self, ax):

"""绘制顶视图"""

area_size = self.scenario['area_size']

# 绘制无人机 - 三角形标记

drones = self.scenario['drone_positions']

if len(drones) > 0:

ax.scatter(drones[:, 0], drones[:, 1],

c=self.colors['drones'], s=100, marker='^', label='无人机')

# 绘制地面站 - 方形标记

stations = self.scenario['station_positions']

if len(stations) > 0:

ax.scatter(stations[:, 0], stations[:, 1],

c=self.colors['stations'], s=100, marker='s', label='地面站')

# 绘制建筑物

buildings = self._buildings_for_plot(limit=80)

for building in buildings:

rect = Rectangle((building.x - building.width / 2, building.y - building.length / 2),

building.width, building.length,

facecolor=building.color, alpha=0.6, edgecolor='k', linewidth=0.5)

ax.add_patch(rect)

# 标注建筑物高度（全量绘制时默认关闭，避免遮挡）

if self.viz_annotate_building_heights:

ax.text(building.x, building.y, f'{building.height:.0f}m',

fontsize=7, ha='center', va='center')

# 绘制干扰源 - 优先用可视化子集

sources_all = list(self.scenario.get('interference_sources', []) or [])

sources_plot = self._sources_for_plot()

for source in sources_plot:

color = getattr(source, 'color', self.colors['interference'])

ax.scatter(source.x, source.y, c=color, s=120, marker='*')

freq_ghz = float(getattr(source, 'frequency', 0.0)) / 1e9

location_type = "地面" if getattr(source, 'is_ground', False) else "建筑物/空中"

ax.text(source.x, source.y + 5, f'{source.name}\n{location_type}\n{freq_ghz:.1f}GHz',

fontsize=6, ha='center', va='bottom')

ax.set_xlabel('X (m)')

ax.set_ylabel('Y (m)')

ax.set_title('顶视图 (XY平面)')

ax.set_xlim(0, area_size)

ax.set_ylim(0, area_size)

ax.set_aspect('equal')

ax.grid(True, alpha=0.3)

ax.legend(loc='upper right', fontsize=9)

ax.text(0.02, 0.98, self._format_source_summary(sources_plot, sources_all),

transform=ax.transAxes, va='top', ha='left', fontsize=8,

bbox=dict(facecolor='white', alpha=0.75, edgecolor='none'))

def _plot_side_view(self, ax):

"""绘制侧视图"""

area_size = self.scenario['area_size']

# 绘制无人机 - 三角形标记

drones = self.scenario['drone_positions']

if len(drones) > 0:

ax.scatter(drones[:, 0], drones[:, 2],

c=self.colors['drones'], s=100, marker='^', label='无人机')

# 绘制地面站 - 方形标记

stations = self.scenario['station_positions']

if len(stations) > 0:

ax.scatter(stations[:, 0], stations[:, 2],

c=self.colors['stations'], s=100, marker='s', label='地面站')

# 绘制建筑物

buildings = self._buildings_for_plot(limit=60)

for building in buildings:

rect = Rectangle((building.x - building.width / 2, 0),

building.width, building.height,

facecolor=building.color, alpha=0.6, edgecolor='k', linewidth=0.5)

ax.add_patch(rect)

# 绘制干扰源 - 优先用可视化子集

sources_all = list(self.scenario.get('interference_sources', []) or [])

sources_plot = self._sources_for_plot()

for source in sources_plot:

color = getattr(source, 'color', self.colors['interference'])

ax.scatter(source.x, source.height, c=color, s=100, marker='*')

# 绘制高度参考线

ax.axhline(y=100, color='gray', linestyle='--', alpha=0.5, linewidth=1)

ax.text(area_size * 0.8, 110, '低空 (<100m)', fontsize=8, color='gray')

ax.axhline(y=300, color='gray', linestyle='--', alpha=0.5, linewidth=1)

ax.text(area_size * 0.8, 310, '高空 (>300m)', fontsize=8, color='gray')

ax.set_xlabel('X (m)')

ax.set_ylabel('高度 (m)')

ax.set_title('侧视图 (XZ平面)')

ax.set_xlim(0, area_size)

max_height = max(self.scenario['drone_height_max'], self.scenario['building_height_max'])

ax.set_ylim(0, max_height + 50)

ax.grid(True, alpha=0.3)

ax.legend(loc='upper right', fontsize=9)

ax.text(0.02, 0.98, self._format_source_summary(sources_plot, sources_all),

transform=ax.transAxes, va='top', ha='left', fontsize=8,

bbox=dict(facecolor='white', alpha=0.75, edgecolor='none'))

def _plot_interference_distribution(self, ax):

"""绘制干扰源分布"""

if 'interference_sources' not in self.scenario or not self.scenario['interference_sources']:

ax.text(0.5, 0.5, '无干扰源', ha='center', va='center', fontsize=12)

ax.set_title('干扰源分布')

return

sources = self.scenario['interference_sources']

# 按类型和位置类型统计

type_counts = {}

ground_counts = {}

building_counts = {}

for source in sources:

type_name = source.name

if type_name not in type_counts:

type_counts[type_name] = 0

ground_counts[type_name] = 0

building_counts[type_name] = 0

type_counts[type_name] += 1

if source.is_ground:

ground_counts[type_name] += 1

else:

building_counts[type_name] += 1

# 绘制分组条形图

types = list(type_counts.keys())

x = np.arange(len(types))

width = 0.35

# 计算地面和建筑物干扰源数量

ground_values = [ground_counts[t] for t in types]

building_values = [building_counts[t] for t in types]

# 使用干扰源颜色

colors = [self.interference_colors.get(t, '#DDA0DD') for t in types]

bars1 = ax.bar(x - width / 2, ground_values, width, label='地面干扰源',

color=colors, alpha=0.8, edgecolor='k')

bars2 = ax.bar(x + width / 2, building_values, width, label='建筑物干扰源',

color=colors, alpha=0.6, edgecolor='k', hatch='//')

ax.set_xlabel('干扰源类型')

ax.set_ylabel('数量')

ax.set_title('干扰源类型与位置分布')

ax.set_xticks(x)

ax.set_xticklabels(types, rotation=45, ha='right')

ax.legend()

ax.grid(True, alpha=0.3, axis='y')

# 标注数值

for bars in [bars1, bars2]:

for bar in bars:

height = bar.get_height()

if height > 0:

ax.text(bar.get_x() + bar.get_width() / 2., height,

'%d' % int(height), ha='center', va='bottom', fontsize=8)

def _plot_interference_location_type(self, ax):

"""绘制干扰源位置类型分布"""

if 'interference_sources' not in self.scenario or not self.scenario['interference_sources']:

ax.text(0.5, 0.5, '无干扰源', ha='center', va='center', fontsize=12)

ax.set_title('干扰源位置类型')

return

sources = self.scenario['interference_sources']

# 统计位置类型：地面 / 屋顶(室外) / 室内(楼层)

ground_count = sum(1 for s in sources if getattr(s, 'is_ground', False))

indoor_count = sum(1 for s in sources if getattr(s, 'is_indoor', False))

rooftop_or_outdoor_count = len(sources) - ground_count - indoor_count

# 绘制饼图（若没有室内源则保持两类）

if indoor_count > 0:

labels = ['地面', '屋顶/室外楼层', '室内(楼层)']

sizes = [ground_count, rooftop_or_outdoor_count, indoor_count]

colors = ['#FF6B6B', '#45B7D1', '#96CEB4']

else:

labels = ['地面干扰源', '建筑物干扰源']

sizes = [ground_count, len(sources) - ground_count]

colors = ['#FF6B6B', '#45B7D1']

wedges, texts, autotexts = ax.pie(sizes, labels=labels, colors=colors, autopct='%1.1f%%',

startangle=90)

ax.set_title('干扰源位置类型分布')

# 美化文本

for autotext in autotexts:

autotext.set_color('white')

autotext.set_fontsize(10)

autotext.set_fontweight('bold')

def _plot_height_analysis(self, ax):

"""绘制高度分析"""

drones = self.scenario['drone_positions']

buildings = self.scenario['buildings']

sources = self.scenario.get('interference_sources', [])

# 收集高度数据

drone_heights = drones[:, 2] if len(drones) > 0 else np.array([], dtype=float)

building_heights = [b.height for b in buildings] if buildings else []

# 收集干扰源高度数据

ground_interference_heights = [s.height for s in sources if getattr(s, 'is_ground', False)]

building_interference_heights = [

s.height for s in sources

if (not getattr(s, 'is_ground', False)) and (not getattr(s, 'is_indoor', False))

]

indoor_interference_heights = [s.height for s in sources if getattr(s, 'is_indoor', False)]

# 创建子图

if drone_heights.size > 0:

# 绘制所有高度数据的直方图

all_heights = []

all_labels = []

all_colors = []

if drone_heights.size > 0:

all_heights.append(drone_heights)

all_labels.append(f'无人机 (平均:{np.mean(drone_heights):.1f}m)')

all_colors.append(self.colors['drones'])

if building_heights:

all_heights.append(building_heights)

all_labels.append(f'建筑物 (平均:{np.mean(building_heights):.1f}m)')

all_colors.append(self.colors['buildings'])

if ground_interference_heights:

all_heights.append(ground_interference_heights)

all_labels.append(f'地面干扰源 (高度:0m)')

all_colors.append('#FF6B6B')  # 红色

if building_interference_heights:

all_heights.append(building_interference_heights)

all_labels.append(f'建筑物干扰源 (平均:{np.mean(building_interference_heights):.1f}m)')

all_colors.append('#45B7D1')  # 蓝色

# 绘制分组直方图

if indoor_interference_heights:

all_heights.append(indoor_interference_heights)

all_labels.append(f'室内(楼层)干扰源 (平均:{np.mean(indoor_interference_heights):.1f}m)')

all_colors.append('#96CEB4')

for heights, label, color in zip(all_heights, all_labels, all_colors):

ax.hist(heights, bins=15, alpha=0.7, label=label, color=color)

ax.set_xlabel('高度 (m)')

ax.set_ylabel('数量')

ax.set_title('高度分布对比')

ax.legend(fontsize=9)

ax.grid(True, alpha=0.3)

else:

ax.text(0.5, 0.5, '无高度数据', ha='center', va='center', fontsize=12)

ax.set_title('高度分析')

def _plot_communication_analysis(self, ax):

"""绘制通信链路分析"""

drones = self.scenario['drone_positions']

stations = self.scenario['station_positions']

if len(drones) == 0 or len(stations) == 0:

ax.text(0.5, 0.5, '无通信链路', ha='center', va='center', fontsize=12)

ax.set_title('通信链路分析')

return

area_size = self.scenario['area_size']

# 计算所有无人机到最近地面站的距离

distances = []

for drone in drones:

min_dist = float('inf')

for station in stations:

dist = np.linalg.norm(drone[:2] - station[:2])  # 仅考虑水平距离

if dist < min_dist:

min_dist = dist

distances.append(min_dist)

# 绘制距离分布

ax.hist(distances, bins=min(10, len(distances)), color='#FFEAA7', alpha=0.7, edgecolor='k')

ax.axvline(np.mean(distances), color='red', linestyle='--', linewidth=2,

label=f'平均距离: {np.mean(distances):.1f}m')

ax.set_xlabel('通信距离 (m)')

ax.set_ylabel('无人机数量')

ax.set_title('无人机到最近地面站距离分布')

ax.legend()

ax.grid(True, alpha=0.3)

def _plot_scenario_parameters(self, ax):

"""绘制场景参数"""

ax.axis('off')

# 基本参数

text_lines = [

"场景参数",

"=" * 40,

f"区域大小: {self.scenario['area_size']}m × {self.scenario['area_size']}m",

f"无人机数量: {len(self.scenario['drone_positions'])}",

f"地面站数量: {len(self.scenario['station_positions'])}",

f"建筑物数量: {len(self.scenario['buildings'])}",

f"干扰源数量: {len(self.scenario.get('interference_sources', []))}",

f"建筑物密度: {self.scenario['building_density']:.2f}",

f"建筑物最大高度: {self.scenario['building_height_max']:.1f}m",

f"无人机最大高度: {self.scenario['drone_height_max']:.1f}m",

f"无人机最大速度: {self.scenario['drone_speed_max']:.1f}m/s",

]

# 无人机信息

if len(self.scenario['drone_positions']) > 0:

text_lines.append("\n无人机信息:")

avg_height = np.mean(self.scenario['drone_positions'][:, 2])

avg_speed = np.mean(self.scenario['drone_speeds']) if 'drone_speeds' in self.scenario else 0

text_lines.append(f"  平均高度: {avg_height:.1f}m")

text_lines.append(f"  平均速度: {avg_speed:.1f}m/s")

# 干扰源信息

if self.scenario.get('interference_sources'):

text_lines.append("\n干扰源信息:")

ground_count = sum(1 for s in self.scenario['interference_sources'] if s.is_ground)

building_count = len(self.scenario['interference_sources']) - ground_count

text_lines.append(f"  地面干扰源: {ground_count}个")

text_lines.append(f"  建筑物干扰源: {building_count}个")

# 功率域通信指标（若上层已注入 analysis）

analysis = self.scenario.get('analysis', {}) if isinstance(self.scenario, dict) else {}

if analysis:

text_lines.append("\n通信(功率域)")

if 'avg_margin_db' in analysis:

text_lines.append(f"  M(功率裕量): {float(analysis.get('avg_margin_db', 0.0)):.2f} dB")

if 'outage_prob' in analysis:

text_lines.append(f"  Outage: {float(analysis.get('outage_prob', 0.0)) * 100.0:.1f}%")

if 'avg_rate_mbps' in analysis:

text_lines.append(f"  R: {float(analysis.get('avg_rate_mbps', 0.0)):.2f} Mbps")

if 'avg_sinr_db' in analysis and np.isfinite(float(analysis.get('avg_sinr_db', -np.inf))):

text_lines.append(f"  SINR(dB): {float(analysis.get('avg_sinr_db', 0.0)):.2f} dB")

text = "\n".join(text_lines)

ax.text(0.05, 0.95, text, transform=ax.transAxes, fontsize=9,

verticalalignment='top', bbox=dict(boxstyle='round', facecolor='wheat', alpha=0.7))

def _plot_interference_analysis(self, ax):

"""绘制干扰源详细分析"""

if 'interference_sources' not in self.scenario or not self.scenario['interference_sources']:

ax.text(0.5, 0.5, '无干扰源', ha='center', va='center')

ax.set_title('干扰源分析')

return

sources = self.scenario['interference_sources']

# 绘制功率和高度散点图

powers = [s.power for s in sources]

heights = [s.height for s in sources]

colors = [s.color for s in sources]  # 使用干扰源自身的颜色

# 统一使用星型标记

for i in range(len(sources)):

ax.scatter(powers[i], heights[i], c=colors[i], s=100, alpha=0.7, marker='*')

ax.set_xlabel('功率 (dBm)')

ax.set_ylabel('高度 (m)')

ax.set_title('干扰源功率-高度分布')

ax.grid(True, alpha=0.3)

# 添加图例 - 使用干扰源自身的颜色

unique_sources = {}

for source in sources:

if source.name not in unique_sources:

unique_sources[source.name] = source.color

legend_handles = []

for name, color in unique_sources.items():

legend_handles.append(Line2D([0], [0], marker='*', color='w',

markerfacecolor=color, markersize=10, label=name))

if legend_handles:

ax.legend(handles=legend_handles, fontsize=8)

def _plot_detailed_height_analysis(self, ax):

"""绘制详细高度分析"""

drones = self.scenario['drone_positions']

buildings = self.scenario['buildings']

if len(drones) == 0:

ax.text(0.5, 0.5, '无无人机数据', ha='center', va='center')

ax.set_title('高度分析')

return

# 绘制无人机高度分布

heights = drones[:, 2]

x_positions = np.arange(len(heights))

bars = ax.bar(x_positions, heights, color=self.colors['drones'], alpha=0.7)

ax.axhline(y=100, color='gray', linestyle='--', alpha=0.5, label='低空界限')

ax.axhline(y=300, color='gray', linestyle='--', alpha=0.5, label='高空界限')

# 标注速度

speeds = self.scenario['drone_speeds'] if 'drone_speeds' in self.scenario else np.zeros(len(heights))

for i, (height, speed) in enumerate(zip(heights, speeds)):

ax.text(i, height + 5, f'{speed:.1f}m/s', ha='center', fontsize=8)

ax.set_xlabel('无人机编号')

ax.set_ylabel('高度 (m)')

ax.set_title('无人机高度与速度分布')

ax.set_xticks(x_positions)

ax.set_xticklabels([f'U{i}' for i in range(len(heights))])

ax.legend()

ax.grid(True, alpha=0.3, axis='y')

def _plot_detailed_communication_analysis(self, ax):

"""绘制详细通信分析"""

analysis = self.scenario.get('analysis', {}) if isinstance(self.scenario, dict) else {}

model = str(analysis.get('model', '')).strip().lower()

if analysis:

comm_label = 'CDI通信劣化' if model not in ('legacy', 'heuristic', 'old') else '通信功率强度变化'

categories = [comm_label, '速度导致的能效劣化']

comm_deg = float(np.clip(analysis.get('avg_comm_deg', 0.0), 0.0, 1.0))

speed_deg = float(np.clip(analysis.get('avg_speed_deg', 0.0), 0.0, 1.0))

degradation_values = np.array([comm_deg, speed_deg], dtype=float)

else:

# 兜底：若没有 analysis，就保留旧的启发式画法

categories = ['通信功率强度变化', '速度导致的通信能效(劣化)']

sources = self.scenario.get('interference_sources', [])

avg_power_dbm = float(np.mean([s.power for s in sources])) if sources else 0.0

power_change = float(1.0 / (1.0 + np.exp(-(avg_power_dbm - 20.0) / 8.0)))

speeds = self.scenario.get('drone_speeds', [])

vmax = float(self.scenario.get('drone_speed_max', 1.0))

vmax = max(vmax, 1e-6)

speed_eff_deg = float(np.mean([(v / vmax) ** 2 for v in speeds])) if len(speeds) else 0.0

speed_eff_deg = float(np.clip(speed_eff_deg / (1.0 + speed_eff_deg), 0.0, 1.0))

degradation_values = np.array([power_change, speed_eff_deg], dtype=float)

# 绘制雷达图

angles = np.linspace(0, 2 * np.pi, len(categories), endpoint=False).tolist()

degradation_values = np.concatenate((degradation_values, [degradation_values[0]]))

angles += angles[:1]

ax.plot(angles, degradation_values, 'o-', linewidth=2)

ax.fill(angles, degradation_values, alpha=0.25)

ax.set_thetagrids(np.degrees(angles[:-1]), categories)

ax.set_ylim(0, 1)

ax.set_title('通信性能劣化雷达图')

ax.grid(True)

def _plot_detailed_scenario_parameters(self, ax):

"""绘制详细场景参数"""

ax.axis('off')

drones = self.scenario['drone_positions']

stations = self.scenario['station_positions']

buildings = self.scenario['buildings']

sources = self.scenario.get('interference_sources', [])

text_lines = [

"详细场景参数",

"=" * 40,

f"区域: {self.scenario['area_size']:.0f}m × {self.scenario['area_size']:.0f}m",

f"无人机: {len(drones)}架",

f"地面站: {len(stations)}个",

f"建筑物: {len(buildings)}栋",

f"干扰源: {len(sources)}个",

"",

"无人机参数:",

]

# 无人机详细信息

for i, drone in enumerate(drones):

speed = self.scenario['drone_speeds'][i] if i < len(self.scenario['drone_speeds']) else 0

text_lines.append(f"  U{i}: ({drone[0]:.1f}, {drone[1]:.1f}, {drone[2]:.1f}m) - {speed:.1f}m/s")

text_lines.append("\n地面站位置:")

for i, station in enumerate(stations):

text_lines.append(f"  S{i}: ({station[0]:.1f}, {station[1]:.1f}, {station[2]:.1f}m)")

if buildings:

text_lines.append(f"\n建筑物统计:")

avg_height = np.mean([b.height for b in buildings])

max_height = np.max([b.height for b in buildings])

text_lines.append(f"  平均高度: {avg_height:.1f}m")

text_lines.append(f"  最大高度: {max_height:.1f}m")

text_lines.append(f"  数量: {len(buildings)}")

if sources:

text_lines.append(f"\n干扰源统计:")

ground_count = sum(1 for s in sources if s.is_ground)

building_count = len(sources) - ground_count

text_lines.append(f"  地面干扰源: {ground_count}个")

text_lines.append(f"  建筑物干扰源: {building_count}个")

text = "\n".join(text_lines)

ax.text(0.05, 0.95, text, transform=ax.transAxes, fontsize=8,

verticalalignment='top', bbox=dict(boxstyle='round', facecolor='lightblue', alpha=0.5))

def _plot_interference_details(self, ax):

"""绘制干扰源详情"""

sources = self.scenario.get('interference_sources', [])

if not sources:

ax.text(0.5, 0.5, '无干扰源', ha='center', va='center')

ax.set_title('干扰源详情')

return

ax.axis('off')

text_lines = ["干扰源详细信息", "=" * 40]

for i, source in enumerate(sources):

freq_ghz = source.frequency / 1e9

location_type = "地面" if source.is_ground else "建筑物"

building_info = f"建筑物上 ({source.building.height:.1f}m)" if source.building else "无建筑物"

text_lines.append(f"{i + 1}. {source.name} [{location_type}]")

text_lines.append(f"   位置: ({source.x:.1f}, {source.y:.1f}, {source.height:.1f}m)")

text_lines.append(f"   功率: {source.power:.1f}dBm, 频率: {freq_ghz:.2f}GHz")

if not source.is_ground:

text_lines.append(f"   建筑物: {building_info}")

text_lines.append("")

text = "\n".join(text_lines)

ax.text(0.05, 0.95, text, transform=ax.transAxes, fontsize=8,

verticalalignment='top', bbox=dict(boxstyle='round', facecolor='lightgreen', alpha=0.5))

def get_user_input():

"""获取用户输入参数"""

print("无人机通信场景优化系统")

print("=" * 60)

print("请输入以下参数:")

params = {}

# 无人机数量

while True:

try:

params['num_drones'] = int(input("无人机数量 (3-15, 推荐5-10): ") or "8")

if 3 <= params['num_drones'] <= 15:

break

else:

print("请输入3-15之间的数字")

except ValueError:

print("请输入有效的数字")

# 地面站数量

while True:

try:

params['num_stations'] = int(input("地面站数量 (2-8, 推荐3-5): ") or "4")

if 2 <= params['num_stations'] <= 8:

break

else:

print("请输入2-8之间的数字")

except ValueError:

print("请输入有效的数字")

# 区域大小

while True:

try:

params['area_size'] = float(input("区域大小 (米, 500-3000, 推荐1000-1500): ") or "1200")

if 500 <= params['area_size'] <= 3000:

break

else:

print("请输入500-3000之间的数字")

except ValueError:

print("请输入有效的数字")

# 无人机最大高度

while True:

try:

params['drone_height_max'] = float(input("无人机最大高度 (米, 100-500, 推荐200-400): ") or "300")

if 100 <= params['drone_height_max'] <= 500:

break

else:

print("请输入100-500之间的数字")

except ValueError:

print("请输入有效的数字")

# 建筑物最大高度

while True:

try:

params['building_height_max'] = float(input("建筑物最大高度 (米, 50-300, 推荐80-150): ") or "120")

if 50 <= params['building_height_max'] <= 300:

break

else:

print("请输入50-300之间的数字")

except ValueError:

print("请输入有效的数字")

# 建筑覆盖率（ITU α）：用于控制建筑总覆盖面积（0~1）

# 说明：这里的“覆盖率α”与下面的“建筑数量密度β(buildings/km^2)”不是同一个概念，避免误解为重复输入。

while True:

try:

alpha = float(input("建筑覆盖率 α (0.1-0.9, 推荐0.3-0.7): ") or "0.5")

if 0.1 <= alpha <= 0.9:

params['building_density'] = alpha  # 保持兼容：历史字段名仍叫 building_density

params['itu_alpha'] = alpha

break

else:

print("请输入0.1-0.9之间的数字")

except ValueError:

print("请输入有效的数字")

# ITU 城市统计参数（可选）：β、γ直接回车使用默认值

# 默认 β 过大时（例如 area_size≈1200m），建筑数量会非常多、视觉上过密；

# 这里把默认值调到更“常规演示”的水平（仍可在自定义模式里改回更高密度）。

default_beta = 120.0

default_gamma = max(5.0, params['building_height_max'] / 2.0)

customize_itu = input("是否自定义ITU参数 β/γ? (y/N): ").strip().lower() == 'y'

if customize_itu:

# β：建筑数量密度（buildings/km^2）

while True:

try:

params['itu_beta'] = float(input(

"ITU建筑数量密度 β (buildings/km^2, 50-2000, 回车默认120): "

) or "120")

if 50 <= params['itu_beta'] <= 2000:

break

else:

print("请输入50-2000之间的数字")

except ValueError:

print("请输入有效的数字")

# γ：Rayleigh 高度分布尺度参数（m）

while True:

try:

params['itu_gamma'] = float(input(

f"ITU建筑高度尺度 γ (m, 5-200, 回车默认{default_gamma:.1f}): "

) or f"{default_gamma:.1f}")

if 5 <= params['itu_gamma'] <= 200:

break

else:

print("请输入5-200之间的数字")

except ValueError:

print("请输入有效的数字")

else:

params['itu_beta'] = default_beta

params['itu_gamma'] = float(default_gamma)

# 无人机最大速度

while True:

try:

params['drone_speed_max'] = float(input("无人机最大速度 (m/s, 5-30, 推荐10-20): ") or "15")

if 5 <= params['drone_speed_max'] <= 30:

break

else:

print("请输入5-30之间的数字")

except ValueError:

print("请输入有效的数字")

return params

def save_scenario_data(scenario: Dict, filename: str):

"""保存场景数据到JSON文件"""

filepath = os.path.join("output", filename)

try:

# 准备场景数据

data = {

'basic_info': {

'area_size': float(scenario['area_size']),

'num_drones': len(scenario['drone_positions']),

'num_stations': len(scenario['station_positions']),

'num_buildings': len(scenario['buildings']),

'num_interference': len(scenario.get('interference_sources', [])),

'building_density': float(scenario['building_density']),

'drone_speed_max': float(scenario['drone_speed_max']),

'drone_height_max': float(scenario['drone_height_max']),

'building_height_max': float(scenario['building_height_max']),

'floor_height_m': float(scenario.get('floor_height_m', 0.0))

},

'semantic_layers': scenario.get('semantic_layers', {}),

'analysis': scenario.get('analysis', {}),

'building_observables': scenario.get('building_observables', {}),

'wifi_density': scenario.get('wifi_density', {}),

'drones': [],

'stations': [],

'buildings': [],

'interference_sources': []

}

# 无人机数据

for i, drone in enumerate(scenario['drone_positions']):

speed = scenario['drone_speeds'][i] if i < len(scenario['drone_speeds']) else 0

data['drones'].append({

'id': i,

'position': [float(drone[0]), float(drone[1]), float(drone[2])],

'speed_mps': float(speed),

'height_m': float(drone[2])

})

# 地面站数据

for i, station in enumerate(scenario['station_positions']):

data['stations'].append({

'id': i,

'position': [float(station[0]), float(station[1]), float(station[2])],

'height_m': float(station[2])

})

# 建筑物数据

floor_h = max(float(scenario.get('floor_height_m', 3.0)), 0.5)

for i, building in enumerate(scenario['buildings']):

footprint = float(building.width) * float(building.length)

floors = max(1, int(math.floor(float(building.height) / floor_h)))

floor_area = float(footprint * float(floors))

volume = float(footprint * float(building.height))

data['buildings'].append({

'id': i,

'position': [float(building.x), float(building.y), float(building.height / 2)],

'height_m': float(building.height),

'width_m': float(building.width),

'length_m': float(building.length),

'color': building.color,

'footprint_m2': float(footprint),

'volume_m3': float(volume),

'floors': int(floors),

'floor_area_m2': float(floor_area)

})

# 干扰源数据

for i, source in enumerate(scenario.get('interference_sources', [])):

info = source.get_info()

data['interference_sources'].append({

'id': i,

'name': info['name'],

'type_id': info['type_id'],

'position': info['position'],

'power_dbm': info['power_dbm'],

'raw_power_dbm': info.get('raw_power_dbm', info['power_dbm']),

'profile_key': info.get('profile_key', None),

'profile_meta': info.get('profile_meta', None),

'power_semantics': info.get('power_semantics', None),

'frequency_ghz': info['frequency_ghz'],

'building_associated': info['building_associated'],

'is_ground': info['is_ground'],

'is_indoor': info.get('is_indoor', False),

'indoor_floor': info.get('indoor_floor', None),

'semantic': info.get('semantic', None),

'activity': info.get('activity', 1.0),

'color': info['color']

})

# 保存到文件

with open(filepath, 'w', encoding='utf-8') as f:

json.dump(data, f, ensure_ascii=False, indent=2)

print(f"\n场景数据已保存: {filepath}")

print("数据格式: JSON")

print("用途: 可用于仿真模拟、进一步分析等")

except Exception as e:

print(f"保存场景数据时出错: {e}")

def main():

"""主函数 - 遗传算法优化"""

print("无人机通信性能劣化场景优化 - 高级版本")

print("=" * 60)

print("说明:")

print("1. 无人机高度作为染色体进行优化")

print("2. 干扰源参数（类型、功率、位置）作为染色体进行优化")

print("3. 干扰源可放置在建筑物或地面（地面干扰源高度为0）")

print("4. 干扰源统一使用星型标记，不同颜色表示不同类型")

print("=" * 60)

# 获取用户参数

user_params = get_user_input()

print("\n" + "=" * 60)

print("参数设置完成:")

display_keys = [

'num_drones',

'num_stations',

'area_size',

'drone_height_max',

'building_height_max',

'building_density',

'drone_speed_max'

]

for key in display_keys:

if key in user_params:

print(f"  {key}: {user_params[key]}")

# 创建问题实例

problem = DroneCommProblem(user_params)

# 检查维度

if not problem.check_dimensions():

print("维度错误，程序退出")

return

print("\n" + "=" * 60)

print("开始遗传算法优化...")

print(f"染色体维度: {problem.Dim}")

print(f"优化目标: 最大化通信劣化程度")

print("=" * 60)

# 构建算法

algorithm = ea.soea_SEGA_templet(

problem,

ea.Population(Encoding='RI', NIND=40),

MAXGEN=80,

logTras=10,

trappedValue=1e-4,

maxTrappedCount=10

)

# 开始优化

start_time = time.time()

try:

res = ea.optimize(

algorithm,

verbose=True,

drawing=1,

outputMsg=True,

drawLog=False,

saveFlag=False

)

elapsed_time = time.time() - start_time

print(f"\n优化完成，耗时: {elapsed_time:.2f}秒")

# 获取最优解

best_x = res['Vars'][0]

best_fitness = res['ObjV'][0][0]

# 打印结果

print_optimization_results(best_x, best_fitness, problem)

# 生成并可视化最优场景

visualize_optimized_scenario(best_x, problem, user_params)

except Exception as e:

print(f"优化过程中出现错误: {e}")

import traceback

traceback.print_exc()

def print_optimization_results(best_x: np.ndarray, best_fitness: float, problem):

"""打印优化结果"""

print("\n" + "=" * 60)

print("遗传算法优化结果:")

print(f"最佳适应度（通信劣化程度）: {best_fitness:.6f}")

print("\n染色体解码:")

idx = 0

# 无人机高度

drone_heights = best_x[idx:idx + problem.num_drones]

idx += problem.num_drones

print(f"无人机高度 (共{len(drone_heights)}个):")

for i, height in enumerate(drone_heights[:5]):  # 只显示前5个

print(f"  无人机{i}: {height:.1f}m")

if len(drone_heights) > 5:

print(f"  ... 还有{len(drone_heights) - 5}个")

# 无人机速度

drone_speeds = best_x[idx:idx + problem.num_drones]

idx += problem.num_drones

print(f"\n无人机速度 (平均: {np.mean(drone_speeds):.1f}m/s):")

# 干扰源类型

interference_types = [int(t) for t in best_x[idx:idx + problem.num_interference]]

idx += problem.num_interference

# 干扰源功率

interference_powers = best_x[idx:idx + problem.num_interference]

idx += problem.num_interference

# 干扰源位置类型

interference_location_types = [int(t) for t in best_x[idx:idx + problem.num_interference]]

idx += problem.num_interference

config = INTERFERENCE_SOURCE_CONFIG

print(f"\n干扰源配置 (共{len(interference_types)}个):")

for i, (type_id, power, location_type) in enumerate(

zip(interference_types, interference_powers, interference_location_types)):

type_config = config.get_config(type_id)

location_str = "地面" if location_type == 0 else "建筑物"

print(f"  干扰源{i}: {type_config['name']}, 功率: {power:.1f}dBm, 位置: {location_str}")

def visualize_optimized_scenario(best_x: np.ndarray, problem, user_params: Dict):

"""可视化最优场景"""

print("\n" + "=" * 60)

print("生成场景可视化...")

# 生成场景

scenario = problem.generate_scenario(best_x)

# 为可视化附加通信分析（CDI/干扰强度/链路裕量等）

try:

scenario['analysis'] = problem.analyze_scenario_metrics(scenario)

except Exception as e:

print(f"场景分析失败（不影响可视化）：{e}")

print(f"\n场景统计:")

print(f"  无人机数量: {len(scenario['drone_positions'])}")

print(f"  地面站数量: {len(scenario['station_positions'])}")

print(f"  建筑物数量: {len(scenario['buildings'])}")

print(f"  干扰源数量: {len(scenario['interference_sources'])}")

print(f"  区域大小: {scenario['area_size']:.0f}m × {scenario['area_size']:.0f}m")

# 无人机统计

drone_heights = scenario['drone_positions'][:, 2]

drone_speeds = scenario['drone_speeds']

print(

f"  无人机平均高度: {np.mean(drone_heights):.1f}m (范围: {np.min(drone_heights):.1f}-{np.max(drone_heights):.1f}m)")

print(f"  无人机平均速度: {np.mean(drone_speeds):.1f}m/s")

# 建筑物统计

if scenario['buildings']:

building_heights = [b.height for b in scenario['buildings']]

print(f"  建筑物平均高度: {np.mean(building_heights):.1f}m")

print(f"  建筑物最大高度: {np.max(building_heights):.1f}m")

# 干扰源统计

if scenario['interference_sources']:

interference_types = {}

ground_count = 0

building_count = 0

for source in scenario['interference_sources']:

name = source.name

interference_types[name] = interference_types.get(name, 0) + 1

if source.is_ground:

ground_count += 1

else:

building_count += 1

print(f"\n干扰源类型分布:")

for name, count in interference_types.items():

print(f"  {name}: {count}个")

print(f"\n干扰源位置分布:")

print(f"  地面干扰源: {ground_count}个")

print(f"  建筑物干扰源: {building_count}个")

# 创建可视化器

visualizer = EnhancedVisualizer(scenario)

# 先生成单独的图表

print("\n生成单独的图表...")

plots_info = visualizer.create_individual_plots("optimized")

print("\n已生成以下图表:")

for name, filename in plots_info:

print(f"  {name}: {filename}")

# 保存场景数据

timestamp = time.strftime("%Y%m%d_%H%M%S")

data_filename = f"scenario_data_{timestamp}.json"

save_scenario_data(scenario, data_filename)

# 询问是否生成综合报告

print("\n是否生成综合报告图? (y/n)")

choice = input("请输入选择: ").lower()

if choice == 'y':

print("生成综合报告...")

report_file = visualizer.create_comprehensive_report("optimized")

print(f"综合报告已保存: {report_file}")

# 分析通信劣化

print("\n" + "=" * 60)

print("通信劣化分析:")

# 计算平均劣化程度

total_degradation = 0

link_count = 0

for drone_idx in range(len(scenario['drone_positions'])):

for station_idx in range(len(scenario['station_positions'])):

degradation = problem.calculate_link_degradation(

scenario['drone_positions'][drone_idx],

scenario['station_positions'][station_idx],

scenario,

drone_speed_mps=float(scenario['drone_speeds'][drone_idx]),

drone_idx=int(drone_idx)

)

total_degradation += degradation

link_count += 1

if link_count > 0:

avg_degradation = total_degradation / link_count

print(f"  平均通信劣化程度: {avg_degradation:.3f}")

# 评级

if avg_degradation > 0.7:

rating = "严重劣化"

suggestion = "强烈建议重新规划飞行路径和通信频段"

elif avg_degradation > 0.5:

rating = "显著劣化"

suggestion = "建议优化无人机高度和避让干扰源"

elif avg_degradation > 0.3:

rating = "中等劣化"

suggestion = "通信质量可接受，建议监控关键链路"

else:

rating = "轻微劣化"

suggestion = "通信质量良好"

print(f"  劣化评级: {rating}")

print(f"  建议: {suggestion}")

print("\n" + "=" * 60)

print("优化完成！所有结果已保存到output目录")

print("=" * 60)

def quick_demo():

"""快速演示"""

print("无人机通信场景快速演示")

print("=" * 60)

print("使用预设参数生成示例场景")

print("=" * 60)

# 预设参数

user_params = {

'num_drones': 6,

'num_stations': 4,

'area_size': 1200,

'drone_height_max': 350,

# 让预设场景“更正常一些”：控制建筑数量密度 β 与覆盖率 α，避免过密

'building_height_max': 120,

'building_density': 0.45,  # 历史字段名；等价于 ITU α

'itu_alpha': 0.45,

'itu_beta': 120.0,

'itu_gamma': 40.0,

'drone_speed_max': 18

}

print("预设参数:")

for key, value in user_params.items():

print(f"  {key}: {value}")

# 创建问题实例

problem = DroneCommProblem(user_params)

# 使用预设染色体（模拟优化结果）

np.random.seed(123)

preset_x = np.random.uniform(problem.lb, problem.ub)

# 整数参数取整

preset_x[problem.num_drones * 2:problem.num_drones * 2 + problem.num_interference] = \

np.round(preset_x[problem.num_drones * 2:problem.num_drones * 2 + problem.num_interference])

preset_x[problem.num_drones * 2 + problem.num_interference * 2:

problem.num_drones * 2 + problem.num_interference * 3] = \

np.round(preset_x[problem.num_drones * 2 + problem.num_interference * 2:

problem.num_drones * 2 + problem.num_interference * 3])

# 生成并可视化场景

visualize_optimized_scenario(preset_x, problem, user_params)

if __name__ == "__main__":

# 选择运行模式

print("无人机通信场景优化与可视化系统")

print("=" * 60)

print("请选择运行模式:")

print("1. 完整遗传算法优化")

print("2. 快速演示（使用预设参数）")

print("3. 退出")

choice = input("请输入选择 (1-3): ")

if choice == "1":

main()

elif choice == "2":

quick_demo()

elif choice == "3":

print("程序退出")

else:

print("无效选择，运行完整遗传算法优化...")

main()